3GPP TSG-RAN WG4 Meeting #102-e	R4-2206446

21st February – 3rd March 2022

Source:	Qualcomm Incorporated

Title:	Calculation of MSD for V2X_n1A-47A and accompanying TP

Agenda item:	9.25.2

Work Item:	          NR_LTE_V2X_PC5_combos-Core

Document for:	Approval

# 1.	Introduction

In RAN#94e a new inter-band V2X combination V2X_n1A-47A was introduced [1]. In this paper we present the MSDs required for band 47 for this V2X band combination.

# 2. 	Discussion

In [1] the inter-band combination V2X_n1A-47A was introduced. For this combination the 3rd harmonic of band 1 can fall into band 47 causing degradation in sensitivity. The 3rd harmonic of band 1 can couple into the 47 receiver through the PCB or can be conducted from the band 1 antenna to the 47 antenna. The below table details the MSD calculation for these V2X combinations.

Table 1: TX to RX H3 budget

|  | B1/n1 PA-->CV2x 47 Rx |
| --- | --- |
| Bandwidth (MHz) | 10 |
| SCS (kHz) | 15 |
| 3GPP Sensitivity(dBm) | -90.4 |
| PA out B1/n1 H3 (dBm) | -7 |
| B1 Tx Filter Rejection+ FE filtering | -71 |
| PCB coupling to B1/n1 PRX/DRX LNA | -70 |
| Tx-Rx Antenna Isolation | -10 |
| 47 front-end loss (dB) | 4 |
| PRX/DRX B1/n1 H3 conducted into 47 RX (dBm) | -88 |
| PRX/DRX B1/n1 H3 coupled into 47 RX (dBm) | -73 |
| PRX/DRX total H3 (dBm) | -72.9 |
| PRX/DRX noise (dBm) | -86.4 |
| Dual sensitivity (dBm) | -73.8 |
| Desensitization (dB) | 16.6 |

The MSD values for the other SCS and BW combinations were calculated in a similar manner and based on these results the following MSD recommendations are made for each V2X combination and BW.

Table 2: Band 47 MSD for V2X_n1A-47A

| BW(MHz) | 10 | 20 |
| --- | --- | --- |
| MSD (dB) | 16.6 | 13.8 |

Proposal 1: Adopt the MSD’s given in Table2 for band 47 for the following V2X combinations: V2X_n1A-47A

# Conclusion

We present the analysis for MSD for inter-band combination V2X_n1A-47A and make the following proposals.

Proposal 1: Adopt the MSD’s given in Table2 for band 47 for the following V2X combination: V2X_n1A-47A

Table 2: Band 47 MSD for V2X_n1A-47A

| BW(MHz) | 10 | 20 |
| --- | --- | --- |
| MSD (dB) | 16.6 | 13.8 |

# Annex - TP for V2X_n1A_47A MSD

## --Start of changes

***********************************Start of the TP*************************************

#### 6.3.7.4	MSD, ∆TIB and ∆RIB values

## MSD test configurations are specified below for V2X_n1A-47A. The MSD values given in tables 6.3.7.4-1. The MSD values given below shall apply to all n1 and 47 BW and SCS combinations.

Table 6.3.7.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for V2X

| UL band | SL band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of SL band | SL band BW (MHz) | MSD value of SL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 47 | 15 | 50 | 10 | 50 | 10 | 16.6 |
| n1 | 47 | 15 | 50 | 10 | 98 | 20 | 13.8 |

--End of changes

# References

[1] R4-2105367, CR for TS 38.101-1, Introduce new band combination of V2X_n79A-n47A, RAN4#98-bis, Apr. 12-20 2021