3GPP TSG-RAN WG4 Meeting # 102-e                               	                       R4-2206525

Electronic Meeting, February 21 – March 3, 2022

Title: 	WF on switching time mask for V2X intra-band con-current operation

Source: 	CATT

Agenda item:	10.15.3

Document for:	Approval

- Introduction

This WF will capture the agreements regarding switching time mask for intra-band V2X con-current operation.

- Way forwards

The following agreements are made as below:

Issue 1-1-1: Whether to include TA difference into switching time mask

Agreement: Define the switching time mask requirement only considering the hardware limitation

- Add the note to clarify that there will be additional TA difference included in the switching time in the real field.

- There is no test case for it.

Issue 1-1-2: Whether to consider the worst case

Agreement: Do not consider the worst case with maximum or minimum TA for switching time mask.

Issue 1-1-3: Scheduling restriction for same carrier switching and different carrier switching

Agreement: No need to inform RRM session of the difference between same carrier switching and different carrier switching for scheduling availability requirements.

Issue 1-1-4: RF test for switching time

Agreement: No RF test is needed for the switching time for TDM intra-band con-current operation.

Issue 1-1-5: Whether to capture into TR 38.785 the statement that no RF test for switching time is needed if agreed.

Agreement: Capture into TR 38.785 the statement that no RF test for switching time is needed and the switching time for different carrier case is 140us.

Issue 1-1-6: Whether to send LS to RAN5 for indication of no RF test for switching time

Agreement: Send LS to RAN5 for indication of no RF test for switching time.

Issue 1-1-7: Switching time mask for same carrier case

Agreement: Adopt the switching time mask for same carrier case as below.

Issue 1-1-8: Switching time mask for different carrier case

Agreement: Adopt the switching time mask for different carrier case as below.

- Reference

- R4-2206431, Email discussion summary for [102-e][131] NRSL_enh_Part_2, Moderator (CATT), RAN4#102-e

- R4-2205133, On switching time mask, Xiaomi, RAN4#102-e

- R4-2203912, Draft CR for TS 38.101-1, Remaining RF requirements for intra-band con-current operation, CATT, RAN4#102-e

- R4-2204155, Draft CR on MPR and ON/OFF time mask for intra-band con-current V2X operation in Rel-17, LG Electronics France, RAN4#102-e

- R4-2204931, Further discussion on switching time mask for intra-band V2X con-current operation, vivo, RAN4#102-e

- R4-2205137, draft CR for TS 38.101-1 on switching time mask between SL and Uu, Xiaomi, RAN4#102-e

- R4-2203911, Time mask for Uu and SL switching, CATT, RAN4#102-e

- R4-2204015, RF switching for V2X intra-band con-current operation with different carriers in TDD bands, Qualcomm Incorporated, RAN4#102-e

- R4-2204920, Synchronous operation between SL and Uu, ZTE Corporation, RAN4#102-e

- R4-2205585, On time mask for SL intra-band con-current operation, Huawei, HiSilicon, RAN4#102-e

- R4-2205586, draft CR for TS 38.101-1: On time mask for SL intra-band con-current operation, Huawei, HiSilicon, RAN4#102-e