3GPP TSG-RAN WG4 Meeting #102-e	R4-2206621

# Online, , 21th Feb 2022 - 3rd March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.307 | CR | 0093 | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR for TS 38.307 Maintenance (Rel-17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, ENDC_UE_PC2_FDD_TDD-Core |  |  |  |  |  | Date: |  |  | 2022-03-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This big CRs merge the mutiple endorsed draft CRs. The reason for change in each endorsed draft CR is copied below. R4-2204066 The Power Class 2 FDD-TDD EN-DC had been introduced in the release 16 specifications with the duty cycle reporting as an optional solution and the UE-based solution (P-MPR) as the default solution. Since the P-MPR solution is compatible with the Rel.15 signalling, it is proposed to introduce the Power Class 2 FDD-TDD EN-DC to be release independent from Rel.15. Note that this is a re-submission of the approved CR R4-2016987, the changes are the same but with the latest spec, since R4-2016987 seems not implemented R4-2204072 During RAN4#101-bis-e, the procedure for introducing release independent features was discussed and there were comments to capture the procedure to the general section of the TS 38.307. This draft CR is based on the agreed procedure in approved WF in R4-2202405 and additional aspect discussed in the discussion paper R4-2204069. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The summary of change in each endorsed draft CR is copied below. R4-2204066 Introduce the Power Class 2 FDD-TDD EN-DC to be release independent from Rel.15. R4-2204072 Specify the information for the procedure of introducing release independent features. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consequences if not approved for each endorsed draft CR are coppied below. R4-2204066 Release independent information for Power class 2 FDD-TDD EN-DC is not specified. R4-2204072 The procedure of introducing release independent features is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 8.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Start of Change>>

# 4	General

TSG-RAN has agreed for certain features (see the following clauses) to introduce them in a "release independent way".

This means for each feature:

-	it is "introduced" in a release N, i.e. TS 38.101 [2-5] and TS 38.133 [6] of release N define certain UE requirements for this feature; the feature is indicated in the tables of the following clauses;

-	it is "release independent" starting from a release M (M<N); M for the given feature is provided in the tables of the following clauses;

-	UEs supporting this feature have to fulfil additional requirements in release M or higher which are specified in one or more Annexes of TS 38.307 of release N; the applicable Annexes for a given feature are provided in the tables of the following clauses.

The applicable UE Categories are specified in TS 38.306 [7] according to the release to which the UE conforms.

In the table of release independent features in subsequent clauses, "FDD, TDD" refers to CA or EN-DC configuration composed by only FDD bands or only TDD bands, respectively. "FDD and TDD" refers to CA or EN-DC configuration including both FDD and TDD bands. "SDL and FDD, SDL and TDD" refers to CA configuration including both SDL and FDD bands or both SDL and TDD bands, respectively.  "TDD and SUL" refers to SUL configuration including both TDD and SUL bands. "FDD and TDD and SUL" refers to EN-DC configuration including both FDD, TDD and SUL bands. Unless stated otherwise,  the release independent for the band combinations are from Rel-15.

When a new feature is introduced only the latest release of release independent spec needs to be updated. The latest release of release independent spec refers to the release which the new feature is introduced in.

If an RF feature introduced in the same release as the release which the feature is independent from, (i.e. M=N), the common UE RF requirements table in annex B.4 is specified from release N+1, otherwise the common UE RF requirements table is specified from release N.

## <<Next of Change>>

### 8.1.2	Interband EN-DC

#### 8.1.2.1	Interband EN-DC within frequency range 1

Requirements for a Rel-16 UE for additional EN-DC interband configurations within FR1 compared to TS 38.101-3 of Rel-16 [4] are introduced via this clause.

Table 8.1.2.1-0: EN-DC interband UE power class

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| Interband EN-DC Power Class 2 | TDD, FDD and TDD | Rel-15 | Table B.4.6-1 |
| Interband EN-DC Power Class 3 | FDD, TDD, FDD and TDD | Rel-15 |  |

Table 8.1.2.1-1: EN-DC interband configurations without SUL within FR1

| Feature | DL/UL | maximum number of E-UTRA bands | maximum number of E-UTRA CCs | maximum number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interband EN-DC | DL | 6 | 6 | 2 | 3 | FDD, TDD, FDD and TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 1 | 2 | 1 | 2 | FDD, TDD, FDD and TDD | Rel-15 |  |

Table 8.1.2.1-2: EN-DC interband configurations with SUL within FR1

| Feature | DL/UL | maximum number of E-UTRA bands | maximum number of E-UTRA CCs | maximum number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interband EN-DC | DL | 2 | 3 | 1 | 1 | FDD, TDD, FDD and TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 1 | 1 | 2 | 2 | FDD, TDD, FDD and TDD and SUL | Rel-15 |  |

#### 8.1.2.2	Interband EN-DC including frequency range 2

Requirements for a Rel-16 UE for additional EN-DC interband configurations including FR2 compared to TS 38.101-3 of Rel-16 [4] are introduced via this clause.

Table 8.1.2.2-1: EN-DC interband configurations including FR2

| Feature | DL/UL | number of E-UTRA bands | maximum number of E-UTRA CCs | number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interband EN-DC | DL | 4 | 6 | 1 | 10 | TDD, FDD and TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 1 | 4 | 1 | 8 | TDD, FDD and TDD | Rel-15 |  |

#### 8.1.2.3	Interband EN-DC including frequency range 1 and frequency range 2

Requirements for a Rel-16 UE for additional EN-DC interband configurations including FR1 and FR2 compared to TS 38.101-3 of Rel-16 [4] are introduced via this clause.

Table 8.1.2.3-1: EN-DC interband configurations including FR1 and FR2

| Feature | DL/UL | maximum number of E-UTRA bands | maximum number of E-UTRA CCs | maximum number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interband | DL FR1 | 4 | 6 | 1 | 2 | TDD, FDD, FDD and TDD | Rel-15 | Table B.4.6-1 |
| EN-DC | DL FR2 |  |  | 1 | 8 | TDD | Rel-15 |  |
|  | UL FR1 | 1 | 2 | 1 | 1 | FDD, TDD, FDD and TDD | Rel-15 |  |
|  | UL FR2 |  |  | 1 | 3 | TDD, | Rel-15 |  |

## <<End of Change>>