3GPP TSG-RAN WG4 Meeting #102-e 												R4-2206754

Electronic Meeting, February 21 – March 3, 2022

Agenda item:			10.8.1 & 10.8.2

Source:	Moderator (CMCC)

Title:	Email discussion summary for [102-e][211] NR_HST_FR1_enh_RRM_1

Document for:	Information

Introduction

This email discussion focuses on RRM for Rel-17 NR FR1 HST, and in particular the agenda items:

10.8.1 General

10.8.2 RRM core requirements

10.8.2.1 Intra-frequency measurements

10.8.2.2 Inter-frequency measurements

10.8.2.3 L1-SINR measurements

10.8.2.4 Other

The targets of email discussion for 1st round and 2nd round are:

- 1st round: focus on discussing the open issues and strive to minimize the open issues

- 2nd round: according to 1st round discussion, discuss left open issues for 2nd round, and strive to agree on the draft CRs.

Topic #1: intra-frequency measurements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2204269 | CMCC | Draft CR on enhanced requirements for SCell measurement for Rel-17 FR1 HST requirements |

Open issues summary

N/A

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2204269 (CMCC) | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Topic #2: inter-frequency measurements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2204889 | Huawei, Hisilicon | Correction on inter-frequency measurements for FR1 HST |

Open issues summary

N/A

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2204889 (Huawei, Hisilicon) | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Topic #3: other

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2203710 | Qualcomm, Inc. | Proposal 1: SNR upper bound for L1-RSRP measurement requirement should follow R16 SS-SINR. Proposal 2: Rel-17 FR1 HST RRM enhancement, including CA enhancement and inter-frequency measurement enhancement, is release independent from Rel-16. |
| R4-2203741 | Apple | Proposal 1: Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB. |
| R4-2203742 | Apple | CR on L1-SINR measurement in FR1 HST |
| R4-2203743 | Apple | Proposal 1: Whether release independent is supported or not shall consider both RRM requirements and demodulation requirements. Proposal 2: Rel-17 FR1 HST RRM enhancement, including CA enhancement and inter-frequency measurement enhancement, is release independent from Rel-16. |
| R4-2203897 | CATT | Proposal 1: Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB. |
| R4-2204204 | MediaTek (Shenzhen) Inc. | Observation 1: Considering the carrier frequency (2.1GHz for 15kHz and 3.6GHz for 30kHz) and UE speed (500 km/hr), the carrier frequency offset should be 1944Hz and 3333Hz. Observation 2: For the channel model in HST, it should be AWGN with 2*Doppler shift. Observation 3: For the frequency tracking, UE is not required to perform the frequency compensation for the SS-SINR and L1-SINR measurement. Observation 4: For L1-SINR in R16 eMIMO, the average accuracy are 2.1 dB and 1.9 dB in the baseband for 15kHz and 30kHz, respectively. Proposal 1: For L1-SINR measurement in HST, the upper bound of the Ês/Iot side condition should be 5 dB. |
| R4-2204260 | CMCC | Proposal 1: Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB Proposal 3: it is proposed that Rel-17 FR1 HST RRM enhancement, including CA enhancement and inter-frequency measurement enhancement, are release independent from Rel-15. |
| R4-2204273 | OPPO | Proposal 1: Can compromise to option 1 that existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB. Proposal 2: Whether release independent is supported or not shall consider both RRM requirements and demodulation requirements. |
| R4-2204334 | vivo | Proposal 1 L1-SINR measurements should be applicable to FR1 HST. Observation 1 RRM requirements was defined for CSIRS-based CMR-only L1-SINR measurements in R16 eMIMO. Observation 2 According to demod discussion in R16 HST, the baseline scenario for defining requirements should be DPS 1a, which is for UE only capable of tracking 1 TCI state. Observation 3 According to TS 38.101-4, 972Hz Doppler shift for SCS15kHz HST single tap is only for performance verification, and it is not meant to indicate the max Doppler shift UE needs to dealt with in real SCS15kHz HST deployment. Observation 4 According to TRS pattern specified in RAN1 specs, the frequency track ability of TRS is +/-1750Hz for SCS15kHz, and +/-3500Hz for SCS30kHz. Observation 5 In R16 SS-SINR accuracy evaluations for HST, no performance degradation is shown on the serving cell SS-SINR measurement if the one-tap scenario is considered, even assuming 1944Hz Doppler shift. Observation 6 The considered scenario in R17 is different, since CSI-RSs are UE-specific RS, but SSBs are cell-specific RSs. Proposal 2 RAN4 confirm to prioritize the case that UE is capable of tracking only 1 TCI for L1-SINR measurements in HST scenario. Proposal 3 RAN4 may add a note in TS 38.133 to the L1-SINR measurement accuracy requirements, ‘For L1-SINR measurements requirements in HST scenario, the considered max doppler shift should not beyond 1750Hz, at least for UE that supports tracking only 1 TCI state.’ Proposal 4 No need to add any upper bound to the side condition of L1-SINR requirements under FR1 HST scenario. Proposal 5 Remove the evaluation assumptions on propagation condition or revise it to Table B3.3-1 in TS 38.101-4. Observation 7 Significant performance degradation can be observed when Doppler spread is 1944Hz and SCS is 15kHz, which is caused by disfunction of TRS-based frequency tracking. |
| R4-2204713 | Ericsson | Proposal 1: Support option 1, Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB. |
| R4-2204714 | Ericsson | Proposal 1: Support Option 2 and Option 3, both RRM requirements and demodulation requirements are release independent from Rel-16. |
| R4-2204890 | Huawei, Hisilicon | Proposal 1: The upper bound of the L1-SINR side condition can be 5dB. |
| R4-2204891 | Huawei, Hisilicon | Proposal 1: Whether release independent is supported or not shall consider both RRM requirements and demodulation requirements. |
| R4-2205209 | Nokia, Nokia Shanghai Bell | Proposal 1a: Simulation evaluation for the L1-SINR accuracy requirement for FR1 HST should be based on the agreed simulation assumptions in the WF, where frequency tracking is enabled. 1b: Current L1-SINR measurement requirement can be reused in HST, no upper bound of side condition. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1: L1-SINR

Issue 3-1-1: how to specify the L1-SINR requirements for high speed scenario

- Proposals

  - Option 1 (QC, Apple, CATT, MTK, CMCC, OPPO, Ericsson, HW): Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB

  - Option 2 (vivo, Nokia): Existing L1-SINR measurement requirements are reused for HST, no upper bound of side condition

- Recommended WF

  - L1	-SINR requirements include both delay requirements (core requirements) and accuracy requirements (performance requirements). It is moderator understanding that the side condition issue is related with accuracy requirements, similar as we did for SS-SINR accuracy requirements in Rel-16 HST

  - This meeting is the last meeting for core part completion, and according to the proposals from companies, all companies agree that the existing L1-SINR requirements can be reused HST. Taking above consideration into account, Moderator would like to check with companies whether following suggestion is agreeable:

    - For L1-SINR for FR1 HST

      - The existing measurement delay requirements for L1-SINR is reused

      - The existing accuracy requirements for L1-SINR is reused, FFS whether to have upper bound of the side condition

| Issue 3-1-1: how to specify the L1-SINR requirements for high speed scenario |  |
| --- | --- |
| Company | Comments |
| QC | Support option 1 |
| Intel | We support Option 1. We also support WF however one thing to clarify: to FFS something in the last meeting might not be proper; otherwise, do we consider side condition as perf. Part? |
| Ericsson | Support Option 1, but can agree with WF to leave side condition issue together with bound issue in performance part. |
| MTK | Support Option 1 and also fine with the recommended WF. |
| CATT | Support Option 1. Can agree with WF to finalize core part in this meeting. |
| Huawei | Support Option 1 and also fine with the recommended WF. |
| Nokia | Support Option 2 and also the recommended WF provided the wording is changed as follows: - For L1-SINR for FR1 HST - The existing measurement delay requirements for L1-SINR is reused - The existing accuracy requirements for L1-SINR is reused, |
| OPPO | Support Option 1 and also fine with the recommended WF. |
| Apple | Support option 1. Fine with the recommended WF. |
| vivo | Support option 2. Regarding the FFS in the recommended WF, we prefer to keep it more general for the 2nd bullet. For example, ‘Whether and how to define applicability of side condition for the L1-SINR accuracy requirements can be further discussed and concluded in the perf. phase’ |
| CMCC | OK with the recommended WF |

Sub-topic 3-2: release independent

Issue 3-2-1: release independent

- Proposals

  - Option 1 (CMCC): Rel-17 FR1 HST RRM enhancement, including CA enhancement and inter-frequency measurement enhancement, is release independent from Rel-15

  - Option 2 (QC, Apple, Ericsson): Rel-17 FR1 HST RRM enhancement, including CA enhancement and inter-frequency measurement enhancement, is release independent from Rel-16.

  - Option 3 (Apple, OPPO, Ericsson, HW): Whether release independent is supported or not shall consider both RRM requirements and demodulation requirements

- Recommended WF

  - More discussion is needed

| Issue 3-2-1: release independent |  |
| --- | --- |
| Company | Comments |
| QC | Option 2 and 3 are both good for us |
| Intel | Clarification question on this issue: how the work is planned in terms of RAN2 timeline? For early implementation do we inform RAN2 as early as possible? Let’s decouple RRM and demod discussion for now. |
| Ericsson | Support Option 2 and Option3. |
| Huawei | Support option 2 and option 3. |
| OPPO | Support option 2 and option 3. |
| Apple | Support option 2 and 3. |
| CMCC | For option 3, considering this is RRM email thread, it is suggested to focus on the discussion from RRM point of view. In demodulation session, there is also a dedicated discussion for this issue. It is not preferred to mix the discussion between RRM and demodulation. For Rel-17 FR1 HST RRM enhancement, including CA enhancement and inter-frequency measurement enhancement, our preference is to be release independent from Rel-15, and we do not see any issues to do so by adopting early implementation method. However, to move forward, we can compromise to Rel-16. Reply To Intel’s comments: According to our RAN2 colleagues, for early implementation, the only work in RAN2 is to add R17 FR1 HST into the Table on Early Implementable Features (Annex C in TS 38.331). If we reach consensus, it is better to send LS to RAN2. |

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2203742 (Apple) | QC: pending RRM discussion, condition should be specified in flag signaled instead of “scenario”? |
|  | Nokia: This CR depends on the conclusion of Issue 3-1-1. |
|  | Apple: CR can be updated accordingly pending outcome of issue 3-1-1. “scenario” is copied and pasted from L3 SINR requirements. We don’t have strong view but slightly prefer to keep the same wording. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1: L1-SINR | Issue 3-1-1: how to specify the L1-SINR requirements for high speed scenario According to the comments, all the companies are OK with the recommended WF, and three companies have suggestions on the wording on FFS part of the recommended WF which are taken into account int the following tentative agreements. Tentative agreements: - For L1-SINR for FR1 HST - The existing measurement delay requirements for L1-SINR is reused - The existing accuracy requirements for L1-SINR is reused - Whether and how to define applicability of side condition for the L1-SINR accuracy requirements can be further discussed and concluded in the perf. phase Candidate options: Whether and how to define applicability of side condition for the L1-SINR accuracy requirements Option 1(QC, Apple, CATT, MTK, CMCC, OPPO, Ericsson, HW): the upper bound of the side condition for L1-SINR accuracy is 5dB Option 2 (vivo, Nokia): no upper bound of side condition for L1-SINR accuracy Recommendations for 2nd round: Continue the discussion on the issue on side condition for the L1-SINR accuracy requirements |
| Sub-topic#3-2: release independent | Issue 3-2-1: release independent According to the comments, all companies are OK that RRM enhancements are release independent from R16. For information, according to the GTW agreements on demodulation session, it was agreed that from demodulation perspective, FR1 HST is release independent from Rel-16. Tentative agreements: For Rel-17 FR1 HST RRM enhancement, including CA enhancement and inter-frequency measurement enhancement, is release independent from Rel-16. Recommendations for 2nd round: As suggested by companies, it is necessary to send LS to RAN2 to inform RAN2 the conclusion on release independent. And the content of the LS can be further checked in the 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on RRM for FR1 HST | CMCC |  |
| LS on release independent for Rel-17 FR1 HST RRM enhancement | CMCC | To: RAN_2 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2204269 | Draft CR on enhanced requirements for SCell measurement for Rel-17 FR1 HST requirements | CMCC | Agreeable |  |
| R4-2204889 | Correction on inter-frequency measurements for FR1 HST | Huawei, Hisilicon | Agreeable |  |
| R4-2203742 | CR on L1-SINR measurement in FR1 HST | Apple | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)