3GPP TSG-RAN WG4 Meeting #102e                	    			         R4-2206787

Electronic Meeting, February 21 – March 3, 2022

Title:	LS on RLM/BFD relaxation for NR UE power saving enhancements

Response to:

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name:	Hsuanli, Lin

E-mail Address:	Hsuanli.Lin (at) mediaTek.com

1. Overall Description

RAN4 has discussed how to provide the relaxation criteria configuration, and how to evaluate the criteria for RLM and BFD relaxation and reached the following agreements

- Agreement

  - The offset X dB for the cell quality criteria is configured per UE. Separate values can be configured per FR.

  - The offset X dB can be configured separately for RLM and BFD

- Agreement

  - the low mobility criteria is evaluated on the

    - NR PCell for the case of NR single carrier, NR CA, NE-DC

    - NR PSCell for the case of EN-DC

    - NR PCell for the case of NR-DC

2. Actions:

ACTION: 	RAN4 respectfully asks RAN2 to take above information in consideration and specify the corresponding procedures and required RRC signalings.

3. Date of Next RAN4 Meetings:

RAN4#103e, E-Meeting, 16 May to 27 May 2022