3GPP TSG-RAN WG4 Meeting # 102-e	R4-2206861

# Electronic Meeting, Feb 21- Mar 4, 2022

Agenda item:	10.10.2.1

Source:	Apple

Title:	WF on further RRM enhancement for NR and MR-DC - SRS antenna port switching

Document for:	Approval

# Impact of SRS antenna port switching to other requirements

Issue 1-1: Impact of SRS antenna port switching to RRM requirements in NR-SA

Agreement in 2nd round:

- NR measurements are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement

  - No requirement for AP L1-RSRP/L1-SINR measurements colliding with AP SRS

# Interruption requirement design for SRS antenna port switching

Issue 2-1: Antenna switching time

- Proposals

  - Option 1 (LGE): The antenna switching time (15us) should be applied only when the symbol before or after SRS transmission occasion is uplink symbol since the switching time is for Tx-to-Tx, and the transient time for Tx(Rx)-to-Rx(Tx) is 10us.

  - Option 2 (CATT, MTK, HW): generic requirement is preferred and no need to consider option 1.

Agreement in 1st round: generic requirement is preferred and no need to consider option 1 for issue 2-1.

Issue 2-3: Interruption requirement (symbol-level) proposals for scenario 1 sync case

Agreement in 2nd round:

- Interruption requirement (symbol-level) for scenario 1 sync case

  - Note: Unit of interruption requirement is symbol of victim CC

| Victim CC SCS (kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 3 | 2 | 2 |
| 30 | 4 | 3 | 3 |
| 60 | 8 | 6 | 5 |
| 120 | 14 | 10 | 8 |

Issue 2-4: Interruption requirement (slot-level) proposals for scenario 1 async case

GTW agreement:

- Interruption requirement (slot-level) for scenario 1 async case

  - Note: Unit of interruption requirement is slot for NR and subframe for LTE of victim CC

| Victim CC SCS (kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 2 | 2 | 2 |
| 120 | 2 | 2 | 2 |

Issue 2-5: Interruption requirement (slot-level) proposals for scenario 2

2nd round Agreement:

- Interruption requirement (slot-level) for scenario 2

  - Note: Unit of interruption requirement is slot for NR and subframe for LTE of victim CC

| Victim CC SCS (kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

# Others

Issue 3-1: Impacts from SRS antenna port switching enhancement in R17 FeMIMO

Agreement in 2nd round:

- Clarify that the interruption requirements applies when SRS resources are allocated in the last 6 symbols in a slot.

- No need to discuss whether or not the SRS resources of a set in a slot are configured in non-consecutive manner.

  - the non-consecutive case is an issue from R17 FeMIMO WI, and we don’t consider R17 FeMIMO in this FeRRM WI

Issue 3-2: further clarifications on interruption requirement

Agreement in 2nd round :

- For “Proposal 1: For the sync case of scenario 1, further discuss whether the case when the last symbol in the slot on the aggressor CC is not used for SRS transmission is only considered for test case design, in which the maximum number interrupted slots for SRS antenna switching is 1 for all 15kHz and 30kHz aggressor CC SCS cases.”,

  - Tentative Agreement: can discuss proposal 1 in performance stage

- For “Proposal 2: For scenarios 1, the symbol based requirement will apply if one SRS resource set is configured. It’s FFS when two SRS resource sets are configured in two consecutive slots.”

  - Tentative Agreement: multiple SRS resource sets could be considered in future release if needed.

# References

[1] R4-2206757, Email discussion summary for [102-e][214] NR_RRM_enh2_1, Moderator (Apple), RAN4 #102-e