3GPP TSG RAN WG4 Meeting #102-e	R4-2206863

# E-meeting, 21 Feb. 2022 – 03 Mar. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Interruption requirement to NR serving cell, and impacts to other NR RRM requirement (if applicable) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 introduced requirements for SRS antenna port switching feature in Rel-17 and the corresponding Interrption requirements on NR serving cells are need to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of interruption requirements to NR serving cell due to SRS antenna port switching, and impacts to other NR RRM requirement (if applicable) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interruption requirements to NR serving cell due to SRS antenna port switching will be missing in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2.18, 8.2.2.2.16, 8.2.3.2.16, 8.2.4.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2205837 |  |  |  |  |  |  |  |  |

<Start of Change 1>

##### 8.2.1.2.18	 Interruptions at NR SRS antenna port switching

The requirements in this clause are applicable to SRS antenna port switching on FR1.

The UE shall perform SRS antenna port switching only if the below conditions are met.

-	 the SRS switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26] if the carrier on which the higher priority transmission is performed is one entry of txSwitchWithAnotherBand or is the same carrier on which SRS is transmitted.

-     the SRS switching is not colliding with any NR measurements (i.e., SSB/CSI-RS based L1/L3 measurements) and the measurements for RLM/BFD/CBD if the carrier on which the NR measurements and the measurements for RLM/BFD/CBD is performed is one entry of txSwitchImpactToRx or is the same carrier on which SRS is transmitted.

No requirements apply to the aperiodic L1-RSRP/L1-SINR measurements configured if the carrier on which the aperiodic L1-RSRP/L1-SINR configured is one entry of txSwitchImpactToRx or is the same carrier on which aperiodic SRS is scheduled/configured.

When SRS antenna port switching is performed, interruption requirements does not depend on per-FR gap.

For interruption caused by SRS antenna port switching, the victim CC would be based on the band groups signaled in txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability. e.g., an UL interruption is allowed on any of the serving cells belonging to the same group to the SRS carrier as indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells belongs to the same group to the SRS carrier as indicated by txSwitchImpactToRx.

When 1 SRS symbol is configured in a slot for SRS antenna switching and the aggresor and victim carriers are synchronized, the interruption requirement in Table 8.2.1.2.18-1 applies. When 1 SRS symbol is configured in a slot for SRS antenna switching and the aggressor and victim carriers are asynchronized, the interruption requirement in Table 8.2.1.2.18-2 applies. For the rest of SRS configurations, the interruption requirement in Table 8.2.1.2.18-3 applies.

Table 8.2.1.2.18-1: Interruption length in symbols when 1 SRS symbol is configured and aggressor and victim CC are synchronized

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 3 | 2 | 2 |
| 30 | 4 | 3 | 3 |
| 60 | 8 | 6 | 5 |
| 120 | 14 | 10 | 8 |

Table 8.2.1.2.18-2: Interruption length in slots when 1 SRS symbol is configured and aggressor and victim CC are asynchronized

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 2 | 2 | 2 |
| 120 | 2 | 2 | 2 |

Table 8.2.1.2.18-3: Interruption length in slots for rest of the SRS configurations for syncronised and asynchronized scenarios

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

<End of Change 1>

<Start of Change 2>

##### 8.2.2.2.16	 Interruptions at NR SRS antenna port switching

The requirements in this clause are applicable to SRS antenna port switching on FR1.

The interruption requirement in this clause is defined based on the band combination capability reported by UE, i.e., based on txSwitchImpactToRx or txSwitchWithAnotherBand.

The UE shall perform SRS antenna port switching only if the below conditions are met.

-	 the SRS switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26] if the carrier on which the higher priority transmission is performed is one entry of txSwitchWithAnotherBand or is the same carrier on which SRS is transmitted.

-     the SRS switching is not colliding with any NR measurements (i.e., SSB/CSI-RS based L1/L3 measurements) and the measurements for RLM/BFD/CBD if the carrier on which the NR measurements and the measurements for RLM/BFD/CBD is performed is one entry of txSwitchImpactToRx or is the same carrier on which SRS is transmitted.

-	 the SRS switching is not colliding with E-UTRA measurement if the carrier on which the E-UTRA measurement is performed is one entry of txSwitchImpactToRx or is the same carrier on which SRS is transmitted.

No requirements apply to the aperiodic L1-RSRP/L1-SINR measurements configured if the carrier on which the aperiodic L1-RSRP/L1-SINR configured is one entry of txSwitchImpactToRx or is the same carrier on which aperiodic SRS is scheduled/configured.

When SRS antenna port switching is performed, interruption requirements does not depend on per-FR gap.

For interruption caused by SRS antenna port switching, the victim CC would be based on the band groups signaled in txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability. e.g., an UL interruption is allowed on any of the serving cells belongs to the same group to the SRS carrier as indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells belongs to the same group to the SRS carrier as indicated by txSwitchImpactToRx.

When 1 SRS symbol is transmitted and the aggresor and victim carriers are synchronized, the interruption requirement in Table 8.2.2.2.16-1 applies. When 1 SRS symbol is transmitted and the aggressor and victim carriers are asynchronized, the interruption requirement in Table 8.2.2.2.16-2 applies. For the rest of configurations, the interruption requirement in Table 8.2.2.2.16-3 applies.

Table 8.2.2.2.16-1: Interruption length in symbols when 1 SRS symbol is configured and aggressor and victim CC are synchronized

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 3 | 2 | 2 |
| 30 | 4 | 3 | 3 |
| 60 | 8 | 6 | 5 |
| 120 | 14 | 10 | 8 |

Table 8.2.2.2.16-2: Interruption length in slots when 1 SRS symbol is configured and aggressor and victim CC are asynchronized

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 2 | 2 | 2 |
| 120 | 2 | 2 | 2 |

Table 8.2.2.2.16-3: Interruption length in slots for rest of the SRS configurations for syncronised and asynchronized scenarios

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

<End of Change 2>

<Start of Change 3>

##### 8.2.3.2.16	 Interruptions at NR SRS antenna port switching

The requirements in this clause are applicable to SRS antenna port switching on FR1.

The interruption requirement in this clause is defined based on the band combination capability reported by UE, i.e., based on txSwitchImpactToRx or txSwitchWithAnotherBand.

The UE shall perform SRS antenna port switching only if the below conditions are met.

-	 the SRS switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26] if the carrier on which the higher priority transmission is performed is one entry of txSwitchWithAnotherBand or is the same carrier on which SRS is transmitted.

-     the SRS switching is not colliding with any NR measurements (i.e., SSB/CSI-RS based L1/L3 measurements) and the measurements for RLM/BFD/CBD if the carrier on which the NR measurements and the measurements for RLM/BFD/CBD is performed is one entry of txSwitchImpactToRx or is the same carrier on which SRS is transmitted.

No requirements apply to the aperiodic L1-RSRP/L1-SINR measurements configured if the carrier on which the aperiodic L1-RSRP/L1-SINR configured is one entry of txSwitchImpactToRx or is the same carrier on which aperiodic SRS is scheduled/configured.

When SRS antenna port switching is performed, interruption requirements does not depend on per-FR gap.

For interruption caused by SRS antenna port switching, the victim CC would be based on the band groups signaled in txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability. e.g., an UL interruption is allowed on any of the serving cells belongs to the same group to the SRS carrier as indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells belongs to the same group to the SRS carrier as indicated by txSwitchImpactToRx.

When 1 SRS symbol is transmitted and the aggresor and victim carriers are synchronized, the interruption requirement in Table 8.2.3.2.16-1 applies. When 1 SRS symbol is transmitted and the aggressor and victim carriers are asynchronized, the interruption requirement in Table 8.2.3.2.16-2 applies. For the rest of configurations, the interruption requirement in Table 8.2.3.2.16-3 applies.

Table 8.2.3.2.16-1: Interruption length in symbols when 1 SRS symbol is configured and aggressor and victim CC are synchronized

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 3 | 2 | 2 |
| 30 | 4 | 3 | 3 |
| 60 | 8 | 6 | 5 |
| 120 | 14 | 10 | 8 |

Table 8.2.3.2.16-2: Interruption length in slots when 1 SRS symbol is configured and aggressor and victim CC are asynchronized

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 2 | 2 | 2 |
| 120 | 2 | 2 | 2 |

Table 8.2.3.2.1-3: Interruption length in slots for rest of the SRS configurations for syncronised and asynchronized scenarios

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

<End of Change 3>

<Start of Change 4>

##### 8.2.4.2.14	 Interruptions at NR SRS antenna port switching

The requirements in this clause are applicable to SRS antenna port switching on FR1.

The interruption requirement in this clause is defined based on the band combination capability reported by UE, i.e., based on txSwitchImpactToRx or txSwitchWithAnotherBand.

The UE shall perform SRS antenna port switching only if the below conditions are met.

-	 the SRS switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26] if the carrier on which the higher priority transmission is performed is one entry of txSwitchWithAnotherBand or is the same carrier on which SRS is transmitted.

-     the SRS switching is not colliding with any NR measurements (i.e., SSB/CSI-RS based L1/L3 measurements) and the measurements for RLM/BFD/CBD if the carrier on which the NR measurements and the measurements for RLM/BFD/CBD is performed is one entry of txSwitchImpactToRx or is the same carrier on which SRS is transmitted.

-	 the SRS switching is not colliding with E-UTRA measurement if the carrier on which the E-UTRA measurement is performed is one entry of txSwitchImpactToRx or is the same carrier on which SRS is transmitted.

No requirements apply to the aperiodic L1-RSRP/L1-SINR measurements configured if the carrier on which the aperiodic L1-RSRP/L1-SINR configured is one entry of txSwitchImpactToRx or is the same carrier on which aperiodic SRS is scheduled/configured.

When SRS antenna port switching is performed, interruption requirements does not depend on per-FR gap.

For interruption caused by SRS antenna port switching, the victim CC would be based on the band groups signaled in txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability. e.g., an UL interruption is allowed on any of the serving cells belongs to the same group to the SRS carrier as indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells belongs to the same group to the SRS carrier as indicated by txSwitchImpactToRx.

When 1 SRS symbol is transmitted and the aggresor and victim carriers are synchronized, the interruption requirement in Table 8.2.4.2.14-1 applies. When 1 SRS symbol is transmitted and the aggressor and victim carriers are asynchronized, the interruption requirement in Table 8.2.4.2.14-2 applies. For the rest of configurations, the interruption requirement in Table 8.2.4.2.14-3 applies.

Table 8.2.4.2.14-1: Interruption length in symbols when 1 SRS symbol is configured and aggressor and victim CC are synchronized

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 3 | 2 | 2 |
| 30 | 4 | 3 | 3 |
| 60 | 8 | 6 | 5 |
| 120 | 14 | 10 | 8 |

Table 8.2.4.2.14-2: Interruption length in slots when 1 SRS symbol is configured and aggressor and victim CC are asynchronized

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 2 | 2 | 2 |
| 120 | 2 | 2 | 2 |

Table 8.2.4.2.14-3: Interruption length in slots for rest of the SRS configurations for syncronised and asynchronized scenarios

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

<End of Change 4>