3GPP TSG-RAN WG4 Meeting #102-e	R4-2207009

# Electronic Meeting, 21 Februry – 3 March, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS38.133 interruptions due to SCG activation/deactivation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-02-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Interruption due to SCG activation/deactivation has been widely discussed in RAN4. This CR is to introduce corresponding requirements. This draft CR is a placeholder. Changes in RAN4#102e are highlighted in yellow, which depends on final conclusion on corresponding issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce requirements for interruption due to SCG activation/deactivation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for interruption due to SCG activation/deactivation would still be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

##### 8.2.4.2.x	Interruptions at SCG activation/deactivation

When SCG is activated or deactivated using an RRCConnectionReconfiguration message as defined in TS 38.331 [2], the UE is allowed an interruption on any activated serving cell in MCG during the RRC reconfiguration procedure as follows:

-	an interruption on any active serving cell in MCG:

-	of up to the duration shown in table 8.2.4.2.14-1, if the active serving cell is not in the same band as the PSCell or SCells being activated or deactivated, where the requriements for Sync apply for synchronous NR-DC, and for asynchronous NR-DC if the active serving cell is in the same CG as all of the PSCell and SCells being activated or deactivated added or released. The requriements for Async apply for asynchronous NR-DC. if the active serving cell is not in the same CG as any of the PSCell or SCells being activated or deactivated added or released, or

Table 8.2.4.2.x-1: Interruption duration for SCG activation/deactivation for inter-band DC/CA

|  | NR Slot length (ms) | Interruption length (slots) |  |  |
| --- | --- | --- | --- | --- |
|  | of victim cell | Sync |  | Async |
| 0 | 1 | TBD |  | TBD |
| 1 | 0.5 | TBD |  | TBD |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | TBD | TBD |
|  |  | Either aggressor cell or victim cell is on FR1 | TBD |  |
| 3 | 0.125 | Aggressor cell is on FR2 | TBD | TBD |
|  |  | Aggressor cell is on FR1 | TBD |  |

End of Change