3GPP TSG-RAN WG4 Meeting #102-e	R4-2207034

Online Meeting, February 21 – March 3, 2022

Agenda item:	12.9.4

Source: 	Huawei, HiSilicon

Title: 		WF on RRM requirements for Rel-17 NB-IoT and LTE-MTC

Document for:	Approval

- Introduction

This document contains the agreements and discussion in RAN4#102-e in email thread [102-e][237] NB_IOTenh4_LTE_eMTC6_RRM _NWM.

- Topic#1 Neighbour cell measurements in connected mode before RLF or NB-IoT

1.2.1	Sub-topic 1-1 inter-frequency measurement requirement

Issue 1-1-1: Minimum length of occasion for cell detection and measurement

Agreement:

Minimum length of occasion for measurements: 50 ms

Minimum length of occasion for cell detection: 200 ms

Issue 1-1-2: Issue 1-1-2: Conditions of maximum value of Tmeasure_inter and Tdetect_inter

Discuss following option in maintenance stage:

The inter-frequency measurement requirements when Tmeasure_inter ≤ 50 sec. The UE is not required to continue trying to measure cells in an inter-frequency carrier if it has not been able to collect the minimum required number of samples after that amount of time.

The inter-frequency detection requirements apply when Tdetect_inter ≤ 60 sec. The UE is not required to continue trying to detect cells in an inter-frequency carrier if it has not been able to collect the minimum required number of samples after that amount of time.

1.2.2	Sub-topic 1-2 Continue measurement on neighbour cell detected in RRC_IDLE/INACTIVE modes

Issue 1-2-1: Continue measurement on neighbour cell detected in RRC_IDLE/INACTIVE modes

Agreement:

Follow the neighbour cell measurement defined in RAN2, and whether UE continues measurement on cells detected in RRC_IDLE/INACTIVE mode is left to UE implementation without spec impact

1.2.3 Sub-topic 1-3 Additional triggering conditions

Issue 1-3-1: Whether to have additional triggering conditions

Agreement:

Do not define additional triggering conditions

3. References

[1] R4-2207078	Email discussion summary for [101bis-e][229] NB_IOTenh4_LTE_eMTC6_RRM _NWM.