3GPP TSG-RAN WG4 Meeting # 102-e 										R4-2207108

Electronic Meeting, Feb 21st - Mar 3rd, 2022

Agenda item:			10.19.3.2

Source:	Samsung

Title:	WF on RRM requirements for inter-cell beam management

Document for:	Approval

Introduction

This contribution is to capture the agreements for the email discussion for Rel-17 FeMIMO RRM w.r.t. inter-cell beam management in RAN4 #102-e meeting.

Agreements During the Meeting

GTW agreements

- Do not define requirements for measurement on NSC inside SMTC for FR2 for the case “TSSB,NSC ≥ TSMTC”

- RRM requirements for UE L1-RSRP measurements on NSC are defined for known NSC case only for FR1 and FR2

- For FR2, introduce sharing factor for SC and NSC and Nmax =1 (no requirement for Nmax >1)

| # | Scenario | PSC | PNSC |
| --- | --- | --- | --- |
| 1 | TSSB,SC = TSSB,NSC < TSMTC | [2] | [2] |
| 2 | TSSB,NSC < TSSB,SC = TSMTC | 1 | 1 |
| 3 | TSSB,SC < TSSB,NSC < TSMTC | 11-TSSB,SCTSSB,NSC | 1 |
| 4 | TSSB,NSC < TSSB,SC < TSMTC | 1 | 11-TSSB,NSCTSSB,SC |
| 5 | TSSB,NSC = TSMTC, | No L1-RSRP requirement applied. |  |

Agreements during the meeting

- Configure L1-RSRP measurement on a cell with different PCI for UE

  - Network shall configure L3 measurement on a cell with different PCI before L1-RSRP measurement is configured for UE on that cell.

- Known condition for a cell with different PCI on which L1-RSRP measurement is performed.

  - A cell with different PCI is considered as known provided

    - SSBs from serving cell and the cell with different PCI for L1-RSRP measurement has the same center frequency, SCS and SFN offset;

    - During the last [5]s before L1-RSRP measurement is configured, the UE has sent a valid L3 measurement report for the cell with different PCI;

    - Timing offset between serving cell and the cell with different PCI are within CP.

  - A cell with different PCI is considered as unknown provided the known condition is not meet.

- Assumptions for defining inter-cell L1-RSRP measurement requirement

  - Do not define requirements for measurement on NSC inside SMTC for FR2 for the case “TSSB,NSC ≥ TSMTC”

- Introduce sharing factor for inter-cell L1-RSRP measurement requirement

  - For FR2, introduce sharing factor for serving cell PSC and cell with different PCI PCDP

| # | Scenario | PSC | PCDP |
| --- | --- | --- | --- |
| 1 | TSSB,SC = TSSB,NSC < TSMTC | [2] | [2] |
| 2 | TSSB,NSC < TSSB,SC = TSMTC | 1 | 1 |
| 3 | TSSB,SC < TSSB,NSC < TSMTC | 11-TSSB,SCTSSB,NSC | 1 |
| 4 | TSSB,NSC < TSSB,SC < TSMTC | 1 | 11-TSSB,NSCTSSB,SC |
| 5 | TSSB,NSC >= TSMTC, | No L1-RSRP requirement applied. |  |

- Applicability of RRM requirements for UE L1-RSRP measurements on NSC

  - RRM requirements for UE L1-RSRP measurements on NSC are defined for known NSC case only for FR1 and FR2

  - For FR2, the requirement is applied to Nmax =1 and no requirement for Nmax >1.

- L1-RSRP measurement requirements on cell with different PCI from serving cell

  - Define the measurement period requirement for known cell with different PCI based on existing L1-RSRP measurement for SC and extra sharing factor may be needed.

- UE behaviour assumed for defining L1-RSRP measurement on a cell with different PCI

  - For outside SMTC, UE scheduling availability for serving cell may be introduced.

  - For FR1, L1-RSRP for SC and cell with different PCI can be both measured inside SMTC and can be performed simultaneously; and L1 and L3 measurement on cell with different PCI can be performed simultaneously.

  - For FR2, measurement on SC and cell with different PCI CANNOT be performed by the same beam.

- Whether to define requirements for CSI-RS based L1-RSRP measurement on a cell with different PCI

  - Define the requirement for SSB-based measurement only in Rel-17.

Remaining Open Issues

- RAN4 could further discuss CRs for this topic and improve the quality of the spec in next meeting.

- The following issues are still open but deprioritized in Rel-17 RAN4

  - Further study the possible impact on RAN4 requirement of UE capability for simultaneous detection of time and frequency full-overlapped SSB.

  - Further study whether the MRTD requirement could be reused for cell with different PCI from serving cell.

  - Other issues are not precluded.