3GPP TSG-RAN WG4 Meeting #102-e	R4-2207207

# Electronic Meeting, 21st Feb – 3rd Mar, 2022

Title:	WF on demodulation requirement for Enhancement on Multi-TRP

Source:	Huawei, HiSilicon

Agenda item:			10.19.4

Document for:	Approval

Introduction

This WF capture all agreements and open issues for the following topics in [102-e][330] NR_FeMIMO_Demod.

- Topic #1: Demodulation requirement for Multi-TRP enhancement

  - Sub-topic #1-1 Test Scope

  - Sub-topic #1-2 Test setup for PDCCH requirement

  - Sub-topic #1-3 Test setup for PDSCH requirement

The agreed WFs on demodulation requirement for Enhancement on Multi-TRP in previous meeting is listed as following.

- R4-2203092, RAN4#101bis-e

Topic #1: Demodulation requirement for Multi-TRP enhancement

Sub-topic #1-1 Test Scope

Issue 1-1-1: Whether to define PDCCH requirement for multi-TRP repetition transmission schemes

Candidate options:

- Option 1: Yes

- Option 2: No

Issue 1-1-2: Whether to define PDSCH requirement to verify whether UE is with proper behaviour of rate matching around the two linked PDCCH.

Agreements:

- No PDSCH requirement defined with rate matching around two linked PDCCH

Issue 1-1-3: Whether to define PDSCH requirement for Multi-TRP inter-cell operation

Candidate options:

- Option 1: Yes

  - Option 1a: Introduce test applicable rule between existing Multi-DCI intra-cell M-TRP test case and new test case for inter-cell Multi-DCI PDSCH

  - Option 1b: Define performance requirement for enhancements on multi-TRP inter-cell operation with full-overlapping resource allocation.

- Option 2: No

  - Option 2a: Define applicability for UE that supports “IntCell-Mtrp” feature that if such UE satisfied Rel-16 minimum requirements for PDSCH multi-DCI based transmission scheme, inter-cell operation can be also guaranteed

Sub-topic #1-2 Test setup for PDCCH requirement

Issue 1-2-1: Multi-TRP repetition transmission schemes for PDCCH requirements if introduced

Agreements:

- Only with FDM repetition in FR1

Issue 1-2-2: Simulation Assumption for PDCCH with FDM repetition scheme if introduced

Agreements:

- Companies are encouraged to provide the simulation results with different AL as {2, 4} with 2x2, and 2x4 antenna configuration in the next meeting with FDM, down selection one of AL under condition of operation SNR>-4dB with 4Rx for PDCCH requirement.

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| Repetition transmission schemes | FDM |  |
| CBW | 10 MHz | 40 MHz |
| CORESET RB | 24 | 48 |
| CORESET duration | 2 |  |
| Aggregation level | 2/4 |  |
| Propagation Condition | TDLA30-10 |  |
| Antenna configuration | 2x2, 2x4 |  |
| CCE to REG mapping type | nonInterleaved |  |
| REG bundle size | 6 |  |
| Payload bits(without CRC) | 39 | 41 |
| Test metric | 1% of Pm-dsg (%) |  |

Sub-topic #1-3 Test setup for PDSCH requirement

Issue 1-3-1: Simulation Assumption for PDSCH requirment for inter-cell operation if introudced

Agreements:

- Reusing test parameters of existing Rel-16 multi-DCI based on TRP transmission test case (Table 5.2.2.1.12-2) with different PCI for TP1 and TP2 i.e.

  - Time offset/frequency offset: -0.5us /200Hz for FR1 FDD 15kHz SCS; -0.25us/300Hz for FR1 TDD 30kHz SCS

  - MCS: 64QAM 1/2

  - PCI ID: [0] for TP1, [3] for TP2

  - SSB transmission: SSB 1 for TP1, SSB 2 for TP2

Candidate options:

- Option 1:

  - RB allocation: frequency non-overlapping

  - Reuse the same requirement of Rel-16 Multi-DCI non-overlapped resource allocation for PDSCH requirement with multi-TRP inter-cell operation

  - Introduce test applicability rule between Rel-16 Multi-DCI with non-overlapped Tx schemes and Rel-17 Multi-TRP inter-cell Tx schemes

- Option 2:

  - RB allocation: frequency full-overlapping

Reference

- R4-2207177, Email discussion summary for [102-e][330] NR_FeMIMO_Demod, RAN4#102-e, Samsung

- R4-2203092, WF on demodulation requirement for Enhancement on Multi-TRP, RAN4#101bis-e, Huawei, HiSilicon