3GPP TSG-RAN WG4 Meeting #102-e 	       R4-2207339

# Electronic Meeting, 21 Feb - 03 Mar, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TR 38.863 Regulatory aspects for HAPS

Agenda item: 	10.13.1.3

Document for:	Approval

- Introduction

The latest NTN WI as given [1] was updated at RAN#94. This contribution provides a TP for clause 5 in TR 38.863 [2] to address spectrum regulatory aspects for HAPS mobile service.

- Text proposal

******************************* Start of TP ***************************************

## 5.4	Regulatory aspects for HAPS

ITU–R began to study HAPS in the 1990s initially for fixed services. The telecommunications ecosystem and technology enablers for HAPS have evolved a lot since then. At WRC-2000, the bands 1885 – 1980 MHz, 2010 – 2025 MHz and 2110 – 2170 MHz were identified for HAPS operating as IMT base stations and may be used by high altitude platform stations as base stations to provide International Mobile Telecommunications (IMT), in accordance with Resolution 221 (Rev.WRC-07) [3]. With increasing interest in HAPS to offer mobile services, the WRC-19 agreed to study certain frequency bands below 2.7 GHz for HAPS as IMT Base Stations (HIBS). ITU Working Party 5D is currently studying co-existence requirements for HIBS in the additional three bands listed in Table 5.4-1 to support spectrum allocation decisions in WRC-23 in accordance with Resolution 247 [3].

NOTE:	In the current ITU terminology, the use of HAPS to implement IMT (i.e. offer mobile wireless services) is referred to as “HAPS as IMT Base Stations” or HIBS.

Table 5.4-1: Frequencies for HAPS IMT Base Stations (HIBS).

| Region | Spectrum | Remarks |
| --- | --- | --- |
| Region 1 and 3 | 1885 – 1980 MHz 2010 – 2025 MHz 2110 – 2170 MHz | Identified HIBS designations at WRC-07 |
| Region 2 | 1885 – 1980 MHz (NOTE 3) 2110 – 2160 MHz (NOTE 4) |  |
| Global | 694 – 960 MHz 1710 – 1885 MHz (NOTE 1) 2500 –2690 MHz (NOTE 2) | Under study, for decision at WRC-23 |
| NOTE 1: 1710-1815 MHz to be used for uplink only in Region 3. NOTE 2: 2500--2535 MHz to be used for uplink only in Region 3, except 2655--2690 MHz in Region 3). NOTE 3: In most of Region 2 1885-1910 is used for Mobile uplink and 1930-1980 is used for Mobile downlink. In the United States, 1910-1915 and 1915-1920 are also used for Mobile uplink. NOTE 4: In Region 2 2110-2160 is used for Mobile downlink. |  |  |

******************************* End of TP ***************************************

# References

- RP-213691, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2203132 3GPP TR 38.863 V0.2.0, "Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects".