3GPP TSG-RAN WG4 Meeting #102-e    				     R4-2207389

Electronic Meeting, 21st of February - 3rd of March, 2022

Agenda item:	     10.13.1.2

Source:	THALES

Title:	SAN spurious emissions (Draft text proposal for spurious emissions - TS 38.108 and TR 38.863)

Document for:	Approval

# Introduction

Draft TS skeleton for Non-terrestrial networks (NTN) related to NR; Satellite Access Node radio transmission and reception (TS 38.108) has been approved in R4-2203087 [1] and the work split has been summarized in R4-2203080 Way Forward on NTN_solutions_Part1 [2] approved in RAN4 101-bis-e meeting. For further information, please also follow the discussions in R4-2203111 [3].

In this document, following changes have been proposed as a TP to update TS 38.108:

- Section x.x: SAN spurious emissions

In this contribution, we follow methodology provided in R4-2205468 (“Further discussion on conducted Tx requirements of satellite access node”, ZTE) and R4-2205046 (“NTN - SAN TX requirement proposals”, Ericsson), and adapted them by taking into account the following RAN4 102-e GTW decisions (23/02/2022):

Agreement GTW discussion [102-e][308]:

Introduce SAN classes in Rel-17: LEO and GEO

Further discuss the requirements associated with SAN classes in case by case manner including [IoT level/ICS, NF, ACLR/ACS, Emission]

Note: Introducing additional classes in future release not precluded.

Agreement GTW discussion [102-e][310]:

Measurement bandwidth: 4kHz

General principle: Follow ITU recommendation 15-41 (SM.1541) at least for the frequency offset range within first two break points

Define two set requirements: one for LEO SAN class and one for GEO SAN class

GTW discussion with respect to SAN spurious emissions [102-e][310]:

Further discuss reference, basic limit and measurement BW.

As a reminder, R4-2205468 and R4-2205046 provided the following information:

| Spurious frequency range | Basic limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150 kHz |  | 1 kHz | Note 1 |
| 150 kHz – 30 MHz |  | 10 kHz | Note 1 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz - 12.75 GHz | -13dBm | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz |  | 1 MHz | Note 1, Note 2, Note 3 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. |  |  |  |

For the reasons mentioned above, we propose new spurious emissions limit values for SAN, taking into account SAN class definition and new measurement bandwidth values.

# References

[1] R4-2203087 Skeleton for TS 38.108 NR Satellite Access Node radio transmission and reception v0.0.1, THALES.

[2] R4-2203080 Way Forward on NTN_solutions_Part1, THALES.

[3] R4-2203111 Email discussion summary for [101-bis-e][306] NTN_Solutions_Part1, THALES.

# Spurious Emissions SAN Proposals

Based on 2 class SAN definition, 4 kHz measurement bandwidth (as in SM.1541-6), spurious emission definition (as in SM.239-12), SAN power of 51 dBm in 20 MHz (GEO class) and 53 dBm in 20 MHz (LEO class), we further define the SAN Spurious Emissions requirement for GEO and LEO classes as presented below.

For satellites operating in MSS S-Band n256 and MSS L-band n255, the following frequency range for measurements should be applied:

Table 1. Frequency range for measurements

| Fundamental frequency range | Frequency range for measurements |  |
| --- | --- | --- |
|  | Lower limit | Upper limit (The test should include the entire harmonic band and not be truncated at the precise upper frequency limit stated) |
| 600 MHz - 5.2 GHz | 800 MHz | 11 GHz (5th harmonic) |

With the following justifications and comments:

- The lower limit of 30 MHz is replaced as per ITU-R SM.329-12: « Systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off wavelength, do not require spurious domain emission measurements below 0.7 times the waveguide cut-off frequency. »

- This lower limit (800 MHz) covers the 0.7 times the standards cut-off frequency of WR510, WR340 and WR430 waveguides to be used for L-Band and S-Band SAN type 1-H. As an example, the cut-off frequency of lowest order mode for WR510 waveguide is 1.157 GHz, and therefore 0.7x1.157 GHz = 808.9 MHz > 800 MHz proposed lower limit.

- The upper limit of 11 GHz corresponds to the 5th harmonic of maximum frequency in S-Band (2200 MHz).

- Tx spurious limit value shall not be applied within the transmit L-Band and S-Band.

Please also note that in SM.329-12 the 43+log(P) attenuation is applicable only if transmitter power < 50W, and attenuation of 60 dBc is applicable for transmitter power > 50W. Therefore, the limit of -13 dBm applies only in the case of transmitter power lower than 50 W, otherwise the spurious limit has to be computed accordingly for transmitter power higher than 50 W.

Using SM.329-12 for P > 50W (4th row from Table 10), one can obtain:

1) -9 dBm (i.e. 21-30 [dBm] = -9 dBm) spurious emission power over 4 kHz measurement bandwidth for the GEO class, and

2) -7 dBm (i.e. 23-30 [dBm] = -7 dBm) spurious emission power over 4 kHz measurement bandwidth for the LEO class.

This computation indicates two spurious emission limits with respect to the GEO and LEO classes.

For all these reasons, one can obtain the following SAN Spurious Emissions limits for GEO and LEO classes as presented in the following table.

Table 2. Basic spurious emission limit for GEO and LEO SAN classes

| Spurious frequency range | Basic limit GEO class (dBm) | Basic limit LEO class (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 800 MHz - 1508 MHz | -9 | -7 | 4 kHz | Notes 1,2,3 |
| 1508 MHz - 1513 MHz | 1 Covered by OBUE requirement | -5 Covered by OBUE requirement | 4 kHz | Note 1 |
| 1513 MHz -1518 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 1518 MHz - 1559 MHz | NA: L-Band transmit Bandwidth |  | 4 kHz | Note 1 |
| 1559 MHz - 1564 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 1564 MHz - 1569 MHz | 1 Covered by OBUE requirement | -5 Covered by OBUE requirement | 4 kHz | Note 1 |
| 1569 MHz - 2160 MHz | -9 | -7 | 4 kHz | Notes 1,3 |
| 2160 MHz - 2165 MHz | 1 Covered by OBUE requirement | -5 Covered by OBUE requirement | 4 kHz | Note 1 |
| 2165 MHz - 2170 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 2170 MHz - 2200 MHz | NA: S-Band transmit Bandwidth |  | 4 kHz | Note 1 |
| 2200 MHz - 2205 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 2205 MHz- 2210 MHz | 1 Covered by OBUE requirement | -5 Covered by OBUE requirement | 4 kHz | Note 1 |
| 2210 MHz- 11000 MHz | -9 | -7 | 4 kHz | Notes 1,2 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Lower and Upper frequency as in ITU-R SM.329 [x], s2.5 table 1. NOTE 3: As per ITU-R SM.329 [x], Spurious requirement is -60 dBc with respect to useful operating power P: 51-60= -9 dBm for GEO class and 53-60= -7 dBm for LEO class. |  |  |  |  |

With the simplified version of Table 2 represented in Table 3:

Table 3. Basic spurious emission limit for GEO and LEO SAN classes

| Spurious frequency range | Basic limit GEO class (dBm) | Basic limit LEO class (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 800 MHz - 1508 MHz | -9 | -7 | 4 kHz | Notes 1,2,3 |
| 1508 MHz - 1518 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 1518 MHz - 1559 MHz | NA: L-Band transmit Bandwidth |  | 4 kHz | Note 1 |
| 1559 MHz - 1569 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 1569 MHz - 2160 MHz | -9 | -7 | 4 kHz | Notes 1,3 |
| 2160 MHz - 2170 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 2170 MHz - 2200 MHz | NA: S-Band transmit Bandwidth |  | 4 kHz | Note 1 |
| 2200 MHz - 2210 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 2210 MHz- 11000 MHz | -9 | -7 | 4 kHz | Notes 1, 2 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Lower and Upper frequency as in ITU-R SM.329 [x], s2.5 table 1. NOTE 3: As per ITU-R SM.329 [x], Spurious requirement is -60 dBc with respect to useful operating power P: 51-60= -9 dBm for GEO class and 53-60= -7 dBm for LEO class. |  |  |  |  |

# Text proposals for TS 38.108

------------------------------------------------<Start of TP>----------------------------------------------

## x.x		Spurious emissions for SAN

For satellites operating in MSS S-Band n256 and MSS L-band n255, the frequency range for measurements shall be applied as in Table x.x-1.

Table x.x-1. Frequency range for measurements

| Fundamental frequency range | Frequency range for measurements |  |
| --- | --- | --- |
|  | Lower limit | Upper limit (The test should include the entire harmonic band and not be truncated at the precise upper frequency limit stated) |
| 600 MHz - 5.2 GHz | 800 MHz | 11 GHz (5th harmonic) |

SAN spurious emissions limits for GEO and LEO classes with respect to MSS S-Band n256 and MSS L-band n255 are defined as described in Table x.x-2.

Table x.x-2. Basic spurious emission limit for GEO and LEO SAN classes

| Spurious frequency range | Basic limit GEO class (dBm) | Basic limit LEO class (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 800 MHz - 1508 MHz | -9 | -7 | 4 kHz | Notes 1,2,3 |
| 1508 MHz - 1518 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 1518 MHz - 1559 MHz | NA: L-Band transmit Bandwidth |  | 4 kHz | Note 1 |
| 1559 MHz - 1569 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 1569 MHz - 2160 MHz | -9 | -7 | 4 kHz | Notes 1,3 |
| 2160 MHz - 2170 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 2170 MHz - 2200 MHz | NA: S-Band transmit Bandwidth |  | 4 kHz | Note 1 |
| 2200 MHz - 2210 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 2210 MHz- 11000 MHz | -9 | -7 | 4 kHz | Notes 1, 2 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Lower and Upper frequency as in ITU-R SM.329 [x], s2.5 table 1. NOTE 3: As per ITU-R SM.329 [x], Spurious requirement is -60 dBc with respect to useful operating power P: 51-60= -9 dBm for GEO class and 53-60= -7 dBm for LEO class. |  |  |  |  |

------------------------------------------------<End of TP>-----------------------------------------------

# Text proposals for TR 38.863

------------------------------------------------<Start of TP>----------------------------------------------

## x.x		Spurious emissions for SAN

Based on 2 class SAN definition, 4 kHz measurement bandwidth (as in SM.1541-6), spurious emission definition (as in SM.239-12), SAN power of 51 dBm in 20 MHz (GEO class) and 53 dBm in 20 MHz (LEO class), we further define the SAN Spurious Emissions requirement for GEO and LEO classes as presented below.

For satellites operating in MSS S-Band n256 and MSS L-band n255, the frequency range for measurements shall be applied as in Table x.x-1.

Table x.x-1. Frequency range for measurements

| Fundamental frequency range | Frequency range for measurements |  |
| --- | --- | --- |
|  | Lower limit | Upper limit (The test should include the entire harmonic band and not be truncated at the precise upper frequency limit stated) |
| 600 MHz - 5.2 GHz | 800 MHz | 11 GHz (5th harmonic) |

With the following justifications and comments:

- The lower limit of 30 MHz is replaced as per ITU-R SM.329-12: « Systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off wavelength, do not require spurious domain emission measurements below 0.7 times the waveguide cut-off frequency. »

- This lower limit (800 MHz) covers the 0.7 times the standards cut-off frequency of WR510, WR340 and WR430 waveguides to be used for L-Band and S-Band SAN type 1-H. As an example, the cut-off frequency of lowest order mode for WR510 waveguide is 1.157 GHz, and therefore 0.7x1.157 GHz = 808.9 MHz > 800 MHz proposed lower limit.

- The upper limit of 11 GHz corresponds to the 5th harmonic of maximum frequency in S-Band (2200 MHz).

- Tx spurious limit value shall not be applied within the transmit L-Band and S-Band.

Please also note that in SM.329-12 the 43+log(P) attenuation is applicable only if transmitter power < 50W, and attenuation of 60 dBc is applicable for transmitter power > 50W. Therefore, the limit of -13 dBm applies only in the case of transmitter power lower than 50 W, otherwise the spurious limit has to be computed accordingly for transmitter power higher than 50 W.

Using SM.329-12 for P > 50W (4th row from Table 10), one can obtain:

1) -9 dBm (i.e. 21-30 [dBm] = -9 dBm) spurious emission power over 4 kHz measurement bandwidth for the GEO class, and

2) -7 dBm (i.e. 23-30 [dBm] = -7 dBm) spurious emission power over 4 kHz measurement bandwidth for the LEO class.

This computation indicates two spurious emission limits with respect to the GEO and LEO classes.

For all these reasons, one can obtain the following SAN Spurious Emissions limits for GEO and LEO classes as represented in Table x.x-2.

Table x.x-2. Basic spurious emission limit for GEO and LEO SAN classes

| Spurious frequency range | Basic limit GEO class (dBm) | Basic limit LEO class (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 800 MHz - 1508 MHz | -9 | -7 | 4 kHz | Notes 1,2,3 |
| 1508 MHz - 1513 MHz | 1 Covered by OBUE requirement | -5 Covered by OBUE requirement | 4 kHz | Note 1 |
| 1513 MHz -1518 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 1518 MHz - 1559 MHz | NA: L-Band transmit Bandwidth |  | 4 kHz | Note 1 |
| 1559 MHz - 1564 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 1564 MHz - 1569 MHz | 1 Covered by OBUE requirement | -5 Covered by OBUE requirement | 4 kHz | Note 1 |
| 1569 MHz - 2160 MHz | -9 | -7 | 4 kHz | Notes 1,3 |
| 2160 MHz - 2165 MHz | 1 Covered by OBUE requirement | -5 Covered by OBUE requirement | 4 kHz | Note 1 |
| 2165 MHz - 2170 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 2170 MHz - 2200 MHz | NA: S-Band transmit Bandwidth |  | 4 kHz | Note 1 |
| 2200 MHz - 2205 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 2205 MHz- 2210 MHz | 1 Covered by OBUE requirement | -5 Covered by OBUE requirement | 4 kHz | Note 1 |
| 2210 MHz- 11000 MHz | -9 | -7 | 4 kHz | Notes 1,2 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Lower and Upper frequency as in ITU-R SM.329 [x], s2.5 table 1. NOTE 3: As per ITU-R SM.329 [x], Spurious requirement is -60 dBc with respect to useful operating power P: 51-60= -9 dBm for GEO class and 53-60= -7 dBm for LEO class. |  |  |  |  |

Table x.x-3. Basic spurious emission limit for GEO and LEO SAN classes – simplified view

| Spurious frequency range | Basic limit GEO class (dBm) | Basic limit LEO class (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 800 MHz - 1508 MHz | -9 | -7 | 4 kHz | Notes 1,2,3 |
| 1508 MHz - 1518 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 1518 MHz - 1559 MHz | NA: L-Band transmit Bandwidth |  | 4 kHz | Note 1 |
| 1559 MHz - 1569 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 1569 MHz - 2160 MHz | -9 | -7 | 4 kHz | Notes 1,3 |
| 2160 MHz - 2170 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 2170 MHz - 2200 MHz | NA: S-Band transmit Bandwidth |  | 4 kHz | Note 1 |
| 2200 MHz - 2210 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 2210 MHz- 11000 MHz | -9 | -7 | 4 kHz | Notes 1, 2 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Lower and Upper frequency as in ITU-R SM.329 [x], s2.5 table 1. NOTE 3: As per ITU-R SM.329 [x], Spurious requirement is -60 dBc with respect to useful operating power P: 51-60= -9 dBm for GEO class and 53-60= -7 dBm for LEO class. |  |  |  |  |

------------------------------------------------<End of TP>-----------------------------------------------