# 3GPP TSG-RAN WG4 Meeting # 102-e	R4-2207416

# Electronic Meeting, Feb 26 – Mar 3 2022

Agenda Item:	10.13.4

Source: 	ZTE

Title: 	WF on NTN UE RF requirements

Document for:	Approval

- Way forward

NTN UE Tx requirement:

Issue 1-1-1:   where to capture isolation region (1.5km) information

Agreement:

To capture the descriptions on the assumption of 1.5km isolation region just in TR 38.863. A clarification that TN UEs not deploying in isolation region is for simplifying the simulation

Issue 1-2-1:   minimum output power

Agreement:

Option 2:  reuse the TN minimum output power requirements for NTN UE

Issue 1-3-1:   UE coexistence requirement

Agreement: Option 1 (-50dBm/MHz), exception for specific bands need to be further discussed and decided.

Issue 1-3-2:   protected TN bands for UE co-existence n255/n256

  - Option 1: all TN bands with some exceptional bands

Agreement: Option 1

Issue 1-3-4:   exceptional TN bands for UE co-existence n255

Agreement: No need exception on band n24.

Issue 1-4-1:   MPR

- Proposals

  - Option 1:  to reuse TN MPR requirement

Agreement: Option 1

Issue 1-4-2:   A-MPR for n256

Agreement:

NS_01, NS_24 and NS_100 for n256

Agreement:

Drafting TP based on no exceptional band for band n256;

Issue 1-4-3:   A-MPR for n255

Agreement:

NS_56 is not applicable for band n255.

[New NS valued ([NS_57]) required for additional spurious emission] (further checking by QC)

NTN UE Rx requirement:

Issue 2-1-1: FFS on the Duplexer assumption for n256 with below candidate options

  - Option 1: dedicated 30MHz [QC, CATT, Ericsson, ZTE, Huawei]

  - Option 2:  reusing n65 [ Xiaomi, Skyworks, Hughes/EchoStar, Omnispace, MTK,Intelsat, THALES, LOCKHEED]

NTN UE Rx requirement for REFSENS with [ ] into TP:

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n256 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |

Issue 2-1-2: maximum input power

- Proposals

  - Option 1:  relaxed by 15dB from TN UE

Agreement: Option 1

Issue 2-1-3: ACS requirement in case 2

Agreement: ACS requirements in case 2 can be specified based on the agreed maximum input level if needed.

- Open issues

Issue 1:

Exceptions that shall be considered for n256:

- n34 with NS_24 A-MPR (we need to check if reusing n65 A-MPR is fully applicable). Should there be a table saying that NS_24 and NS_100 applies to n256

  - FFS whether existing A-MPR value with NS_24 can be reused, keep A-MPR value with [ ] in the drafting TP

- Band n2, n25, n70 shall be protected; FFS how to protect n2, n25, n70 since the n256 UL overlap their DL

  - Include band n2, n25, n70 into co-existence table and including note “FFS on how to protect n2, n25, n70”

- Band n39 shall be protected; FFS how to protect band n39 if reusing band n65 filter

  - Including a note into TP “FFS on how to protect n39”

- Band 33, 35 and 37 shall be protected and FFS how to protect band 33,35 and 37

Issue 2:

FFS for OOBB requirements for band n256

Issue 3:

Issue 1-4-3:   A-MPR for n255

Including New NS value NS_57 with additional spurious emission into TP with A-MPR as TBD

RAN4 will further evaluate and conclude A-MPR for this additional spurious emission.