3GPP TSG-RAN WG4 Meeting # 102-bis-e          									     R4-2207456

Electronic Meeting, 21st of February – 3rd of March, 2022

Agenda item:			10.13.3.3

Source:	THALES, ZTE

Title:	Way Forward on SAN SEM and spurious emission

Document for:	Agreement

# Introduction

This proposed way forward is based on the outcomes of “Email discussion summary for [102-e][310] NTN_Solutions_Part3”, see R4-2207440	 (revision of R4-2207167). Moreover, as suggested by “RAN4#102-bis-e E-meeting Arrangements and Guidelines”, this Way Forward uses WORD document rather than POWERPOINT (.PPTX) in order to facilitate others to comment and easily track the changes.

During the BS RF GTW discussion on 28/02/2022, it was decided to discuss the WF with respect to SAN SEM (SAN OBUE and SAN spurious emissions) in this WF.

During the BS RF GTW discussion on 23/02/2022 it was decided to consider 2 classes (1 for LEO and 1 for GEO). The LEO class incorporates both LEO@600km and LEO@1200km.

Agreement GTW discussion [102-e][308]:

Introduce SAN classes in Rel-17: LEO and GEO

Further discuss the requirements associated with SAN classes in case by case manner including [IoT level/ICS, NF, ACLR/ACS, Emission]

Note: Introducing additional classes in future release not precluded.

Agreement GTW discussion [102-e][310]:

Measurement bandwidth: 4kHz

General principle: Follow ITU recommendation 1541 (SM.1541) at least for the frequency offset range within first two break points

Define two set requirements: one for LEO SAN class and one for GEO SAN class

GTW discussion with respect to SAN spurious emissions [102-e][310]:

Further discuss reference, basic limit and measurement BW.

# Agreements

      - OBUE SAN

- Option 1 (ZTE):

Table x.x-1. SAN GEO Class OBUE limit values

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) dBm | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz |  | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | 1 dBm | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | -9dBm | 4kHz |

Table x.x-2. SAN LEO Class OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) dBm | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz |  | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | -13 | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | -13 | 4kHz |

- Option 2 (THALES):

The SAN OBUE requirements for GEO and LEO classes are defined as described in Table x.x.-1 and Table x.x.-2.

- Table x.x-1. SAN GEO Class OBUE limit values

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) dBm | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | 3.0dBm-25foffsetMHz-0.002dB | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | 1 dBm | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | Not applicable for B<100 MHz | 4kHz |

Table x.x-2. SAN LEO Class OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) dBm | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | 5.0dBm-105foffsetMHz-0.002dB | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | -5 dBm | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | Not applicable for B<100 MHz | 4kHz |

- Option 3: Other solutions are not precluded.

      - Spurious emissions SAN

- Option 1 (Ericsson):

| Spurious frequency range | Basic limit GEO class (dBm) | Basic limit LEO class (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 9kHz – 12.75 GHz | [-13] | [-13] | 4 kHz |

FFS how to handle limit when satellite output power > 50W.

- Option 2 (THALES):

For satellites operating in MSS S-Band n256 and MSS L-band n255, the frequency range for measurements shall be applied as in Table x.x-1.

Table x.x-1. Frequency range for measurements

| Fundamental frequency range | Frequency range for measurements |  |
| --- | --- | --- |
|  | Lower limit | Upper limit (The test should include the entire harmonic band and not be truncated at the precise upper frequency limit stated) |
| 600 MHz - 5.2 GHz | 800 MHz | 11 GHz (5th harmonic) |

SAN spurious emissions limits for GEO and LEO classes with respect to MSS S-Band n256 and MSS L-band n255 are defined as described in Table x.x-2.

Table x.x-2. Basic spurious emission limit for GEO and LEO SAN classes

| Spurious frequency range | Basic limit GEO class (dBm) | Basic limit LEO class (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 800 MHz - 1508 MHz | -9 | -7 | 4 kHz | Notes 1,2,3 |
| 1508 MHz - 1518 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 1518 MHz - 1559 MHz | NA: L-Band transmit Bandwidth |  | 4 kHz | Note 1 |
| 1559 MHz - 1569 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 1569 MHz - 2160 MHz | -9 | -7 | 4 kHz | Notes 1,3 |
| 2160 MHz - 2170 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 2170 MHz - 2200 MHz | NA: S-Band transmit Bandwidth |  | 4 kHz | Note 1 |
| 2200 MHz - 2210 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 2210 MHz- 11000 MHz | -9 | -7 | 4 kHz | Notes 1, 2 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Lower and Upper frequency as in ITU-R SM.329 [x], s2.5 table 1. NOTE 3: As per ITU-R SM.329 [x], Spurious requirement is -60 dBc with respect to useful operating power P: 51-60= -9 dBm for GEO class and 53-60= -7 dBm for LEO class. |  |  |  |  |

- Option 3: Other solutions are not precluded.