3GPP TSG-RAN WG4 Meeting #103-e	R4-2207637

e-Meeting, May 2022

Title:	On delta(RIB) for n258+n261 DL inter-CA

Source:	Qualcomm Incorporated

Agenda item:	9.4.2.1.1

Release:	Rel-17

Work Item:	NR_RF_FR2_req_enh2-Core

Responsible WG:	RAN4

Document for:	Discussion

# 1.	Introduction

We propose delta(RIB) for inter-band DLCA over n258+n261, assuming IBM.

# 2. 	Discussion

The discussion below considers the de-prioritization of CBM from FR2 enhancement WI, as agreed in RAN#95-e. The focus can therefore be limited to IBM inter-band DLCA across n258+n261. We retain the IBM assumption of existence of a dedicated RF chain for each band. The proposal below was previously captured in [2] and reproduced here after removal of CBM content.

## 2.1	delta(RIB_spherical)

Recall that for IBM, delta(RIB_sph) for a band pair sourced from different band groups (example: n260+n261) was derived as the cumulative effect of relaxations from multiple mechanisms:

- Relaxation ‘R_overlap’ due to imperfect overlap in spatial coverage of the participating bands

- MBR

- Mutli-chain desense: A de-sense factor from having to keep 2 receiver sets simultaneously operational with practical PSD difference

Our analysis in [1] shows that for an IBM PC3 UE, R_overlap is 1.6 dB in n258+n261. The estimate for R_overlap allows delta(RIB_spherical) to be calculated from the list of mechanisms as:

| Mechanism | PC3, Contribution to delta(RIB_spherical) | Notes |
| --- | --- | --- |
| Imperfect overlap in coverage areas (R_overlap) | 1.6 dB | This parameter is unique to enabling compliance with common spherical coverage requirements. |
| MBR | 0.7 dB |  |
| Multi-chain de-sense | 1.0 dB | For inter- DLCA, de-sense (~ 1 dB) was used to capture impact on SNR when receivers from multiple bands were simultaneously operational |
| Total | 3.3 dB |  |

We round up our estimate:

Proposal 1: For DL CA for n258+n261, delta(RIB_spherical) for PC3 is 3.5 dB for IBM.

## 2.2	delta(RIB_peak)

The derivation of the parameter value for delta(RIB_peak) for bands from different band groups suffers from a lack of common understanding on the list of contributing mechanisms, resulting in a value that is not easy to justify in retrospect. A more transparent approach for deriving the parameter delta(RIB_peak) can be established by first listing all the physical mechanisms that apply. In [1], we identified the list and re-used values from assumptions made elsewhere where possible:

| Mechanism | PC3, Contribution to delta(RIB_peak) | Notes |
| --- | --- | --- |
| Imperfect overlap in coverage areas (R_overlap) | Not applicable | Delta(RIB_peak) only applies to REFSENS, which is a single direction measurement per band. Since no common coverage concept applies, this mechanism is N/A. |
| MBR | 0.7 dB | (unchanged from delta(RIB_spherical) consideration) |
| Multi-chain de-sense | 1.0 dB | (unchanged from delta(RIB_spherical) consideration) |
| Total | 1.7 dB |  |

We round up our estimate:

Proposal 2: For DL CA for n258+n261, delta(RIB_peak) for PC3 is 2.0 dB for IBM.

# 3. 	Conclusion

Proposal 1: For DL CA for n258+n261, delta(RIB_spherical) for PC3 is 3.5 dB for IBM.

Proposal 2: For DL CA for n258+n261, delta(RIB_peak) for PC3 is 2.0 dB for IBM.

# 4. 	References

- R4-2201968, ‘On delta(RIB) for n258+n261 DL inter-CA’, Qualcomm, RAN4#101-Bis-e, Jan. 2022

- R4-2206055, ‘On delta(RIB) for n258+n261 DL inter-CA’, Qualcomm, RAN4#102-e, Feb.-Mar. 2022