3GPP TSG-RAN WG4 Meeting #103-e	R4-2207718

Electronic Meeting, 09th May – 20th May 2022

Source:	AT&T

Title:	TP for TR 37.717-21-11 Addition of DC_14-66_n5

Agenda item:			8.12.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.717-21-11 to add DC_14-66_n5.

# 2. Text Proposal

---Start of changes---

## 5.X	DC_14-66_n5

### 5.X.1	Configurations for DC

Table 5.X.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_14A-66A_n5A | DC_14A_n5A DC_66A_n5A |
| DC_14A-66A-66A_n5A | DC_14A_n5A DC_66A_n5A |

### 5.X.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be seen that:

-	There are no IMD products produced by Band 14 and n5 that might fall in Rx of band 66

-	There are no IMD products produced by Band 66 and n5 that might fall in Rx of band 14.

### 5.X.3	∆TIB and ∆RIB values

For DC_14-66_n5, the TIB,c and RIB,c values are given in the tables below and are derived from the values for DC_12-66_n5.

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-66_n5 DC_14-66-66_n5 | 14 | 0.8 |
|  | 66 | 0.8 |
|  | n5 | 0.3 |

Table 5.X.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_14-66_n5 DC_14-66-66_n5 | 14 | 0.5 |
|  | 66 | 0.5 |
|  | n5 | 0 |

### 5.X.4	Reference sensitivity exceptions

There is no IMD impact from UL 14_n5 affecting DL band 66 and there is no IMD impact from UL 66_n5 affecting DL band 14. Therefore, MSD is not needed to be defined.

---End of changes---

# References