3GPP TSG-RAN WG4 Meeting #103-e	R4-2207721

Electronic Meeting, 09th May – 20th May 2022

Source:	AT&T

Title:	TP for TR 37.717-41-11 Addition of DC_2-30-66-(n)5

Agenda item:			8.14.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.717-41-11 to add DC_2-30-66-(n)5.

# 2. Text Proposal

---Start of changes---

### 5.1.X	DC_2-30-66-(n)5

#### 5.1.X.1	Configuration for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_2A-30A-66A-(n)5AA | DC_2A_n5A DC_30A_n77A DC_66A_n77A DC_(n)5AA4 |
| NOTE 4: Only single switched UL is supported. |  |

#### 5.1.X.2	∆TIB and ∆RIB values

For DC_2-30-66-(n)5, the TIB,c and RIB,c values are derived from DC_2-30-66_n5 and are given in the tables below.

Table 5.1.X.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-30-66-(n)5 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |

Table 5.1.X.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-30-66-(n)5 | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n5 | 0 |

#### 5.1.X.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

---End of changes---

# References