3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207730

# Electronic Meeting, May 09 – May 20, 2022

Agenda item:	9.9.2.1

Source: 	Qualcomm Incorporated

Title: 	On SRS antenna switching Test

Document for:	Discussion

# Introduction

In this contribution, we present our view on SRS antenna port switching test scope and configurations.

# Discussion

## Test Scope

There are three sets of interruption requirement defined in core discussion. Since Table 8.2.1.2.18-3 covers most configurations and is testable by counting missing ACK/NACK, we propose to define the test based on Table 8.2.1.2.18-3 requirement. We may need tests for both EN-DC ans NR-SA to verify the interruption lengths in the two scenarios, and therefore we propose to introduce two tests in total to verify the requirement in Table 8.2.1.2.18-3 in 38.133 and 2 subframes on LTE.

NE-DC tests have been discussed in RAN4#101e meeting, and only the following tests are agreed [1]:

- Develop following NE-DC RRM test cases for FR1 PCell+E-UTRAN PSCell in Rel-15.

| No. | Test case | Core requirements |
| --- | --- | --- |
| 1 | LTE PSCell addition in NE-DC | 38.133 section 8.8.2 38.133 section 8.2.3.2.3 |
| 2 | SFTD measurement accuracy for NE-DC | 38.133 section 10.1.21.1 |
| 3 | NE-DC active BWP switch | 36.133 section 7.36.2.6 |

- Applicability rule for each of test cases #2 and #3 for UE supporting both EN-DC and NE-DC is as follows:

  - UE capable of both EN-DC and NE-DC operations needs to be tested with one of the tests in either EN-DC or NE-DC operations

The principle is that we introduce NE-DC test in addition to corresponding EN-DC test only when there is major functional difference from requirement perspective. For SRS carrier switching, switchings on MCG and SCG are identical from interruption perspective, and we believe EN-DC test is sufficient.

Proposal 1: Define two tests based on the requirement in Table 8.2.1.2.18-3, 8.2.2.2.16-3 (more than 1 SRS symbol) in 38.133 and 2 subframes on LTE (more than 1 SRS symbol configured) in NR-SA and EN-DC, but not NE-DC.

For simplicity and avoid one SRS source set span to multiple slots, we propose to test 1T2R configuration.

Proposal 2: Use 1T2R and the following configurations.

Sounding Reference Symbol Configuration for SCS=15kHz

|  | SRS.x1 TDD |  |
| --- | --- | --- |
| Field | Value | Comment |
| c-SRS | 12 |  |
| b-SRS | 0 |  |
| b-hop | 0 | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching |  |
| startPosition | 5 | resourceMapping setting |
| nrofSymbols | 2 |  |
| repetitionFactor | n1 | without repetition. |
| transmissionComb | n2 |  |
| combOffset | 0 | transmissionComb setting |
| cyclicShift | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| resourceType | Periodic |  |
| periodicityAndOffset-p | sl40, 2 | SRS transmission periodicity |

Table A.3.24-2: Sounding Reference Symbol Configuration for SCS=30kHz

|  | SRS.x2 TDD |  |
| --- | --- | --- |
| Field | Value | Comment |
| c-SRS | 24 |  |
| b-SRS | 0 |  |
| b-hop | 0 | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching |  |
| startPosition | 5 | resourceMapping setting |
| nrofSymbols | 2 |  |
| repetitionFactor | n1 | without repetition. |
| transmissionComb | n2 |  |
| combOffset-n2 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| resourceType | Periodic |  |
| periodicityAndOffset-p | sl80, 4 | SRS transmission periodicity is 40ms |

Note that from RAN2 spec 38.331:

resourceMapping

OFDM symbol location of the SRS resource within a slot including nrofSymbols (number of OFDM symbols), startPosition (value 0 refers to the last symbol, value 1 refers to the second last symbol, and so on) and repetitionFactor (see TS 38.214 [19], clause 6.2.1 and TS 38.211 [16], clause 6.4.1.4). The configured SRS resource does not exceed the slot boundary. If resourceMapping-r16 is signalled, UE shall ignore the resourceMapping (without suffix). For CLI SRS-RSRP measurement, the network always configures nrofSymbols and repetitionFactor to 'n1'.

startPosition counts from the last symbol, and therefore we set it to 5 and the SRS symbols occupies symbol 8 and 9.

For requirement on 1 symbol SRS in synchronized case, there is no existing test mechanism to verify interruption counted in symbols except CSI-RS measurement. However, since SRS antenna switching conflict with CSI-RS is either prioritize CSI-RS or up to UE implementation, CSI-RS can not verify the SRS antenna switching interruption. Therefore, testability issue should be resolved first before introducing test case for 1 symbol SRS antenna switching test.

Proposal 3: Testability issue should be resolved first before introducing test case for 1 symbol SRS antenna switching test.

# Conclusion

Proposal 1: Define two tests based on the requirement in Table 8.2.1.2.18-3, 8.2.2.2.16-3 (more than 1 SRS symbol) in 38.133 and 2 subframes on LTE (more than 1 SRS symbol configured) in NR-SA and EN-DC, but not NE-DC.

Proposal 2: Use 1T2R and the following configurations.

Sounding Reference Symbol Configuration for SCS=15kHz

|  | SRS.x1 TDD |  |
| --- | --- | --- |
| Field | Value | Comment |
| c-SRS | 12 |  |
| b-SRS | 0 |  |
| b-hop | 0 | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching |  |
| startPosition | 5 | resourceMapping setting |
| nrofSymbols | 2 |  |
| repetitionFactor | n1 | without repetition. |
| transmissionComb | n2 |  |
| combOffset | 0 | transmissionComb setting |
| cyclicShift | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| resourceType | Periodic |  |
| periodicityAndOffset-p | sl40, 2 | SRS transmission periodicity |

Table A.3.24-2: Sounding Reference Symbol Configuration for SCS=30kHz

|  | SRS.x2 TDD |  |
| --- | --- | --- |
| Field | Value | Comment |
| c-SRS | 24 |  |
| b-SRS | 0 |  |
| b-hop | 0 | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching |  |
| startPosition | 5 | resourceMapping setting |
| nrofSymbols | 2 |  |
| repetitionFactor | n1 | without repetition. |
| transmissionComb | n2 |  |
| combOffset-n2 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| resourceType | Periodic |  |
| periodicityAndOffset-p | sl80, 4 | SRS transmission periodicity is 40ms |

Proposal 3: Testability issue should be resolved first before introducing test case for 1 symbol SRS antenna switching test.

# Reference

[1] R4-2120247