3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207812

# Electronic Meeting, May 09 - May 20, 2022

Agenda item:	9.19.5.3.1

Source:	Apple Inc.

Title:	On CQI Reporting Requirements for Reduced Capability Devices in NR

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#102-e RedCap UE demodulation and CSI reporting requirements was discussed and way forward [1] was agreed. In this contribution we present initial simulation results for CQI reporting requirements for RedCap UE.

# 2. Discussion

In [1] the scope and simulation assumptions for CQI reporting requirements for RedCap UE were agreed.

Whether to define CQI reporting test with CQI table 2 for RedCap 2Rx UE supporting 256QAM in FR1 FDD - Define test with only CQI table 1. Test points for CQI reporting test with CQI table 1 - Set two SNR test points based on simulation results to cover SNR region with CQI report corresponding to: - 64QAM - QPSK or 16QAM

CQI Reporting in AWGN

For the agreed simulation assumptions [1], we provide initial simulation results for CQI reporting requirements in AWGN. Since FR2 requirements for RedCap UE will be reused from existing Rel-15 requirements, we only provide results for FR1.

Table 1: CQI reporting in AWGN-2x1

| AWGN | FDD |  | TDD |  |
| --- | --- | --- | --- | --- |
|  | SNR | Med CQI | SNR | Med CQI |
| QPSK | 0 | 5 | 0 | 5 |
|  | 1 | 6 | 1 | 6 |
|  | 2 | 6 | 2 | 6 |
|  | 3 | 6 | 3 | 6 |
| 16QAM | 4 | 7 | 4 | 7 |
|  | 5 | 8 | 5 | 8 |
|  | 6 | 8 | 6 | 8 |
|  | 7 | 9 | 7 | 9 |
|  | 8 | 9 | 8 | 9 |
| 64QAM | 9 | 10 | 9 | 10 |
|  | 10 | 10 | 10 | 10 |
|  | 11 | 11 | 11 | 11 |
|  | 12 | 11 | 12 | 11 |
|  | 13 | 12 | 13 | 12 |
|  | 14 | 12 | 14 | 12 |
|  | 15 | 13 | 15 | 13 |

Based on the results above, we propose to choose SNR points as [0,1] dB for CQI reporting in QPSK range and [12,13] dB for CQI reporting in 64QAM range for 2x1.

Proposal #1: Choose SNR points [0,1] dB for CQI reporting in QPSK range and [12,13] dB for CQI reporting in 64QAM range for 2x1 in AWGN.

Table 2: CQI reporting in AWGN-2x2

| AWGN | FDD |  | TDD |  |
| --- | --- | --- | --- | --- |
|  | SNR | Med CQI | SNR | Med CQI |
| QPSK | 0 | 3 | 0 | 3 |
|  | 1 | 4 | 1 | 4 |
|  | 2 | 4 | 2 | 4 |
|  | 3 | 5 | 3 | 5 |
|  | 4 | 5 | 4 | 5 |
|  | 5 | 6 | 5 | 6 |
|  | 6 | 6 | 6 | 6 |
| 16QAM | 7 | 7 | 7 | 7 |
|  | 8 | 7 | 8 | 7 |
|  | 9 | 8 | 9 | 8 |
|  | 10 | 8 | 10 | 9 |
|  | 11 | 9 | 11 | 9 |
| 64QAM | 12 | 10 | 12 | 10 |
|  | 13 | 10 | 13 | 10 |
|  | 14 | 11 | 14 | 11 |
|  | 15 | 11 | 15 | 11 |

Based on the results above, we propose to choose SNR points as [2,3] dB for CQI reporting in QPSK range and [14,15] dB for CQI reporting in 64QAM range for 2x2.

Proposal #2: Choose SNR points [2,3] dB for CQI reporting in QPSK range and [14,15] dB for CQI reporting in 64QAM range for 2x2 in AWGN.

WB CQI Reporting in Fading channel

Table 3: CQI reporting in fading channel with 64QAM

| 2x2 |  |
| --- | --- |
| SNR | Med CQI |
| -3 | 4 |
| -2 | 5 |
| -1 | 5 |
| 0 | 6 |
| 1 | 6 |
| 2 | 7 |
| 3 | 7 |
| 4 | 8 |
| 5 | 9 |
| 6 | 9 |
| 7 | 10 |
| 8 | 10 |
| 9 | 11 |
| 10 | 11 |
| 11 | 12 |
| 12 | 12 |
| 13 | 13 |
| 14 | 13 |
| 15 | 13 |

Based on the results, for 2RX, we propose to choose SNR [4,5] dB for SNR in 16QAM range and SNR [12,13] dB for SNR in 64QAM range for 2x2.

Proposal #3: Choose SNR [4,5] dB for SNR in 16QAM range and SNR [12,13] dB for SNR in 64QAM range for 2x2 in fading channel.

For 2x1 requirements, use SNR 3dB lower than that of 2x2 requirements.

Proposal #4: Choose SNR points 3dB lower than 2RX for requirements in fading channel with 1RX.

For the test requirements in fading channel, we propose to reuse the same test metrics and pass/fail criteria as existing WB CQI reporting tests in fading channel.

Proposal #5: Reuse the same test metrics and pass/fail criteria as existing WB CQI reporting tests in fading channel for RedCap UE.

# 3. Conclusion

In this paper, we provide initial simulation results for CQI reporting requirements for RedCap UE. Our observations and proposals are captured below:

Proposal #1: Choose SNR points [0,1] dB for CQI reporting in QPSK range and [12,13] dB for CQI reporting in 64QAM range for 2x1 in AWGN.

Proposal #2: Choose SNR points [2,3] dB for CQI reporting in QPSK range and [14,15] dB for CQI reporting in 64QAM range for 2x2 in AWGN.

Proposal #3: Choose SNR [4,5] dB for SNR in 16QAM range and SNR [12,13] dB for SNR in 64QAM range for 2x2 in fading channel.

Proposal #4: Choose SNR points 3dB lower than 2RX for requirements in fading channel with 1RX.

Proposal #5: Reuse the same test metrics and pass/fail criteria as existing WB CQI reporting tests in fading channel for RedCap UE.

# Reference

- R4-2207206, “WF on RedCap UE demodulation and CQI reporting requirements”, Ericsson.