3GPP TSG-RAN WG4 Meeting #103-e	R4- 2207818

Electronic Meeting, May. 09 – May. 20, 2022

Agenda Item:	9.8.2.1

Source:	Apple

Title:	General aspect in FR2 HST RRM

Document for:	Discussion

# 1	Introduction

In Rel-17 NR support for high-speed train scenario in FR2 [1], potential RRM enhancement is proposed:

  - Study and identify RRM requirements impact and possible enhancement for

    - Idle/inactive mode cell reselection requirements enhancement

    - Connected mode requirements

      - Handover delay requirement

      - Measurement requirements including both L1 and SSB based L3 measurement

      - Beam management requirements including beam failure detection, candidate beam detection performance requirements

      - Other requirements if identified

In this paper, we discuss the capability for one shot large UL timing adjustment in HST FR2.

# 2	Discussion

In [2], the description of “support of FR2 FST operation” is endorsed in the RRM. One remaining item is the UE feature on one shot UL timing adjustment.

Agreement: - Introduce feature group x-2 “Support of one shot large UL timing adjustment” with prerequisite feature group (x-1, “Support of FR2 HST operation”) Way forward: Companies are encouraged to discuss further the following two options: - Option 1: Define feature as mandatory with capability signaling. - Option 2: Define feature as optional with capability signaling.

UE autonomous one-shot UL timing adjustment is an advanced feature. It can be enabled and disabled by network signaling. The network assistance information has been discussed and defer to R18. If UE does not support the feature, or if network explicit signaling disable the one-shot timing adjustment, the legacy RACH procedure can be used when UE does not support the one-shot UL timing adjustment. Therefore, we prefer the support of one shot large UL timing adjustment to be an optional feature with capability signaling.

Proposal 1: Define UE autonomous one-shot large UL timing adjustment feature as optional with capability signaling.

# 3	Summary

In this paper, we provide our view on mobility aspect of RRM enhancement for HST FR2.

Proposal 1: Define UE autonomous one-shot large UL timing adjustment feature as optional with capability signaling.

# References

- RP-202118, New WID on NR support for high-speed train scenario in FR2, Samsung, Nokia, Sep 2020

- R4-2206848, Way forward for FR2 HST RRM (part 1), Nokia, Feb 2022

- R4-2202767, Way forward for uplink timing for FR2 HST, Samsung, Jan 2022