3GPP TSG-RAN WG4 Meeting #103-e	R4-2207826

# Electronic Meeting, May. 9 – 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | <CR#> | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-3: Missing definitions of PEMAX_NE-DC in Pcmax formulae (R17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SoftBank Corp. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This is a mirror CR of R15 relevant (but noted in R15 CR cover, some references have already been fixed in R16/R17). In configured output power section for Inter-band NE-DC, a term PEMAX, NE-DC is frequently mentioned but no definition is found. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reference to “p-UE-FR1” is added for PEMAX, NE-DC. Errors of referring to PEMAX, EN-DC and some formats are also corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The configured output power is not properly specified for NE-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.4.1.3a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR38.521-1 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Unaffected Portions Skipped]

6.2B.4.1.3a	Inter-band NE-DC within FR1

For inter-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum transmission  power for NE-DC operation, PTotalNE-DC= 10log10(PtotalNE-DC) with PtotalNE-DC as specified in clause 7.6.1A of TS 38.213 [10].

The configured maximum output power PCMAX_ E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier shall be set within the bounds:

PCMAX_L_ E-UTRA,c (p) ≤  PCMAX_ E-UTRA,c (p) ≤  PCMAX H _ E-UTRA,c (p)

where PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for a serving cell c as specified in TS 36.101 [4] clause 6.2.5 modified by PLTE as follows:

PCMAX_L_ E-UTRA,c = MIN { PEMAX, NE-DC , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), MIN(PEMAX,c , PLTE) – tC_ E-UTRA, c,  (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MPRc + A-MPRc + ΔTIB,c  + TC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PEMAX, NE-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PLTE, PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA}

with exception that

-	if no symbol of slot  of the NR that is indicated as uplink or flexible by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated overlaps with subframe  of the E-UTRA; or

-	if NR slot(s) that is indicated as downlink by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated does not overlap with subframe  of the E-UTRA; then

PCMAX_L_ E-UTRA,c = MIN { PEMAX, NE-DC , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PEMAX,c  – tC_ E-UTRA, c,  (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MPRc + A-MPRc + ΔTIB,c  + TC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PEMAX, NE-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA}

The configured maximum output power PCMAX,f,c,NR (q) in physical-channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L,f,c,NR and PCMAX_H,f,c,NR are the limits for a serving cell c as specified in clause 6.2.4 of TS 38.101-1 [2] modified by PNR as follows:

PCMAX_L,f,c,NR = MIN { PEMAX, NE-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), MIN(PEMAX,c , PNR ) - TC_NR, c,  (PPowerClass,NR – ΔPPowerClass,NR) – MAX(MPRc + A-MPRc+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, NE-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PNR , PPowerClass,NR – ΔPPowerClass,NR }

-	PEMAX,NE-DC signalled by RRC as p-UE-FR1 in TS 38.331 [9];

-	PLTE signalled by RRC as p-MaxEUTRA in TS 36.331 [8];

-	PNR signalled by RRC as p-NR-FR1 defined in TS 38.331 [9];

-	ΔTc_E-UTRA, c = 1.5dB when NOTE 2 in Table 6.2.2-1 in TS 36.101 [4] applies for a serving cell c, otherwise TC_ E-UTRA,c = 0dB;

-	TC_NR,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] applies for a serving cell c, otherwise TC_NR,c = 0dB;

-	ΔTIB,c specified in clause  6.2B.4.2.3 for NE-DC, the individual Power Class defined in table 6.2B.1.3a and any other additional power reductions parameters specified in clauses  6.2B.2.3a for NE-DC are applicable to PCMAX_ E-UTRA,c and PCMAX,f,c,NR evaluations.

-	PPowerClass, NE-DC is defined in clause 6.2B.1.3a for inter-band NE-DC;

-	PPowerClass,NR is the nominal UE power of the power class that the UE supports for the NR band of the NE-DC combination as defined in clause 6.2.1 of 38.101-1 [2]; in case IE [powerClassNRPart] as defined in TS 38.331 [9] is indicated, PPowerClass,NR should use that value instead.

-	PPowerClass,E-UTRA is the nominal UE power of the power class that the UE supports for the E-UTRA band of the NE-DC combination as defined in clause 6.2.2 of 36.101 [4];

-	ΔPPowerClass,NE-DC = 3 dB for a power class 2 capable NE-DC UE when requirements of default power class had been applied as specified in sub-clause 6.2B.1; otherwise ΔPPowerClass,NE-DC = 0 dB;

If the transmissions from NR and E-UTRA do not overlap, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply with the modifications specified above. The lower value between PPowerClass, NE-DC or PEMAX, NE-DC shall not be exceeded at any time by UE.

PTotalNE-DC = 10log10(PtotalNE-DC) with PTotalNE-DC the configured maximum transmission power for NE-DC operation as specified in clause 7.6 of TS 38.213 [10].

The total configured maximum transmission power for both synchronous and non-synchronous operation is

PTotalNE-DC= MIN { PEMAX, NE-DC ,PPowerClass, NE-DC – ΔPPowerClass, NE-DC }

If the UE does not support dynamic power sharing,

PTotalNE-DC= MIN { PEMAX, NE-DC ,PPowerClass, NE-DC  – ΔPPowerClass, NE-DC } + 0.3 dB

If the NE-DC UE does not support dynamic power sharing, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications TS 36.101 [4] and TS 38.101-1 [2] respectively apply with the modifications specified above and PTotalNE-DC applies.

When a UE supporting dynamic  sharing is configured for overlapping E-UTRA uplink and NR uplink transmissions, the UE can set its configured maximum output power PCMAX_ E-UTRA,c and PCMAX,f,c,NR for the configured E-UTRA and NR uplink carriers, respectively, and its configured maximum transmission power for NE-DC operation, PTotalNE-DC, as specified above.

The measured total maximum output power PUMAX over both CGs/RATs, measured over the transmission reference time duration is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,c,NR],

where pUMAX,c,E-UTRA and pUMAX,c,NR denotes the measured output power of serving cell c for E-UTRA and NR respectively, expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_L) and THIGH(PCMAX_H) for applicable values of PCMAX specified in Table 6.2B.4.1.3a-2.

When an UL subframe transmission p from E-UTRA overlap with a physical-channel q from the NR, then for PUMAX evaluation, the E-UTRA subframe p is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4.1.3a-1 when same or different subframe and physical-channel durations are used in aggregated carriers. PPowerClass ,NE-DC shall not be exceeded by the UE during any evaluation period of time.

Table 6.2B.4.1.3a-1: PCMAX evaluation window

| transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different RAT carriers | LTE Subframe | Min(Tno_hopping, Physical Channel Length) |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX { PCMAX_ NE-DC _H (p,q) , PCMAX_ NE-DC _H (p,q+1), … , PCMAX_ NE-DC _H (p,q+n) }

where PCMAX_ NE-DC _H are the applicable upper limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with LTE subframe p.

While PCMAX_L is computed as follows:

PCMAX_L = MIN { PCMAX_ NE-DC _L (p,q) , PCMAX_ NE-DC _L (p,q+1), … , PCMAX_ NE-DC _L (p,q+n)}

where PCMAX_NE-DC_L are the applicable lower limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with LTE subframe p,

With

PCMAX_ NE-DC _H(p,q) = MIN {10 log10 [pCMAX H _ E-UTRA,c (p) + pCMAX H,f,c,NR (q)], PEMAX, NE-DC ,PPowerClass, NE-DC}

And:

a = 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) ] > PTotalNE-DC

If a = TRUE

PCMAX_ NE-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) ], PEMAX, NE-DC ,PPowerClass, NE-DC}

Else

PCMAX_ NE-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,NR (q)], PEMAX, NE-DC ,PPowerClass, NE-DC}

where

-	pCMAX H _ E-UTRA,c (p) is the E-UTRA higher limit of the maximum configured power expressed in linear scale;

-	pCMAX H,f,c,NR (q) is the NR higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L _ E-UTRA,c (p) is the E-UTRA lower limit of the maximum configured power expressed in linear scale;

-	pCMAX L,f,c,NR (q) is the NR lower limit of the maximum configured power expressed in linear scale;

-	PPowerClass, NE-DC is defined in clause 6.2B.1.3a for inter-band NE-DC;

-	pCMAX_ E-UTRA,c (p) is the linear value of PCMAX_ E-UTRA,c (p), the real configured max power for E-UTRA

-	pCMAX,f,c,NR (q) is the linear value of PCMAX,f,c,NR (q), the real configured max power of NR

Table 6.2B.4.1.3a-2: PCMAX tolerance for Dual Connectivity E-UTRA-NR

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX< 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |
| NOTE 1: For UEs not indicating support of dynamic power sharing, the upper tolerance Thigh shall be reduced by 0.3 dB for P ≥ 20 dBm. |  |  |

When E-UTRA and NR transmissions overlap and the condition a = TRUE, PUMAX,f,c,NR (q) for MCG, under nominal conditions, shall meet

PUMAX,f,c,NR (q) ≤  10log(pCMAX H, f,c,,NR c (q)) + THIGH (10log(pCMAX H, f,c,,NR c (q))).

with the tolerances TLOW and THIGH for applicable values of PCMAX specified in Table 6.2B.4a.1.3-2.

When LTE and NR transmissions overlap and the condition a = FALSE), then PUMAX, under nominal conditions, shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

where PCMAX_L, PCMAX_H, and PUMAX are specified above with the tolerances TLOW and THIGH specified in Table 6.2B.4a.1.3-2 for applicable values of PCMAX_L and PCMAX_H.

[Unaffected Portions Skipped]

[Unaffected Portions Skipped]