3GPP TSG RAN WG4 Meeting #103-e	R4-2207852

Online, 9th – 20th May 2022

Agenda item:	8.15.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-11-21: EN-DC_8-11_n1-n77

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 2B NR of DC_8-11_n1-n77 was approved in RAN [1]. This TP is to capture the basic aspects for the EN-DC.

2	Text Proposal

[Unchanged Parts Skipped]

7.X	DC_8-11_n1-n77

7.X.1	Operating bands for DC

Table 7.X.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-11_n1-n77 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.X.2	Channel bandwidths per operating band for DC

Table 7.X.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Channel bandwidth [MHz] | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-11A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_11A_n1A DC_11A_n77A | 8 | 5, 10 | 140 |
|  |  | 11 | 5, 10 |  |
|  |  | n1 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| DC_8A-11A_n1A-n77(2A) | DC_8A_n1A DC_8A_n77A DC_11A_n1A DC_11A_n77A | 8 | 5, 10 | 240 |
|  |  | 11 | 5, 10 |  |
|  |  | n1 | 5, 10, 15, 20 |  |
|  |  | n77 | n77(2A) |  |

7.X.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-11_n1, DC_8-11_n77, DC_8_n1-n77 and DC_11_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.X.4	∆TIB and ∆RIB values

For DC_8-11_n1-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-11_n1-n77 | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 7.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-11_n1-n77 | 8 | 0.2 |
|  | 11 | 0 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

7.X.5	MSD

As mentioned in 7.X.3, there is no need to specify additional MSD requirement for this UL DC configuration.

[Unchanged Parts Skipped]

7.  Reference

[1]  RP-211750  Revised WID on EN-DC of x bands (x=1,2,3,4) LTE and NR 2 bands, LG Electronics