3GPP TSG-RAN WG4 Meeting #103-e	R4-2207907

# Electronic,  , May 09 - 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | DRAFT | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to TS 38.104 on HST FR2 Channel Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2-Perf |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At RAN4#101-e [R4-2120703] it was agreed to define only one HST FR2 model for PUSCH requirements based on bidirectional scenario-B model. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of a new model for high speed train conditions in FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | BS demodulation performance requirements for HST FR2 are not fully defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS 38.141-2, drafCR R4-2207908 |  |  |
| affected: |  |  | N | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## < Start of change>

# G.3	High speed train condition

High speed train conditions are as follows:

-	Scenario 1-NR350 / Scenario 1-NR500: Open space

-	Scenario 3-NR350 / Scenario 3-NR500: Tunnel

-	Scenario B-NR350: Open space

The high speed train conditions for the test of the baseband performance are three non-fading propagation channels. For BS with Rx diversity, the Doppler shift time variation is the same for each antenna at each time instant.

Doppler shift for all three scenarios is given by:

(G.3.1)

where  is the Doppler shift and  is the maximum Doppler frequency.

For Sceanrio 1 and Scenario 3, he cosine of angle  is given by:

, 	(G.3.2)

, 	(G.3.3)

, 	(G.3.4)

where  is the initial distance of the train from BS, and  is BS-Railway track distance, both in meters;  is the velocity of the train in m/s,  is time in seconds.

For Sceanrio B, the cosine of angle θt is given by:

cosθt=Ds-vtDmin2+Ds-vt2,    0<t≤(0.5*Ds)/v	(G.3.5)

cosθt=-vtDmin2+vt2,   (0.5*Ds)/v<t≤Ds/v	(G.3.6)

cosθt=cosθt modDsv,    t>Ds/v	(G.3.7)

where Ds is the initial distance of the train from BS, and Dmin is BS-Railway track distance, both in meters; v is the velocity of the train in m/s; t is time in seconds.

The required input parameters are listed in table G.3-1 and G.3-2. The resulting time varying Doppler shift is shown in Figure G.3-1, G.3-2, G.3-3, G.3-4 and G.3-9 for 350km/h scenarios, and in Figure G.3-5, G.3-6, G.3-7 and G.3-8 for 500km/h scenarios. For 350km/h scenarios, the Doppler shift was derived such that it corresponds to a velocity of around 350km/h for band n1 for the 15kHz SCS, for band n77 for the 30kHz SCS, and for band n257 for the 120kHz SCS. For 500km/h scenarios, the Doppler shift was derived such that it corresponds to a velocity of around 500km/h for band n3 for the 15kHz SCS and for band n77 for the 30kHz SCS. However, the same Doppler shift requirement shall be applied regardless of the frequency of operation of the basestation and thus for lower frequencies, the supported speed is higher.

Table G.3-1: Parameters for high speed train conditions for UE velocity 350 km/h

| Parameter | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Scenario 1-NR350, FR1 |  | Scenario 3-NR350, FR1 |  | Scenario B-NR350, FR2 |
|  | 700 m |  | 300 m |  | 700 m |
|  | 150 m |  | 2 m |  | 150 m |
|  | 350 km/h |  | 350 km/h |  | 350 km/h |
|  | 1340 Hz for 15kHz SCS 2334 Hz for 30kHz SCS |  | 1340 Hz for 15kHz SCS 2334 Hz for 30kHz SCS |  | 19444 Hz for 120kHz SCS |

Table G.3-2: Parameters for high speed train conditions for UE velocity 500 km/h

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
|  | Scenario 1-NR500, FR1 |  | Scenario 3-NR500, FR1 |  |
|  | 700 m |  | 300 m |  |
|  | 150 m |  | 2 m |  |
|  | 500 km/h |  | 500 km/h |  |
|  | 1740 Hz for 15kHz SCS 3334 Hz for 30kHz SCS |  | 1740 Hz for 15kHz SCS 3334 Hz for 30kHz SCS |  |

Figure G.3-1: Doppler shift trajectory for scenario 1-NR350 (15 kHz SCS)

Figure G.3-2: Doppler shift trajectory for scenario 3-NR350 (15 kHz SCS)

Figure G.3-3: Doppler shift trajectory for scenario 1-NR350 (30 kHz SCS)

Figure G.3-4: Doppler shift trajectory for scenario 3-NR350 (30 kHz SCS)

Figure G.3-5: Doppler shift trajectory for scenario 1-NR500 (15 kHz SCS)

Figure G.3-6: Doppler shift trajectory for scenario 3-NR500 (15 kHz SCS)

Figure G.3-7: Doppler shift trajectory for scenario 1-NR500 (30 kHz SCS)

Figure G.3-8: Doppler shift trajectory for scenario 3-NR500 (30 kHz SCS)

Figure G.3-9: Doppler shift trajectory for scenario B-NR350 (120 kHz SCS)

## < End of change>