3GPP TSG-RAN WG4 #103-e	R4-2207977

Electronic meeting, 9th – 20th May 2022

Source: 	Ericsson

Title:  	Repeater radiated conformance testing considerations

Agenda Item:	9.5.5.3.2

Document for:	Discussion

# Introduction

The repeater core specification was completed at RAN4#102 and now work should start on the conformance specification. This contribution presents some considerations on radiated conformance testing.

# Discussion

### Test set-up

Unlike BS and UE testing, repeater testing necessitates providing an input signal to the repeater and measuring an output signal. The test procedures need to be written taking this into account.

Observation 1: Test equipment procedures need to take into account that for a repeater, there is both an input and an output signal.

Similar to output power, the test set-up for OTA repeater testing will be more complex than for a basestation. For TRP requirements on unwanted emissions, the DUT needs to be rotated  or an alternative is needed such as multiple probes etc., whilst providing an input signal in the correct direction with respect to the DUT. Any variations in the input direction due to the TRP measurement procedure need to be taken into account when considering measurement uncertainties.

Observation 2: Measuring TRP whilst maintaining a directional input signal in the correct direction with respect to the DUT may be challenging and may impact measurement uncertainties.

It should also be investigated whether there are circumstances in which the signal from the test equipment may interfere with the measured quantity. This could happen if the test signal would propagate directly to the measurement receiver via e.g. sidelobes of the test antenna. This may depend on the positioning of the transmit and receive parts of the test setup and the antenna characteristics used for the test. On the other hand, with a directional test antenna and considering the impact of amplification in the repeater, it may be that direct propagation from the test equipment to the measurement antenna does not play a significant role.

Observation 3: MU may need to take into account that the repeater output signal is measured simultaneously with the TE signal being transmitted.

Proposal 1: Investigate further the test set-up and whether propagation of the test signal from the transmit test antenna to the receive test antenna could impact the measurement.

A Rel-17 repeater does not perform beamforming. IF the repeater has a wide antenna pattern or a directional antenna then the radiation pattern for unwanted emissions may be more related to the antenna pattern for the wanted signal than is the case for AAS basestations. This may be useful for simplifying the measurement procedure for unwanted emissions, but is implementation dependent.

## Unwanted emissions from the Test Equipment

When testing the repeater unwanted emissions, the test equipment itself may generate unwanted emissions that are amplified by the repeater. Since the objective of the test is to measure unwanted emissions levels, it is important that the output level of re-amplified TE noise is much lower than the emissions levels that are to be tested.

When measuring ACLR, test equipment emissions in the adjacent channel may be amplified by the repeater. The ratio between the gain in the passband and the gain in the adjacent channel will be equivalent to the ACLR. The total level of amplified emissions from the test equipment will be equivalent to the test equipment ACLR plus the repeater ACRR.

That is to say:

- Limit on repeater generated emissions = Repeater output power – Repeater ACLR

- Amplified Test Equipment emissions = TE output power + repeater gain -  TE ACLR – ACRR = Repeater output power – TE ACLR – ACRR

The difference between the repeater unwanted emissions limit and the amplified TE emissions will be:

(- Repeater ACLR) + (TE ACLR) + (Repeater ACRR)

Thus, the ratio of ACLR limit on the repeater to total amplified emissions from the test equipment will be significantly greater than the repeater ACLR limit and will not impact the measurement.

When measuring OBUE, it is important that any TE noise that is amplified through the repeater is well below the requirement level.

The OBUE requirements for FR2 are as follows:

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| 0.1*Minimum {400MHz, passband BW}  f_offset_CW < 150 MHz | 68 dB |
| 150 MHz  f_offset_CW < 400 MHz | 55 dB |
| 400 MHz  f_offset_CW < f_offset_max | 35 dB |

Category B OBUE is as follows:

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – 35 dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fB | 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz | Min(-13 dBm, Max(Prated,t,TRP – 43 dB, -20 dBm)) | 1 MHz |
| fB f < fmax | fB +5 MHz f_offset < f_ offsetmax | Min(-5 dBm, Max(Prated,t,TRP – 33 dB, -10 dBm)) | 10 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within gaps between passbands is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the gap between passbands. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz. |  |  |  |

It can be observed that for frequencies below 0.1* Minimum {400MHz, passband BW}, there is no requirement on the maximum gain, but there is a requirement on the maximum emissions level.

For frequencies below 10% of the passband bandwidth, the OBUE limit for a high-power repeater is -5dBm / MHz. As an example, assume a repeater with 35dBm output power (TRP) in 100MHz. The EIRP of the input signal at the repeater RIB would need to be

35dBm – Repeater gain

In PSD, this would be

15dBm / MHz – Repeater gain

The unwanted emissions limit would be -5dBm/MHz. In the worst case that the gain just outside of the passband is the same as in the passband then the test equipment emissions PSD at the repeater input would need to be well below:

-5dBm – Repeater gain

That is to say, the TE emissions PSD just outside of the passband would need to be much lower than 20dB below the output power PSD in order to avoid that amplified TE noise would impact the measurement of repeater emissions. It is not obvious that the test equipment PA would achieve this much suppression of emissions just outside of the wanted signal. Some filtering may be needed, which would impact the overall measurement set-up and MU.

At just above 0.1* Minimum {400MHz, passband BW}, the repeater gain cannot exceed 68dB. If 68dB is lower than the passband gain then the requirement on emissions suppression from the test equipment will be somewhat lower than for the first part of the OBUE. However, it is not obvious that 68dB will be substantially less than the in-band repeater gain.

It may be the case that the ACRR requirement, and not the OOB gain requirement dominates the dimensioning of the repeater’s out of passband gain, and hence the repeater gain is in practice significantly lower than the OOB gain limits. This could be considered further.

In summary, it is quite possible that emissions from the TE could be amplified through the repeater and impact measurement of emissions that are generated within the repeater for FR2.

Proposal 2: Investigate further what kind of filtering would be needed for the TE output for the unwanted emissions measurement and the impact on test set-up and measurement uncertainty.

# Conclusion

Proposal 1: Investigate further the test set-up and whether propagation of the test signal from the transmit test antenna to the receive test antenna could impact the measurement.

Proposal 2: Investigate further what kind of filtering would be needed for the TE output for the unwanted emissions measurement and the impact on test set-up and measurement uncertainty.