3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207989

# Electronic Meeting, May 9 - 20, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	Completion of inter-band band combinations with intra-band ULCA in UL configuration

Agenda Item:	8.3.1 UE RF requirements [WI code]

Document for:	Approval

# Introduction

In Release 17, after the introduction of NR ULCA in Release 16, several inter-band combinations have been requested with contiguous or non-contiguous UL CA as part of the UL configuration. This is either a single UL band, for which the IMDs of the two CC allocation may generate MSD in the second band, or two UL bands, with one or two bands using contiguous ULCA, which may be subject to Triple beat MSD. Since this was a new issue in 3GPP, these cases were handled in a specific AI of band combinations “not for block approval”. Since we are approaching the conclusion of Release 17 for the band combination baskets, we have evaluated the latest revisions of 38.101-1 and 38-101-3 for the completeness of such band combinations, and propose appropriate corrections where needed.

# Discussion

## Two band inter-band NR CA with one UL band intra-band ULCA

In Table 1 we provide all the two band inter-band NRCA combinations that have one band UL with intra-band ULCA that are in the latest revision of the Release 17 38.101-1 specification [2].

Table 1: Two band inter-band NR CA with one UL band intra-band ULCA MSD analysis

| Band Combination | UL configuration | IMD BW [MHz] | Dist. [MHz] | IMD order | 38.101-1 MSD | comment |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n5B | CA_n5B | 20 | 1081 | 111 | na |  |
| CA_n5B-n66A | CA_n5B | 20 | 2110 | 213 | na |  |
| CA_n5B-n77A | CA_n5B | 20 | 2601 | 263 | na |  |
| CA_n1A-n7B | CA_n7B | 50 | 330 | 15 | na |  |
| CA_n3A-n7B | CA_n7B | 50 | 620 | 27 | na |  |
| CA_n5A-n7B | CA_n7B | 50 | 1606 | 67 | na |  |
| CA_n7B-n28A | CA_n7B | 50 | 1697 | 69 | na |  |
| CA_n7B-n78A | CA_n7B | 50 | 730 | 31 | na |  |
| CA_n25A-n41C | CA_n41C | 190 | 501 | 7 | yes | freq DL n25 missing + brackets |
| CA_n28A-n41C | CA_n41C | 160 | 1693 | 23 | na |  |
| CA_n40A-n41C | CA_n41C | 160 | 96 | 3 | no | OK non-sim tx/Rx |
| CA_n41C-n66A | CA_n41C | 190 | 296 | 5 | yes | IMD5, brackets to remove |
| CA_n41C-n71A | CA_n41C | 190 | 1844 | 21 | na |  |
| CA_n41C-n77A | CA_n41C | 190 | 760 | 11 | yes | OK based on NS04 brackets to remove |
| CA_n41C-n79A | CA_n41C | 160 | 1710 | 23 | na |  |
| CA_n2A-n77(2A) | CA_n77(2A) | 530 | 1460 | 7 | yes | IMD7 accounting for US n77 spectrum |
| CA_n3A-n77(2A) | CA_n77(2A) | 600 | 1520 | 7 | yes | IMD7 japan spectrum bracket to remove |
| CA_n5A-n77(2A) | CA_n77(2A) | 530 | 2556 | 11 | no | OK with reduced US range |
| CA_n28A-n77(2A) | CA_n77(2A) | 600 | 2597 | 11 | no | OK with reduced Japan spectrum |
| CA_n30A-n77(2A) | CA_n77(2A) | 530 | 1090 | 7 | yes | IMD7 accounting for US n77 spectrum + brackets |
| CA_n66A-n77(2A) | CA_n77(2A) | 530 | 1250 | 7 | yes | IMD7 with US spectrum brackets |
| CA_n3A-n78(2A) | CA_n78(2A) | 400 | 1520 | 9 | no | Evaluate based on n77(2A) for Japan spectrum |
| CA_n28A-n78(2A) | CA_n78(2A) | 400 | 2497 | 15 | na |  |

Proposal on IMD cases for UL NRCA:

- CA_n25A-n41C/CA_n41C DL/UL configurations needs to have DL frequency added to MSD test point

- CA_n41C-n66A/CA_n41C, CA_n2A-n77(2A)/CA_n77(2A), CA_n3A-n77(2A)/CA_n77(2A), CA_n30A-n77(2A)/CA_n77(2A), CA_n66A-n77(2A)/CA_n77(2A) and CA_n41C-n77A/CA_n41C DL/UL configuration IMD MSDs have already been treated and brackets can be removed in 38.101-1 [4]

- CA_n5A-n77(2A)/CA_n77(2A) and CA_n28A-n77(2A)/CA_n77(2A) DL/UL configurations are subject to IMD11 when taking the respective regional spectrum, no MSD needs to be specified

- CA_n3A-n78(2A)/CA_n78(2A) DL/UL configuration has an IMD9 issue and requires analysis. This can be derived from the IMD7 case of CA_n3A-n77(2A)/CA_n77(2A) DL/UL configuration and added in 38.101-1 [4]

## Two band inter-band ENDC with one UL band intra-band ULCA

In Table 2 we provide all the two band inter-band NRCA combinations that have one band UL with intra-band ENDC that are in the latest revision of the Release 17 38.101-3 specification [3].

Table 2: Two band inter-band ENDC with one UL band intra-band ULCA MSD analysis

| Band Combination | UL configuration | AggBW [MHz] | Dist. [MHz] | IMD order | comment |
| --- | --- | --- | --- | --- | --- |
| DC_8A-(n)3AA | DC_(n)3AA | 50 |  |  | DC_(n)3AA not finalized |
| DC_2A-(n)5AA | DC_(n)5AA | 25 |  |  | SUO only no IMD |
| DC_12A-(n)5AA | DC_(n)5AA | 25 |  |  | SUO only no IMD |
| DC_30A-(n)5AA | DC_(n)5AA | 25 |  |  | SUO only no IMD |
| DC_48A-(n)5AA | DC_(n)5AA | 25 |  |  | SUO only no IMD |
| DC_66A-(n)5AA | DC_(n)5AA | 25 |  |  | SUO only no IMD |
| DC_2A-(n)12AA | DC_(n)12AA | 15 |  |  | SUO only no IMD |
| DC_5A-(n)12AA | DC_(n)12AA | 15 |  |  | SUO only no IMD |
| DC_48A-(n)12AA | DC_(n)12AA | 15 |  |  | SUO only no IMD |
| DC_66A-(n)12AA | DC_(n)12AA | 15 |  |  | SUO only no IMD |
| DC_3A-(n)41AA | DC_(n)41AA | 120 | 791 | 15 | OK IMD15 |
| DC_25A-(n)41AA | DC_(n)41AA | 120 | 501 | 11 | IMD11 with NS04 ?? |
| DC_2A-(n)71AA | DC_(n)71AA | 25 | 1232 | 101 | large IMD order |
| DC_66A-(n)71AA | DC_(n)71AA | 25 | 1412 | 115 | large IMD order |
| DC_25A-41A_n41A | DC_41A_n41A | 190 | 501 | 7 | reuse MSD from CA_n25A-n41C with CA_n41C UL |
| DC_41A_n3A-n41A | DC_41A_n41A | 160 | 791 | 11 | OK with China spectrum and NS01 |

Proposal on IMD cases for UL ENDC:

- DC_8A-(n)3AA/ DC_(n)3AA DL/UL configuration needs DC_(n)3AA band combination to be finalised. Provided this is the case, there is no IMD MSD required for DC_8A-(n)3AA thanks to SUO only

- DC_25A-(n)41AA/DC_(n)41AA DL/UL configuration has an IMD11 issue and given this is an NS04 case for the UL this requires further analysis

- DC_25A-(n)41CA	 with DC_41A_n41A UL configuration has IMD7 issue that is not specified, MSD from CA_n25A-n41C with CA_n41C can be reused and added in CR [5]

## Two band inter-band ENDC with two UL band and intra-band ULCA

In this section we provide all the two band inter-band NRCA combinations that have two band UL with at least one intra-band ULCA that are in the latest revision of the Release 17 38.101-3 specification [3]:

- Table 3 lists the cases with LTE intra-band ULCA (max 40MHz aggregated BW)

- Table 4 lists the cases with NR intra-band ULCA (max 200MHz aggregated BW)

- Table 5 shows the example case chosen for triple beat analysis in [1]

Table 3: Two band inter-band ENDC with two UL band and LTE intra-band ULCA MSD analysis

| Band Combination | UL configuration | 1UL | Duplex [MHz] | max DLBW [MHz] | max ULBW [MHz] | 2UL | max aggBW [MHz] | Dist [MHz] | IMD order | TB order | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1C_n3A | DC_1C_n3A | n3A | 95 | 50 | 50 | 1C | 40 | 40 | 5 | 1.113 | in Rel 16! cover based on DC_3_n3? |
| DC_2C_n41A | DC_2C_n41A | n41A | NA | 100 | 100 | 2C | 40 | 586 | 31 | NA | no TB 1UL is TDD |
| DC_2C_n71A | DC_2C_n71A | n71A | -46 | 35 | 20 | 2C | 40 | 1198 | 63 | -1.85 | 2nd order TB OK |
| DC_3C_n1A | DC_3C_n1A | n1A | 190 | 50 | 50 | 3C | 40 | 325 | 19 | 3.488 |  |
| DC_3C_n5A | DC_3C_n5A | n5A | 45 | 25 | 20 | 3C | 40 | 816 | 43 | 0.55 | Needs further analysis MB to LB |
| DC_3C_n7A | DC_3C_n7A | n7A | 120 | 50 | 50 | 3C | 40 | 835 | 45 | 1.738 | 2nd order TB OK |
| DC_3C_n28A | DC_3C_n28A | n28A | 55 | 30 | 30 | 3C | 40 | 907 | 47 | 0.613 | Needs further analysis MB to LB |
| DC_3C_n41A | DC_3C_n41A | n41A | NA | 100 | 100 | 3C | 40 | 791 | 43 | NA | no TB 1UL is TDD |
| DC_3C_n77A | DC_3C_n77A | n77A | NA | 100 | 100 | 3C | 40 | 1035 | 55 | NA | no TB 1UL is TDD |
| DC_3C_n78A | DC_3C_n78A | n78A | NA | 100 | 100 | 3C | 40 | 1035 | 55 | NA | no TB 1UL is TDD |
| DC_3C_n79A | DC_3C_n79A | n79A | NA | 100 | 100 | 3C | 40 | 2615 | 135 | NA | no TB 1UL is TDD |
| DC_7C_n1A | DC_7C_n1A | n1A | 190 | 50 | 50 | 7C | 40 | 330 | 19 | 3.488 |  |
| DC_7C_n3A | DC_7C_n3A | n3A | 95 | 50 | 50 | 7C | 40 | 620 | 33 | 1.113 | 2nd order TB OK |
| DC_7C_n5A | DC_7C_n5A | n5A | 45 | 25 | 20 | 7C | 40 | 1606 | 83 | 0.55 | TB OK due to band distance? HB to LB |
| DC_7C_n28A | DC_7C_n28A | n28A | 55 | 30 | 30 | 7C | 40 | 1697 | 87 | 0.613 | TB OK due to band distance? HB to LB |
| DC_7C_n78A | DC_7C_n78A | n78A | NA | 100 | 100 | 7C | 40 | 250 | 15 | NA | no TB 1UL is TDD |
| DC_39C_n41A | DC_39C_n41A | n41A | NA | 100 | 100 | 39C | 35 | 576 | 35 | NA | no TB 1UL is TDD |
| DC_40C_n78A | DC_40C_n78A | n78A | NA | 100 | 100 | 40C | 40 | 420 | 23 | NA | no TB 1UL is TDD |
| DC_41C_n1A | DC_41C_n1A | n1A | 190 | 50 | 50 | 41C | 40 | 326 | 19 | 3.488 |  |
| DC_41C_n3A | DC_41C_n3A | n3A | 95 | 50 | 50 | 41C | 40 | 616 | 33 | 1.113 | 2nd order TB OK |
| DC_41C_n28A | DC_41C_n28A | n28A | 55 | 30 | 30 | 41C | 40 | 1693 | 87 | 0.613 | TB OK due to band distance? HB to LB |
| DC_41C_n77A | DC_41C_n77A | n77A | NA | 100 | 100 | 41C | 40 | 610 | 33 | NA | no TB TDD+TDD |
| DC_41C_n78A | DC_41C_n78A | n78A | NA | 100 | 100 | 41C | 40 | 610 | 33 | NA | no TB TDD+TDD |
| DC_41C_n79A | DC_41C_n79A | n79A | NA | 100 | 100 | 41C | 40 | 1710 | 89 | NA | no TB TDD+TDD |
| DC_42C_n1A | DC_42C_n1A | n1A | 190 | 50 | 50 | 42C | 40 | 1230 | 65 | 3.488 |  |
| DC_42C_n3A | DC_42C_n3A | n3A | 95 | 50 | 50 | 42C | 40 | 1520 | 79 | 1.113 | 2nd order TB OK |
| DC_42C_n28A | DC_42C_n28A | n28A | 55 | 30 | 30 | 42C | 40 | 2597 | 133 | 0.625 | TB OK due to band distance? HB to LB |
| DC_48B_n46A | DC_48B_n46A | n46A | NA | 100 | 100 | 48B | 20 | 1450 | 151 | NA | no TB TDD+TDD |
| DC_3C_n75A | DC_3C_n75A |  |  |  |  |  |  |  |  |  | remove from spec n75 is SDL!!! |

Table 4: Two band inter-band ENDC with two UL band and NR intra-band ULCA MSD analysis

| Band Combination | UL configuration | 1UL | Duplex [MHz] | max DLBW [MHz] | max ULBW [MHz] | 2UL | max aggBW [MHz] | Dist [MHz] | IMD order | TB order | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n7B | DC_3A_n7B | 3A | 95 | 20 | 20 | n7B | 50 | 620 | 27 | 1.5 | 2nd order TB OK |
| DC_8A_n79C | DC_8A_n79C | 8A | 45 | 10 | 10 | n79C | 200 | 3240 | 35 | 0.175 | OK due to band distance? VHB to LB |
| DC_28A_n7B | DC_28A_n7B | 28A | 55 | 20 | 20 | n7B | 50 | 1697 | 71 | 0.7 | OK due to band distance? HB to LB |

Table 5: Two band inter-band ENDC with two UL band and NR intra-band ULCA used as example in [1]

| Band Combination | UL configuration | 1UL | Duplex [MHz] | max DLBW [MHz] | max ULBW [MHz] | 2UL | max aggBW [MHz] | Dist [MHz] | IMD order | TB order | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n41C | DC_3A_n41C | 3A | 95 | 20 | 20 | 41C | 160 | 616 | 9 | 0.469 | no TB MSD in 38.101-3, TP? |

Proposal on triple beat cases:

- DC_3C_n75A UL configuration is removed in 38.101-3 in [5] as it is not a valid configuration as n75 is an SDL band

- DC_1C_n3A IMD5 MSD is introduced in 38.101-3 in both Release 16 and 17 [5]

- DC_3C_n5A and DC_3C_n28A need further analysis for triple beat MSD and are postponed to Release 18. UL configurations are removed from 38.101-3 [5]

- DC_3A_n41C IMD and triple beat MSD are introduced in 38.101-3 in [5] provided the rest of the TP is complete and triple beat analysis crosschecked by multiple companies following [1]

- DC_7C_n5A, DC_7C_n28A, DC_41C_n28A, DC_42C_n28A DC_8A_n79C and DC_28A_n7B do not require TB MSD thanks to band separation.

## Inter-band ENDC with two UL band and intra-band ULCA missing fallbacks

In Table 6 we provide the cases where higher order ENDC combinations have triple beat cases that do not have their two band fallback specified in the latest revision of the Release 17 38.101-3 specification [3].

Table 6: Two band inter-band ENDC with two UL band and NR intra-band ULCA used as example in [1]

| BC | UL | 1UL | Duplex [MHz] | max DLBW [MHz] | max ULBW [MHz] | 2UL | max aggBW [MHz] | Dist [MHz] | IMD order | TB order | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-5A-30A_n2A | DC_2A_n2A | Missing DC_2A_n2A UL in DC_2A-5A_n2A fallback +> no MSD SUO only => add fallback |  |  |  |  |  |  |  |  |  |
| DC_2A-5A-66A_n2A | DC_2A_n2A |  |  |  |  |  |  |  |  |  |  |
| DC_1A-28A_n7B | DC_1A_n7B | 1A | 190 | 20 | 20 | n7B | 50 | 330 | 15 | 3.4 | not in 2B: no MSD |
| DC_3C-28A_n7B-n78A DC_1A-3C-28A_n7B-n78A | 3C_n7B | 3A | 95 | 40 | 40 | n7B | 50 | 835 | 35 | 1.1 | not in 2B: no MSD |
|  | 3C_n7B | n7A | 120 | 50 | 50 | 3C | 40 | 620 | 33 | 1.75 | not in 2B: no MSD |

Proposal on missing ENDC fallbacks:

- Add DC_2A_n2A (SUO only) UL configuration to in DC_2A-5A_n2A missing fallback for DC_2A-5A-30A_n2A and DC_2A-5A-66A_n2A in CR [5]

- DC_1A-28A_n7B/DC_1A_n7B DL/UL configuration: add DC_1A_n7B UL configuration to DC_1A_n7B BC in CR as no MSD [5]

- DC_3C-28A_n7B-n78A and DC_1A-3C-28A_n7B-n78A/3C_n7B DL/UL configuration: since this is missing for a number of fallbacks, remove UL configuration 3C_n7B from DC_1A-3C-28A_n7B-n78A in CR [5]

## Analysis for the identified cases

### CA_n3A-n78(2A) with CA_n78(2A) UL IMD9 issue

For CA_n3A-n78(2A) with CA_n78(2A) UL there are multiple regional cases captured in Table 7 and the worst case is for IMD9.

Table 7: IMD order for CA_n3A-n78(2A) with CA_n78(2A) UL in different regions

| Band Combination | UL configuration | AggBW [MHz] | Dist. [MHz] | IMD order | Region |
| --- | --- | --- | --- | --- | --- |
| CA_n3A-n78(2A) | CA_n78(2A) | 400 | 1520 | 9 | Japan/Korea/Europe |
| CA_n3A-n78(2A) | CA_n78(2A) | 300 | 1420 | 11 | China |

Table 8 is a copy of the MSD requirement in 38.101-1 for CA_n3A-n77(2A) with CA_n77(2A) UL configuration related to IMD7 calculated for n77 spectrum in Japan with frequency start of 3.4GHz and 600MHz total UL bandwidth which can be used as a basis to calculate CA_n78(2A) IMD9 MSD.

Table 8: IMD7 MSD for CA_n77(2A) in band n3

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n77 | n3 | N/A | 5 | N/A | 1877.5 | [2.2] | FDD | IMD7 |
|  | n77 | 3455 | 10 | 1 (RBstart=10) | 3455 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBstart=0) | 3945 |  |  |  |

If we make the assumption that the same attenuation is available from the filter than in the n3-n77 case, and 15dB lower IMD9 versus IMD7 then the calculated MSD is 0.09dB and thus can be ignored.

Proposal for CA_n3A-n78(2A) with CA_n78(2A) UL: no MSD is required

### CA_n25A-n41C with CA_n41C UL DL frequency

When checking the latest revision of the specification, it was discovered that the band 25 DL frequency is missing but also that the exact UL allocation still had options. After further verification, the exact band n41UL allocation with IMD7 hitting the center of the highest DL 5MHz channel in n25 was identified.

Proposal for CA_n25A-n41C with CA_n41C UL: the following test point and MSD in Table 9 is introduced in 38.101-1 [4]:

Table 9: IMD7 MSD for CA_n41C in band n25

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n41 | n25 | N/A | 5 | N/A | 1992.5 | 8.5 | FDD | IMD7 |
|  | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=221) | 2640 |  |  |  |

### DC_25A_(n)41AA with DC_(n)41AA UL IMD11 issue

The MSD for DC_25A_(n)41AA related to IMD11 of DC_(n)41AA (UL aggregated bandwidth is 120MHz) can be derived from CA_n25-n41 MSD in Table 9 which is for IMD7 (CA_n41C is 190MHz aggregated BW) by assuming 24dB lower IMD11 versus IMD7. When doing the calculation, this results in 0.1dB MSD, and can thus be ignored.

Proposal for DC_25A_(n)41AA with DC_(n)41AA UL IMD11: no MSD is required

### DC_1C_n3A with DC_1C_n3A UL IMD5 issue

This particular case can be derived from the 1PA DC_3_n3 IMD5 case in band n3 copied in Table 10 (31dB MSD case). Since the DC_1C_n3A uses a quad-plexer instead of the band 3 duplexer, the only difference is an assumption of 45dB cross-band isolation instead of a 50dB duplexer isolation.

Table 10: DC_3_n3 IMD5 in n3 and IMD7 in band 3 MSD for 1PA

| MSD / DC bandwidth class A + A |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_3_n3 | 3 | 1782.5 | 5 | 12 (RBstart = 9) | 1877.5 | 29 | FDD |
|  | n3 | 1752.5 | 5 | 12 (RBstart = 0) | 1847.5 | 31 |  |

Based on this evaluation, the IMD5 MSD of band 1C in n3 is 5dB higher at 36dB. Note that though the 1RB+1RB allocation of band 1C concentrates all the IMD5 spectrum into band n3 DL channel, while for DC_3_n3 with 12RB on each CC the IMD5 spectrum is wider than the band n3 DL channel, still the majority of the peak of the IMD5 is integrated.

Proposal for DC_1C_n3A with DC_1C_n3A UL: the following test point and MSD in Table 11 is introduced in 38.101-3 in both Release 16 and 17 [5]:

Table 11: IMD5 MSD for CA_1C in band n3

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1_n3 | n3 | N/A | 5 | N/A | 1877.5 | 36 | FDD | IMD5 |
|  | 1 | 1950 | 20 | 1 (RBstart=0) | 2140 | N/A | TDD | N/A |
|  |  | 1970 | 20 | 1 (RBstart=67) | 2160 |  |  |  |

## NRCA combinations with missing intra-band ULCA fallbacks

In Table 12 we provide the cases where higher order NRCA  combinations have triple beat cases that do not have their two band fallback specified in the latest revision of the Release 17 38.101-3 specification [3].

Table 12: NRCA with missing UL configuration fallbacks

| BC | UL | comment |
| --- | --- | --- |
| CA_n46B-n48(2A) | CA_n46B-n48A | CA_n46B UL does not exist => change to CA_n46A-n48A |
| CA_n28A-n41A-n78(2A) | CA_n78(2A) | CA_n78(2A) UL does not exist for CA_n41A-n78(2A) => change UL configuration to “-“ |
| CA_n2A-n5A-n48B-n66A-n77A | CA_n48B | CA_n48B UL not specified for 2band fallbacks => remove from UL configurations |
| CA_n2A-n48A-n77C | CA_n77C | CA_n77C UL not specified for 2band fallbacks => remove from UL configurations |
| CA_n5A-n48A-n77C |  |  |
| CA_n48A-n66A-n77C |  |  |
| CA_n2A-n5A-n48A-n66A-n77C |  |  |

Proposal on missing NRCA fallbacks:

- Change UL configuration CA_n46B-n48A to CA_n46A-n48A in CA_n46B-n48(2A) in CR [4] as CA_n46B UL does not exist

- Change UL configuration to “-“ for CA_n28A-n41A-n78(2A) as CA_n78(2A) UL does not exist for CA_n41A-n78(2A) in CR [4]

- Remove UL configuration CA_n48B from CA_n2A-n5A-n48B-n66A-n77A in CR [4] as UL configuration CA_n48B does not exist in 2 band lower fallbacks

- Remove UL configuration CA_n77C from CA_n2A-n48A-n77C, CA_n5A-n48A-n77C, CA_n48A-n66A-n77C and CA_n2A-n5A-n48A-n66A-n77C in CR [4] as UL configuration CA_n77C does not exist in 2 band lower fallbacks

# Conclusions

In this contribution, we crosschecked all band combinations in the latest revisions of 38.101-1 and 38.101-3 which have UL configurations including intra-band UL CA for completeness. This enabled us to identify missing fallbacks, re-check cases that did not need MSD, and correct several cases. We submitted a number of proposals in this contribution, but in this conclusion we only retain cases that result in corrections in draft CRs [4, 5] and CR for R16 for DC_1C_n3A [6].

Proposal on IMD cases for UL NRCA:

- CA_n41C-n66A/CA_n41C, CA_n2A-n77(2A)/CA_n77(2A), CA_n3A-n77(2A)/CA_n77(2A), CA_n30A-n77(2A)/CA_n77(2A), CA_n66A-n77(2A)/CA_n77(2A) and CA_n41C-n77A/CA_n41C DL/UL configuration IMD MSDs have already been treated and brackets can be removed in 38.101-1 [4]

Proposal on IMD cases for UL ENDC:

- DC_8A-(n)3AA/ DC_(n)3AA DL/UL configuration needs DC_(n)3AA band combination to be finalised. Provided this is the case, there is no IMD MSD required for DC_8A-(n)3AA thanks to SUO only

- DC_25A-(n)41CA	 with DC_41A_n41A UL configuration has IMD7 issue that is not specified, MSD from CA_n25A-n41C with CA_n41C can be reused and added in CR [5]

Proposal on triple beat cases:

- DC_3C_n75A UL configuration is removed in 38.101-3 in [5] as it is not a valid configuration as n75 is an SDL band

- DC_3C_n5A and DC_3C_n28A need further analysis for triple beat MSD and are postponed to Release 18. UL configurations are removed from 38.101-3 [5]

- DC_3A_n41C IMD and triple beat MSD are introduced in 38.101-3 in [5] provided the rest of the TP is complete and triple beat analysis crosschecked by multiple companies following [1]

Proposal on missing ENDC fallbacks:

- Add DC_2A_n2A (SUO only) UL configuration to in DC_2A-5A_n2A missing fallback for DC_2A-5A-30A_n2A and DC_2A-5A-66A_n2A in CR [5]

- DC_1A-28A_n7B/DC_1A_n7B DL/UL configuration: add DC_1A_n7B UL configuration to DC_1A_n7B BC in CR as no MSD [5]

- DC_3C-28A_n7B-n78A and DC_1A-3C-28A_n7B-n78A/3C_n7B DL/UL configuration: since this is missing for a number of fallbacks, remove UL configuration 3C_n7B from DC_1A-3C-28A_n7B-n78A in CR [5]

Proposal for CA_n3A-n78(2A) with CA_n78(2A) UL: no MSD is required

Proposal for CA_n25A-n41C with CA_n41C UL: the following test point and MSD in Table 9 is introduced in 38.101-1 [4]:

Table 9: IMD7 MSD for CA_n41C in band n25

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n41 | n25 | N/A | 5 | N/A | 1992.5 | 8.5 | FDD | IMD7 |
|  | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=221) | 2640 |  |  |  |

Proposal for DC_1C_n3A with DC_1C_n3A UL: the following test point and MSD in Table 11 is introduced in 38.101-3 in both Release 16 [6] and 17 [5]:

Table 11: IMD5 MSD for CA_1C in band n3

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1_n3 | n3 | N/A | 5 | N/A | 1877.5 | 36 | FDD | IMD5 |
|  | 1 | 1950 | 20 | 1 (RBstart=0) | 2140 | N/A | TDD | N/A |
|  |  | 1970 | 20 | 1 (RBstart=67) | 2160 |  |  |  |

Proposal on missing NRCA fallbacks:

- Change UL configuration CA_n46B-n48A to CA_n46A-n48A in CA_n46B-n48(2A) in CR [4] as CA_n46B UL does not exist

- Change UL configuration to “-“ for CA_n28A-n41A-n78(2A) as CA_n78(2A) UL does not exist for CA_n41A-n78(2A) in CR [4]

- Remove UL configuration CA_n48B from CA_n2A-n5A-n48B-n66A-n77A in CR [4] as UL configuration CA_n48B does not exist in 2 band lower fallbacks

- Remove UL configuration CA_n77C from CA_n2A-n48A-n77C, CA_n5A-n48A-n77C, CA_n48A-n66A-n77C and CA_n2A-n5A-n48A-n66A-n77C in CR [4] as UL configuration CA_n77C does not exist in 2 band lower fallbacks

# References

- R4-2206394	WF on capturing triple beat MSD in 38.101-1 and 38.101-3,	Skyworks, Qualcomm, RAN4#102-e

- 3GPP TS 38.101-1 V17.5.0 (2022-03)

- 3GPP TS 38.101-3 V17.5.0 (2022-03)

- R4-2207993 Draft CR to TS 38.101-1 V17.5.0 on intra-band ULCA UL configurations, Skyworks Solutions Inc., RAN4#103-e

- R4-2210108 Draft CR to TS 38.101-3 V17.5.0 on intra-band ULCA UL configurations, Skyworks Solutions Inc., RAN4#103-e

- R4-2210109 Draft CR to R16 TS 38.101-3 on adding MSD for DC_1C_n3A UL configuration, Skyworks Solutions Inc., RAN4#103-e