3GPP TSG-RAN WG4 Meeting #103-e 		R4-2208012

Electronic meeting, 9th – 20th May, 2022

Source:	Ericsson

Title:	Discussion on PUCCH demodulation requirements for NR coverage enhancement

Agenda item:	9.17.2.2

Document for:	Discussion

# Introduction

In the last RAN4 meeting, companies have discussions on the NR coverage enhancement demodulation requirements. Following WF was agreed on the PUCCH demodulation requirement part.

## General

### Issue 2-1-1: Test metric for BS PUCCH demodulation test cases

Agreement: Agree to reuse the existing test metric for different PUCCH formats as a baseline for BS PUCCH demodulation requirements with JCE (if introduced) i.e: - For PUCCH format 3: Test UCI block error probability - For PUCCH format 1: Test NACK to ACK detection probability and ACK missed detection probability

## PUCCH demodulation with Joint Channel Estimation (JCE)

### Issue 2-2-1: Whether to define BS PUCCH demodulation requirements with JCE

Agreement: Baseline assumption: Introduce BS PUCCH demodulation requirements with JCE with 2Rx only.

### Issue 2-2-2: PUCCH format for BS PUCCH demodulation requirements with JCE (if introduced)

Agreement: Cover both format 1 and format 3 for BS PUCCH demodulation requirements with JCE (if introduced)

### Issue 2-2-3: Actual TDW length for JCE in BS PUCCH demod requirements (if introduced)

Agreement: In case of BS PUCCH demodulation requirements with JCE is introduced, use the same aTDW agreed for PUSCH JCE.

### Issue 2-2-6: Frequency hopping for BS PUCCH demodulation requirements with JCE (if introduced)

Agreement: In case of BS PUCCH demodulation requirements with JCE is introduced, agree to disable PUCCH intra-slot frequency hopping.

- Inter-slot frequency hopping:

  - Option 1: Disabled

  - Option 2: Activate inter-slot frequency hopping, with inter-slot hopping interval length 2 for TDD and 4 for FDD

### Issue 2-2-7: TDD UL-DL pattern for BS PUCCH demodulation requirements with JCE (if introduced)

Agreement: Agree to use the same TDD UL-DL pattern for BS PUCCH demodulation requirements as in PUSCH JCE demod requirementss.

### Issue 2-2-9: Phase and power offset modelling for BS PUCCH demod requirements with JCE (if introduced)

Agreement: Agree to align both phase offset model and power offset model with the JCE test modelling for PUSCH

### Issue 2-2-10: Bit payload for BS PUCCH demodulation requirements with JCE (if introduced)

Agreement: Agree to use 2 bits payload for PUCCH format 1, and 16 bits payload for PUCCH format 3.

### Issue 2-2-11: Resource allocation for BS PUCCH demodulation requirements with JCE (if introduced)

Agreement: Agree to use 14 symbols and 1 PRB for both PUCCH format 1 and format 3.

### Issue 2-2-12: Other parameters for BS PUCCH demodulation requirements with JCE (if introduced)

- Other test parameters:

  - Option 1: Apply existing test parameters specified in Rel-15 for PUCCH requirement with JCE as starting point

  - Option 2:

Table 2-1: Test Parameters for PUCCH JCE format 1

| Parameter | Test |
| --- | --- |
| First PRB prior to frequency hopping | 0 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |

Table 2-2: Test Parameters for FR1 PUCCH JCE format 3

| Parameter | Test |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| First symbol | 0 |

In this contribution, open issues for PUSCH demodulation requirements will be further analyzed.

# Discussion

Configured TDW length

### Issue 2-2-4: Configured TDW length for JCE in BS PUCCH demod requirements (if introduced)

- For TDD

  - Option 1: Use the max number cTDW length to be [16] slots

  - Option 2: cTDW length is configured same as the aTDW length

- For FDD

  - Option 1: 8 slots

  - Option 2: cTDW length is configured same as the aTDW length

Same as discussed for PUSCH JCE in [2], for PUCCH JCE, the configured TDW length can be configured same as the active TDW length, where for TDD, the active TDW length is equal to the number of consecutive UL slots in the UL-DL pattern, while for FDD, the active TDW length is equal to the number of repetitions.

Proposal 1: For PUCCH JCE demodulation requirement, the configured TDW length can be configured same as the active TDW length, where

- for TDD, the active TDW length is equal to the number of consecutive UL slots, i.e., 2, in the UL-DL pattern

- for FDD, the active TDW length is equal to the number of repetitions.

Number of repetitions

### Issue 2-2-5: Number of repetitions for BS PUCCH demodulation requirements with JCE (if introduced)

- Option 1: 8 for FDD and TDD

- Option 2: 2

- Option 3: Same with aTDW length

Same as for PUSCH JCE in [2], a large number of repetitions is preferred to be considered for PUCCH JCE demodulation requirement definition to verify the improvement of JCE over non-JCE, e.g., 8 repetitions.

Proposal 2: For PUCCH JCE demodulation requirement, 8 repetitions is considered for TDD and FDD.

Frequency range

### Issue 2-2-8: Frequency range coverage for BS PUCCH demodulation requirements with JCE (if introduced)

- Option 1: Only cover FR1

- Option 2: Cover both FR1 and FR2

As discussed in [3], the demodulation requirement for PUCCH JCE is not suggested to be defined for FR2, because

- PUCCH coverage limitation only occurs in O2I scenario which is not a typical scenario for FR2

- Lack of the demodulation requirement for Rel-15 multi-slot PUCCH in FR2 which implicitly indicate that the coverage enhancement would be unnecessary for FR2 PUCCH.

Proposal 3: Define PUCCH JCE demodulation requirement for FR1 only.

Antenna configuration

Antenna configuration:

  - Option 1: 1T2R only

  - Option 2: cover 1T with 2Rx 4Rx and 8Rx

Similar as discussion in [2], only 2 Rx antennas are considered for PUCCH JCE could be enough and furtherly reduce the simulation and test effort.

Proposal 4: Only 2 Rx antennas are considered for PUCCH JCE.

DMRS configuration for PUCCH format 3:

  - Option 1: only with or without additional DMRS

  - Option 2: Cover both with and without additional DMRS

Additional DMRS for PUCCH format 3

For both the demodulation requirement of Rel-15 PUCCH format 3 for FR1 and FR2, both with and without additional DMRS are considered, because the support of PUCCH format 3 with and/or without additional DMRS is up to manufacturer declaration. To avoid no requirements for a certain DM-RS support, it seems natural for PUCCH JCE to follow the rule, i.e., both with and without additional DMRS are considered.

Proposal 5: For PUCCH format 3, both with and without additional DMRS are considered.

# Conclusions

Proposal 1: For PUCCH JCE demodulation requirement, the configured TDW length can be configured same as the active TDW length, where

- for TDD, the active TDW length is equal to the number of consecutive UL slots, i.e. 2, in the UL-DL pattern

- for FDD, the active TDW length is equal to the number of repetitions.

Proposal 2: For PUCCH JCE demodulation requirement, 8 repetitions is considered for TDD and FDD.

Proposal 3: Define PUCCH JCE demodulation requirement for FR1 only.

Proposal 4: Only 2 Rx antennas are considered for PUCCH JCE.

Proposal 5: For PUCCH format 3, both with and without additional DMRS are considered.

# References

[1]	R4-2207211, WF for PUCCH coverage enhancement, Nokia

[2] 	R4-2208010, Discussion on PUSCH demodulation requirements for NR coverage enhancement, Ericsson

[3] 	R4-2204034, Discussion on NR coverage enhancement PUCCH demodulation, Ericsson