3GPP TSG-RAN WG4 Meeting #103-e 		R4-2208015

Electronic meeting, 9th – 20th May, 2022

Source:	Ericsson

Title:	Discussion on PUSCH demodulation requirements for SAN

Agenda item:	9.12.8.2.1

Document for:	Discussion

# Introduction

In the last RAN4 meeting, companies have discussions on the non-terrestrial network demodulation requirements. Following WF was agreed on the general issue for SAN and PUSCH demodulation requirement issues.

Issue 2-2-1: Scope of PUSCH requirements

Agreements

- Not to consider the requirements for mapping Type B with non-slot transmission

Candidate options

- Option 1: Do not consider 2 step RACH case

- Option 2: Consider the 2 step RACH case

Issue 2-2-2: Channel model for PUSCH

Agreements

- Select NTN-TDL-A and NTN-TDL-C as the channel model for PUSCH requirements

Issue 2-2-3: SCS/CBW set for PUSCH requirements

Candidate options

- Option 1: 15kHz SCS: 5MHz/10MHz/20MHz, 30kHz SCS: 10MHz/20MHz

- Option 2: a few of PRBs for all SCS

Issue 2-2-4: Modulation order for PUSCH requirements

Agreements

- Select MCS4 for PUSCH requirements

Issue 2-2-5: Antenna configuration for PUSCH requirements

Agreements

- UE 1Tx

- SAN 2Rx

Candidate options

- FFS: SAN 1Rx, SAN 4Rx and SAN 8Rx

Issue 2-2-6: Test parameters for NTN PUSCH

For information, not agreement

- Interested companies are encouraged to provide initial simulation results in next meeting.

| Parameter |  | Value |
| --- | --- | --- |
| Channel model |  | NTN-TDL-A and NTN-TDL-C |
| MCS |  | MCS4 |
| Transform precoding |  | Disabled and Enabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0, group hopping and sequence hopping are disabled |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | FFS |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |

Issue 2-2-7: Test parameters for NTN UL timing adjustment

For information, not agreement

- Interested companies are encouraged to provide initial simulation results in next meeting.

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | FFS |
| MCS |  | MCS4 |
| Channel model |  | NTN-TDL-A |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
| Time domain resource assignment | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
|  | PUSCH mapping type | Both A and B |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | FFS |
|  | Starting PRB index | FFS |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note 1) | slot #1 in radio frames |
|  | SRS resource allocation | FFS |
| NOTE 1. The transmission of SRS is optional. And the transmission comb and SRS periodic are configured as KTC = 2, and TSRS = 10 respectively. |  |  |

| Parameter | Scenario X |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: NTN-TDLA<Doppler>-<DS> |
| UE speed | 120 km/h |
| CP length | Normal |
| A | 15 kHz: 10 s 30 kHz: 5 s |
|  | 15 kHz: 0.04 s-1 30 kHz: 0.08 s-1 |

Issue 2-2-8: Test parameters for NTN PUSCH repetition type A

For information, not agreement

- Interested companies are encouraged to provide initial simulation results in next meeting.

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel model |  | NTN-TDL-A |
| MCS |  | 5 in Table 3 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 2] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain resource assignment | RB assignment | FFS |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 2: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

Issue 2-2-9: Test parameters for NTN msgA PUSCH for 2-step RA type

For information, not agreement

- Interested companies are encouraged to provide initial simulation results in next meeting.

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | FFS |
| Channel model |  | NTN-TDL-A |
| MCS |  | FFS |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos2 or pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID = 0 |
| Time domain resource assignment | PUSCH mapping type | Both A and B |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 2 PRBs |
|  | Starting PRB index | 0 |
|  | Frequency hopping | Disabled |
| Time offset (TO) Cycling (µs) | start:end | 15k SCS: FFS |
|  |  | 30k SCS: FFS |
| Test Metric | BLER | 0.01 |
| NOTE 1: A single requirement is defined that is applicable regardless of whether pos1 or pos2 is configured for the additional DM-RS position NOTE 2: The power ratio between preamble and msgA (msgA-DeltaPreamble) is set to be sufficient to achieve 100% preamble detection. The SNR for the requirement is defined on the msgA PUSCH |  |  |

In this contribution, open issues for general configurations of SAN demodulation and  PUSCH demodulation requirements will be further analyzed.

# Discussion

## 2.1. 	 General

Channel model

The channel model for PUSCH has been discussed in another our contribution [2], and the more investigation is needed on the satellite elevation angle and corresponding scenario. The performance should be defined for the worst case.

Scope

The 2-step RACH is targeting to reduce the access time when the UE condition is good (high RSRP). This feature would be helpful for NTN deployment regarding very large RTT. RAN2 have agreement that both 2-step RACH and 4-step RACH are supported for NTN network.

On the other hand, NTN UE can pre-compensate the timing before transmission based on GNSS and ephemeris. The common estimation errors are ±50m for satellite position and ±30m for UE position. In that case, the remaining timing error could be quite small, i.e., 0.3us. Compared to the normal PUSCH transmission, it might not be an issue from the demodulation point of view.

Observation 1: The remaining timing error could be very small in MsgA of 2-step RACH regarding UE pre-compensation precision.

## 2.2. 	 PUSCH configurations

SCS and BW

The problem here is the power limitation in UL transmission would impact the PRB allocation in real life. Regarding the link budget table 2-1 in [3], target UL CNR for LEO600 is 2.8dB (satellite RF parameter set1) or -3.2dB (satellite RF parameter set2) with the bandwidth 0.4MHz. If higher channel bandwidth is applied, lower target CNR will be achieved.

Observation 2: Only very limited PRBs could achieve targe SNR based on link budget.

If better condition is considered, it is still suspicious that full bandwidth could be applied. To estimate the real network condition, a few of PRB allocation is reasonable for the requirement. Furthermore, the benefit is no need to simulate and test different bandwidth if the requirements are defined for a few of PRB allocation. The specific number of PRB could be further discussed.

Proposal 1: It is better to define the SAN PUSCH requirement for a few of PRB allocation.

Antenna configuration

Based on the analysis in another our contribution [2], both 1Rx and 2Rx are typical for satellite. Regarding the additional depolarization loss, 1Rx would have more difficulties to achieve enough SNR for PUSCH transmission regarding the analysis above.

Observation 3: 2Rx would be better for SAN demodulation requirement than 1Rx regarding to less additional loss caused by possible depolarization.

On the other hand, it is still possible for an NTN UE access 1Rx satellite when the condition is good enough. It seems no particular limitation for a 1Rx satellite to be adopted in NTN network. It might be practical to take LOS channel for 1Rx satellite requirement.

Proposal 2: Define SAN PUSCH demodulation requirement for 1Tx2Rx and 1Tx1Rx. Only take LOS channel for 1Tx1Rx requirements.

MCS

Both MCS 2 and 4 could be possible for SAN PUSCH demodulation, but the channel model has not been settled yet. The simulation is needed when channel model is agreed. The decision could depend on the NLOS/LOS channel and antenna configuration. The worse performance but feasible MCS should be chosen for the requirement.

Observation 4: The decision of MCS depends on the simulation with agreed channel model, but the proper channel model should be investigated at the first.

# Conclusions

Observation 1: The remaining timing error could be very small in MsgA of 2-step RACH regarding UE pre-compensation precision.

Observation 2: Only very limited PRBs could achieve targe SNR based on link budget.

Proposal 1: It is better to define the SAN PUSCH requirement for a few of PRB allocation.

Observation 3: 2Rx would be better for SAN demodulation requirement than 1Rx regarding to less additional loss caused by possible depolarization.

Proposal 2: Define SAN PUSCH demodulation requirement for 1Tx2Rx and 1Tx1Rx. Only take LOS channel for 1Tx1Rx requirements.

Observation 4: The decision of MCS depends on the simulation with agreed channel model, but the proper channel model should be investigated at the first.

# References

[1]	R4-2207198, WF for SAN demodulation, Qualcomm

[2]	R4-2208014, Discussion on general issue for SAN and UE demodulation requirement, Ericsson.

[3]	R4-2204027, Discussion on general issue of NTN demodulation, Ericsson

[4] 	TR38.821 v16.1.0