3GPP TSG-RAN WG4 Meeting #103-e 		R4-2208016

Electronic meeting, 9th – 20th May, 2022

Source:	Ericsson

Title:	Discussion on PUCCH demodulation requirements for SAN

Agenda item:	9.12.8.2.2

Document for:	Discussion

# Introduction

In the last RAN4 meeting, companies have discussions on the non-terrestrial network demodulation requirements. Following WF was agreed on PUCCH demodulation requirments for SAN.

Issue 2-3-1: Scope of PUCCH requirements

Agreements

- In addition to PUCCH format 0/1/2/3/4, RAN4 to define NTN multi-slot PUCCH demodulation requirements

- Prioritize UCI with HARQ on PUCCH demodulation requirement

Issue 2-3-2: Channel model for PUCCH requirements

Agreements

- RAN4 to only consider NTN-TDLA channel model for PUCCH requirements definition

Issue 2-3-3: SCS/CBW set for PUCCH requirements

Agreements

- To follow the same SCS/CBW set as PUSCH as the start point

Candidate options

- Option 1: No need to reduce test

- Option 2: Need to reduce test cases (specify if any)

Issue 2-3-4: Antenna configuration for PUCCH

Agreements

- UE 1Tx

- SAN 2Rx

Candidate options

- FFS: SAN 1Rx, SAN 4Rx and SAN 8Rx

Issue 2-3-5: Test parameters for NTN PUCCH format 0/1/2/3/4

For information, not agreement

- Interested companies are encouraged to provide initial simulation results in next meeting.

| Parameter | Test |
| --- | --- |
| PUCCH format | 0 |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |
| Channel model | NTN-TDLA |
| Number of OFDM symbols | FFS |
| Test metric | ACK missed detection probability <1% |

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

| Parameter | Value |
| --- | --- |
| PUCCH format | 3 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | FFS |
| Number of symbols | FFS |
| The number of UCI information bits | FFS |
| First symbol | 0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | FFS |
| Channel model | NTN-TDLA |
| Test metric | FFS |

| Parameter | Value |
| --- | --- |
| PUCCH format | 4 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | FFS |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | FFS |
| Channel model | NTN-TDLA |
| Test metric | FFS |

Issue 2-3-6:  Test parameters for NTN PUCCH multi-slot PUCCH format 1

For information, not agreement

- Interested companies are encouraged to provide initial simulation results in next meeting.

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 (multi-slot) |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

In this contribution, open issues for PUCCH demodulation will be further analyzed.

# Discussion

Channel model

As discussed in another our contribution [2], the NTN-TDLA channel with the angle leading to the worst performance should be evaluated by simulations for PUSCH at the first.  PUCCH could take the same channel as PUSCH.

SCS and CBW

Regarding the discussion in our contribution for PUSCH [3], the UL PUSCH PRBs might be quite limited due to power limitation. If it is true, it won’t be necessary to test all bandwidths for PUSCH, and PUCCH could be the same situation as well. If full bandwidth could be applied based on the further investigation, only testing minimum and maximum supported bandwidth could be better. The Rel-15 applicability rule for FR1 PUCCH bandwidth could be reused.

Proposal 1: Follow the agreement on SAN PUSCH demodulation discussion.

- If only a few of PRBs can be used for UL, only one requirement will be applied for all bandwidths.

- If full bandwidth can be used for UL, only requirements for minimum and maximum bandwidths could be introduced. The Rel-15 FR1 PUCCH applicability rule on bandwidth can be reused.

Antenna configuration

Based on the analysis in [2], we think both 1Tx1Rx and 1Tx2Rx are typical for satellite. 1Tx1Rx would have 3dB more depolarization loss compared to 1Tx2Rx which looks like a big limitation for NTN UL regarding quite small UE power. But it is still possible for a UE to access 1Rx satellite when condition is good. In that case, LOS channel might be suitable for 1Rx requirements.

Proposal 2: Define SAN PUCCH requirements for 1Tx2Rx and 1Tx1Rx. Only take LOS channel model for 1Tx1Rx requirements.

Number of PUCCH PRB in format 2/3

Regarding the possibility of a few PRB allocation for UL due to power limitation, the PUCCH PRB number should also be considered either. For format 2/3, the PRB number should depend on the conclusion of PUSCH part.

Proposal 3: The PUCCH PRB number of PF2/3 need to wait for the conclusion of PUSCH part.

DM-RS configuration for format 3/4

In Rel-15 FR1, PUCCH format 3/4 have manufactory declaration on DM-RS configuration. In that case, requirements are defined for both with and without additional DM-RS.

For SAN, it is not clear that if the similar manufactory declaration would be applied. If yes, then the same approach for the requirement should also be used, otherwise, there will be no requirements for a certain SAN don’t support the DM-RS configuration which is defined in the requirement.

Proposal 4: If the same BS manufactory declaration in Rel-15 would be applied for SAN, PUCCH format 3/4 requirements should be defined for both with and without additional DM-RS to avoid no requirement situation.

# Conclusions

Proposal 1: Follow the agreement on SAN PUSCH demodulation discussion.

- If only a few of PRBs can be used for UL, only one requirement will be applied for all bandwidths.

- If full bandwidth can be used for UL, only requirements for minimum and maximum bandwidths could be introduced. The Rel-15 FR1 PUCCH applicability rule on bandwidth can be reused.

Proposal 2: Define SAN PUCCH requirements for 1Tx2Rx and 1Tx1Rx. Only take LOS channel model for 1Tx1Rx requirements.

Proposal 3: The PUCCH PRB number of PF2/3 need to wait for the conclusion of PUSCH part.

Proposal 4: If the same BS manufactory declaration in Rel-15 would be applied for SAN, PUCCH format 3/4 requirements should be defined for both with and without additional DM-RS to avoid no requirement situation.

# References

[1]	R4-2207198, WF for SAN demodulation, Huawei, HiSilicon

[2]	R4-2208014, Discussion on general issues for SAN and UE demodulation requirement, Ericsson

[3] 	R4-2208015, Discussion on PUSCH demodulation requirement for SAN, Ericsson