3GPP TSG-RAN WG4 Meeting #103-e 		R4-2208023

Electronic meeting, 9th – 20th May, 2022

Source:	Ericsson

Title:	Simulation results for FR2-2 PUCCH demodulation requirements

Agenda item:	9.15.10.3.2

Document for:	Information

# Introduction

In the last RAN4 meeting, companies have discussions on the FR2-2 demodulation requirements. Regarding the WF [1] in previous meeting, simulation results for PUCCH multi-PRB are delivered in this contribution

# Simulation results

## 2.1 	multi-PRB PF0

Table 2.1-1 Simulation results for multi-PRB PF0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 16 or 10 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 1 or 2 symbols (13 for 1 symbol, 12 for 2 symbol) |
| SCS and bandwidth | 120kHz SCS: 100M |
| Channel model | TDLA20_200 TDLA20_2000 TDLA5_200 TDLA5_2000 |
| Test metric | SNR@DTX àACK <1% |

Table 2.1-2 simulation results for multi-PRB PF0, 120kHz SCS 100MHz BW

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | Number of OFDM symbols | Number of PRB | Channel bandwidth / SNR (dB) DTX->ACK <1% |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA5-200 Low | 1 | 16 | -6.5 |
|  |  |  | 2 | 16 | -8.6 |
|  |  |  | 1 | 10 | -4.5 |
|  |  |  | 2 | 10 | -6 |
|  |  | TDLA5-2000 Low | 1 | 16 | -6.6 |
|  |  |  | 2 | 16 | -8.4 |
|  |  |  | 1 | 10 | -4.6 |
|  |  |  | 2 | 10 | -6.4 |
|  |  | TDLA20-200 Low | 1 | 16 | -7.5 |
|  |  |  | 2 | 16 | -9.6 |
|  |  |  | 1 | 10 | -5.4 |
|  |  |  | 2 | 10 | -7.3 |
|  |  | TDLA20-2000 Low | 1 | 16 | -7.5 |
|  |  |  | 2 | 16 | -9.6 |
|  |  |  | 1 | 10 | -5.3 |
|  |  |  | 2 | 10 | -7.2 |

## 2.2 	 Multi-PRB PF1

Table 2.2-1 Simulation results for multi-PRB PF1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 16 or 10 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| SCS and bandwidth | 120kHz SCS: 100M |
| Channel model | TDLA20_200 / TDLA20_2000 TDLA5_200 / TDLA5_2000 |
| Test metric | SNR@NACK àACK <0.1% |
|  | SNR@ACK miss <1% |

Table 2.2-2 simulation results for multi-PRB PF1, 120kHz SCS, 100MHz

| Number of TX | Number of Demodulation | Cyclic Prefix | Propagation conditions and | Number of PRB | Channel bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| antennas | Branches |  | correlation matrix |  | NACK -> ACK<0.1% | ACK miss <1% |
| 1 | 2 | Normal | TDLA5-200 Low | 16 | -19.2 | -19.5 |
|  |  |  |  | 10 | -17.6 | -17.4 |
|  |  | Normal | TDLA5-2000 Low | 16 | -18.7 | -16.7 |
|  |  |  |  | 10 | -19.4 | -17.5 |
|  |  | Normal | TDLA20-200 Low | 16 | -18.8 | -18.9 |
|  |  |  |  | 10 | -17.1 | -17.4 |
|  |  | Normal | TDLA20-2000 Low | 16 | -19.1 | -18.9 |
|  |  |  |  | 10 | -17.1 | -17.1 |

## 2.3 	 Multi-PRB PF4

Table 2.3-1 Simulation results for multi-PRB PF4

| Parameter | Test |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hoppingstartingPRB | 0 |
| Number of PRBs | 16 or 10 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| DM-RS position | pos0 or pos1 |
| SCS and bandwidth | 120kHz SCS: 100M |
| Channel model | TDLA20_200 / TDLA20_2000 TDLA5_200 / TDLA5_2000 |
| Test metric | SNR@BLER <1% |

Table 2.3-2 simulation results for multi-PRB PF4, 120kHz SCS, 100MHz

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix | Additional DM-RS configuration | Number of PRB | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA5-200 Low | 1+0 | 16 | -13.7 |
|  |  |  |  | 1+1 | 16 | -14.1 |
|  |  |  |  | 1+0 | 10 | -11.7 |
|  |  |  |  | 1+1 | 10 | -12 |
|  |  | Normal | TDLA5-2000 Low | 1+0 | 16 | -13.6 |
|  |  |  |  | 1+1 | 16 | -13.9 |
|  |  |  |  | 1+0 | 10 | -11.9 |
|  |  |  |  | 1+1 | 10 | -11.8 |
|  |  | Normal | TDLA20-200 Low | 1+0 | 16 | -13 |
|  |  |  |  | 1+1 | 16 | -13.8 |
|  |  |  |  | 1+0 | 10 | -11.3 |
|  |  |  |  | 1+1 | 10 | -11.6 |
|  |  | Normal | TDLA20-2000 Low | 1+0 | 16 | -13 |
|  |  |  |  | 1+1 | 16 | -13.9 |
|  |  |  |  | 1+0 | 10 | -11.2 |
|  |  |  |  | 1+1 | 10 | -11.7 |

# Conclusions

Simulation results are delivered for FR2-2 PUCCH demodulation further discussion.

# References

[1]	R4-2207223, WF for NR extended to 71GHz demodulation, Intel, RAN4#102-e