3GPP TSG-RAN WG4 Meeting #103-e	R4-2208028

Online Meeting, May 9 – 20, 2022

Agenda item:	9.20.2.2

Source: 	Qualcomm Incorporated

Title: 	Additional simulation results with reduced number of PRS samples

Document for:	Discussion

# Introduction

RAN4 reached some agreements regarding the simulation assumptions for measurement accuracy with a reduced number of samples in RAN4#101-e [1, 2] and in RAN4#101-bis-e [3].

This paper includes additional simulation results for the following:

- UE Rx-Tx time difference

- PRS-RSRP

- RSTD

# Simulation results for UE Rx-Tx time difference

This section includes simulation results for FR1, SCS = 15 kHz and SCS = 30 kHz, with propagation channels AWGN and TDL-A.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 15 | 256 | 1 | 1 | 120 | 122 | 131 |
|  |  |  |  | 2 | 1 | 118 | 120 | 124 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 118 | 118 | 121 |
|  |  |  |  | 2 | 4 | 118 | 118 | 119 |
|  |  |  |  | 4 | 4 | (78) |  | (101) |
|  | 52 |  | 128 | 1 | 1 | 58 | 58 | 61 |
|  |  |  |  | 2 | 1 | 58 | 58 | 59 |
|  |  |  |  | 4 | 1 | (59) |  | (75) |
|  |  |  |  | 1 | 4 | 58 | 58 | 58 |
|  |  |  |  | 2 | 4 | 58 | 58 | 58 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 104 |  | 64 | 1 | 1 | 31 | 31 | 31 |
|  |  |  |  | 2 | 1 | 31 | 31 | 31 |
|  |  |  |  | 4 | 1 | (30) |  | (37) |
|  |  |  |  | 1 | 4 | 31 | 31 | 31 |
|  |  |  |  | 2 | 4 | 31 | 31 | 31 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 1: UE Rx-Tx simulation results for FR1, SCS = 15 kHz, AWGN. Values in parentheses are the Rel-16 baseline requirements (without margin).

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 30 | 128 | 1 | 1 | 58 | 59 | 66 |
|  |  |  |  | 2 | 1 | 58 | 58 | 61 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 58 | 58 | 60 |
|  |  |  |  | 2 | 4 | 58 | 58 | 59 |
|  |  |  |  | 4 | 4 | (57) |  | (58) |
|  | 48 |  | 64 | 1 | 1 | 30 | 30 | 31 |
|  |  |  |  | 2 | 1 | 30 | 30 | 30 |
|  |  |  |  | 4 | 1 | (30) |  | (39) |
|  |  |  |  | 1 | 4 | 30 | 30 | 30 |
|  |  |  |  | 2 | 4 | 30 | 30 | 30 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 132 |  | 32 | 1 | 1 | 16 | 16 | 16 |
|  |  |  |  | 2 | 1 | 16 | 16 | 16 |
|  |  |  |  | 4 | 1 | (15) |  | (16) |
|  |  |  |  | 1 | 4 | 16 | 16 | 16 |
|  |  |  |  | 2 | 4 | 16 | 16 | 16 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 2: UE Rx-Tx simulation results for FR1, SCS = 30 kHz, AWGN. Values in parentheses are the Rel-16 baseline requirements (without margin).

Observation 1: There are differences in sampling rate captured in the simulation assumptions Tdoc from RAN4#101 [2] vs. the assumptions in Rel-16.

- for FR1, SCS = 15 kHz, num RB = 24, the sampling period was 128 Tc in Rel-16

- for FR2, SCS = 60 kHz, num RB = 24, the sampling period was 32 Tc in Rel-16

- for FR2, SCS = 120 kHz, num RB = 32, the sampling period was 16 Tc in Rel-16

Observation 2: For FR1, SCS = 15 kHz and 30 kHz, and num RB ≥48, Rel-16 UE Rx-Tx accuracy requirements in AWGN at SINR = -13 dB can be met at SINR ≥ -6 dB with number of samples M1 = 1, 2.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 15 | 256 | 1 | 1 | 163 | 164 | 224 |
|  |  |  |  | 2 | 1 | 162 | 163 | 186 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 162 | 163 | 167 |
|  |  |  |  | 2 | 4 | 162 | 162 | 163 |
|  |  |  |  | 4 | 4 | (137) |  | (180) |
|  | 52 |  | 128 | 1 | 1 | 106 | 106 | 113 |
|  |  |  |  | 2 | 1 | 106 | 106 | 108 |
|  |  |  |  | 4 | 1 | (96) |  | (98) |
|  |  |  |  | 1 | 4 | 105 | 105 | 106 |
|  |  |  |  | 2 | 4 | 105 | 105 | 106 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 104 |  | 64 | 1 | 1 | 66 | 66 | 69 |
|  |  |  |  | 2 | 1 | 65 | 66 | 66 |
|  |  |  |  | 4 | 1 | (62) |  | (68) |
|  |  |  |  | 1 | 4 | 65 | 65 | 65 |
|  |  |  |  | 2 | 4 | 65 | 65 | 64 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 3: UE Rx-Tx simulation results for FR1, SCS = 15 kHz, TDL-A. Values in parentheses are the Rel-16 baseline requirements (without margin).

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 30 | 128 | 1 | 1 | 106 | 109 | 159 |
|  |  |  |  | 2 | 1 | 105 | 106 | 116 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 105 | 105 | 107 |
|  |  |  |  | 2 | 4 | 104 | 105 | 104 |
|  |  |  |  | 4 | 4 | (87) |  | (87) |
|  | 48 |  | 64 | 1 | 1 | 69 | 70 | 83 |
|  |  |  |  | 2 | 1 | 69 | 69 | 74 |
|  |  |  |  | 4 | 1 | (68) |  | (85) |
|  |  |  |  | 1 | 4 | 69 | 69 | 71 |
|  |  |  |  | 2 | 4 | 69 | 69 | 71 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 132 |  | 32 | 1 | 1 | 42 | 43 | 43 |
|  |  |  |  | 2 | 1 | 42 | 43 | 43 |
|  |  |  |  | 4 | 1 | (44) |  | (44) |
|  |  |  |  | 1 | 4 | 42 | 43 | 43 |
|  |  |  |  | 2 | 4 | 43 | 43 | 43 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 4: UE Rx-Tx simulation results for FR1, SCS = 30 kHz, TDL-A. Values in parentheses are the Rel-16 baseline requirements (without margin).

Observation 3: For FR1, SCS = 15 kHz and 30 kHz, and num RB ≥ 104, Rel-16 UE Rx-Tx accuracy requirements in TDL-A at SINR = -13 dB can be met at SINR = -3 dB with number of samples M1 = 1, 2.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR2 | 32 | 120 | 32 | 1 | 1 | 15 | 15 | 16 |
|  |  |  |  | 2 | 1 | 15 | 15 | 15 |
|  |  |  |  | 4 | 1 | (12) |  | (14) |
|  |  |  |  | 1 | 4 | 15 | 15 | 15 |
|  |  |  |  | 2 | 4 | 15 | 15 | 15 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 64 |  | 16 | 1 | 1 | 8 | 8 | 8 |
|  |  |  |  | 2 | 1 | 8 | 8 | 8 |
|  |  |  |  | 4 | 1 | (7) |  | (9) |
|  |  |  |  | 1 | 4 | 8 | 8 | 8 |
|  |  |  |  | 2 | 4 | 8 | 8 | 8 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 128 |  | 8 | 1 | 1 | 4 | 4 | 4 |
|  |  |  |  | 2 | 1 | 4 | 4 | 4 |
|  |  |  |  | 4 | 1 | (4) |  | (4) |
|  |  |  |  | 1 | 4 | 4 | 4 | 4 |
|  |  |  |  | 2 | 4 | 4 | 4 | 4 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 5: UE Rx-Tx simulation results for FR2, SCS = 120 kHz, AWGN. Values in parentheses are the Rel-16 baseline requirements (without margin).

Observation 4: For FR2 and SCS = 120 kHz, Rel-16 UE Rx-Tx accuracy requirements in AWGN at SINR = -13 dB can be met at SINR ≥ -6 dB with number of samples M1 = 1, 2.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR2 | 32 | 120 | 32 | 1 | 1 | 82 | 86 | 155 |
|  |  |  |  | 2 | 1 | 83 | 85 | 90 |
|  |  |  |  | 4 | 1 | (61) |  | (60) |
|  |  |  |  | 1 | 4 | 83 | 86 | 87 |
|  |  |  |  | 2 | 4 | 82 | 84 | 86 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 64 |  | 16 | 1 | 1 | 34 | 78 | 82 |
|  |  |  |  | 2 | 1 | 34 | 79 | 81 |
|  |  |  |  | 4 | 1 | (64) |  | (66) |
|  |  |  |  | 1 | 4 | 34 | 79 | 81 |
|  |  |  |  | 2 | 4 | 33 | 79 | 81 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 128 |  | 8 | 1 | 1 | 29 | 30 | 79 |
|  |  |  |  | 2 | 1 | 29 | 30 | 78 |
|  |  |  |  | 4 | 1 | (55) |  | (62) |
|  |  |  |  | 1 | 4 | 29 | 30 | 78 |
|  |  |  |  | 2 | 4 | 29 | 30 | 78 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 6: UE Rx-Tx simulation results for FR2, SCS = 120 kHz, TDL-C. Values in parentheses are the Rel-16 baseline requirements (without margin).

Observation 5: For FR2, SCS = 120 kHz and num RB ≥ 64, Rel-16 UE Rx-Tx accuracy requirements in TDL-C at SINR = -13 dB can be met at SINR = -3 dB with number of samples M1 = 1, 2.

# Simulation results for PRS-RSRP

This section includes simulation results for FR1, SCS = 15 kHz and SCS = 30 kHz, with propagation channels AWGN and TDL-A.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 15 | 256 | 1 | 1 | 0.7 | 0.9 | 2.1 |
|  |  |  |  | 2 | 1 | 0.6 | 0.7 | 1.6 |
|  |  |  |  | 4 | 1 |  |  | (6.0) |
|  |  |  |  | 1 | 4 | 0.5 | 0.6 | 1.2 |
|  |  |  |  | 2 | 4 | 0.4 | 0.5 | 0.9 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 52 |  | 128 | 1 | 1 | 0.5 | 0.6 | 1.6 |
|  |  |  |  | 2 | 1 | 0.4 | 0.5 | 1.1 |
|  |  |  |  | 4 | 1 |  |  | (6.0) |
|  |  |  |  | 1 | 4 | 0.3 | 0.4 | 0.8 |
|  |  |  |  | 2 | 4 | 0.3 | 0.3 | 0.6 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 104 |  | 64 | 1 | 1 | 0.3 | 0.5 | 1.2 |
|  |  |  |  | 2 | 1 | 0.3 | 0.3 | 0.8 |
|  |  |  |  | 4 | 1 |  |  | (3.5) |
|  |  |  |  | 1 | 4 | 0.2 | 0.3 | 0.6 |
|  |  |  |  | 2 | 4 | 0.2 | 0.2 | 0.4 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 7: PRS-RSRP simulation results for FR1, SCS = 15 kHz, AWGN. Values in parentheses are the Rel-16 baseline requirements (without margin).

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 30 | 128 | 1 | 1 | 0.7 | 1.1 | 3.2 |
|  |  |  |  | 2 | 1 | 0.5 | 0.8 | 2.3 |
|  |  |  |  | 4 | 1 |  |  | (6.0) |
|  |  |  |  | 1 | 4 | 0.4 | 0.5 | 1.7 |
|  |  |  |  | 2 | 4 | 0.3 | 0.4 | 1.2 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 48 |  | 64 | 1 | 1 | 0.5 | 0.7 | 1.6 |
|  |  |  |  | 2 | 1 | 0.4 | 0.5 | 1.2 |
|  |  |  |  | 4 | 1 |  |  | (6.0) |
|  |  |  |  | 1 | 4 | 0.3 | 0.4 | 0.8 |
|  |  |  |  | 2 | 4 | 0.2 | 0.3 | 0.6 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 132 |  | 32 | 1 | 1 | 0.3 | 0.5 | 1.3 |
|  |  |  |  | 2 | 1 | 0.2 | 0.3 | 0.9 |
|  |  |  |  | 4 | 1 |  |  | (2.0) |
|  |  |  |  | 1 | 4 | 0.2 | 0.2 | 0.6 |
|  |  |  |  | 2 | 4 | 0.1 | 0.2 | 0.4 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 8: PRS-RSRP simulation results for FR1, SCS = 30 kHz, AWGN. Values in parentheses are the Rel-16 baseline requirements (without margin).

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 15 | 256 | 1 | 1 | 0.8 | 1.2 | 3.1 |
|  |  |  |  | 2 | 1 | 0.6 | 0.9 | 2.3 |
|  |  |  |  | 4 | 1 |  |  | (6.0) |
|  |  |  |  | 1 | 4 | 0.5 | 0.7 | 1.7 |
|  |  |  |  | 2 | 4 | 0.4 | 0.5 | 1.2 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 52 |  | 128 | 1 | 1 | 0.6 | 0.8 | 2.2 |
|  |  |  |  | 2 | 1 | 0.4 | 0.6 | 1.5 |
|  |  |  |  | 4 | 1 |  |  | (6.0) |
|  |  |  |  | 1 | 4 | 0.3 | 0.4 | 1.1 |
|  |  |  |  | 2 | 4 | 0.3 | 0.3 | 0.8 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 104 |  | 64 | 1 | 1 | 0.4 | 0.6 | 1.7 |
|  |  |  |  | 2 | 1 | 0.3 | 0.4 | 1.1 |
|  |  |  |  | 4 | 1 |  |  | (3.5) |
|  |  |  |  | 1 | 4 | 0.2 | 0.3 | 0.8 |
|  |  |  |  | 2 | 4 | 0.2 | 0.2 | 0.5 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 9: PRS-RSRP simulation results for FR1, SCS = 15 kHz, TDL-A. Values in parentheses are the Rel-16 baseline requirements (without margin).

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 30 | 128 | 1 | 1 | 1.0 | 1.6 | 4.7 |
|  |  |  |  | 2 | 1 | 0.7 | 1.1 | 3.4 |
|  |  |  |  | 4 | 1 |  |  | (6.0) |
|  |  |  |  | 1 | 4 | 0.5 | 0.8 | 2.4 |
|  |  |  |  | 2 | 4 | 0.4 | 0.6 | 1.8 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 48 |  | 64 | 1 | 1 | 0.6 | 0.8 | 2.1 |
|  |  |  |  | 2 | 1 | 0.4 | 0.6 | 1.5 |
|  |  |  |  | 4 | 1 |  |  | (6.0) |
|  |  |  |  | 1 | 4 | 0.3 | 0.4 | 1.0 |
|  |  |  |  | 2 | 4 | 0.3 | 0.3 | 0.7 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 132 |  | 32 | 1 | 1 | 0.4 | 0.6 | 1.8 |
|  |  |  |  | 2 | 1 | 0.3 | 0.4 | 1.2 |
|  |  |  |  | 4 | 1 |  |  | (2.0) |
|  |  |  |  | 1 | 4 | 0.2 | 0.3 | 0.7 |
|  |  |  |  | 2 | 4 | 0.2 | 0.2 | 0.5 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 10: PRS-RSRP simulation results for FR1, SCS = 30 kHz, TDL-A. Values in parentheses are the Rel-16 baseline requirements (without margin).

Observation 6: For FR1, Rel-16 PRS-RSRP absolute accuracy requirements at SINR = -13 dB can be met at SINR = -6 dB with number of samples M1 = 1, 2.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR2 | 32 | 120 | 32 | 1 | 1 | 0.7 | 0.9 | 2.2 |
|  |  |  |  | 2 | 1 | 0.5 | 0.7 | 1.8 |
|  |  |  |  | 4 | 1 |  |  | (±4.5) |
|  |  |  |  | 1 | 4 | 0.5 | 0.5 | 1.6 |
|  |  |  |  | 2 | 4 | 0.4 | 0.4 | 1.4 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 64 |  | 16 | 1 | 1 | 0.5 | 0.7 | 1.8 |
|  |  |  |  | 2 | 1 | 0.5 | 0.5 | 1.6 |
|  |  |  |  | 4 | 1 |  |  | (±4.5) |
|  |  |  |  | 1 | 4 | 0.4 | 0.4 | 1.4 |
|  |  |  |  | 2 | 4 | 0.4 | 0.4 | 1.3 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 128 |  | 8 | 1 | 1 | 0.4 | 0.5 | 1.6 |
|  |  |  |  | 2 | 1 | 0.4 | 0.5 | 1.4 |
|  |  |  |  | 4 | 1 |  |  | (±2.0) |
|  |  |  |  | 1 | 4 | 0.4 | 0.4 | 1.3 |
|  |  |  |  | 2 | 4 | 0.4 | 0.3 | 1.2 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 11: PRS-RSRP simulation results for FR2, SCS = 120 kHz, AWGN. Values in parentheses are the Rel-16 baseline requirements (without margin).

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR2 | 32 | 120 | 32 | 1 | 1 | 0.7 | 0.9 | 2.6 |
|  |  |  |  | 2 | 1 | 0.6 | 0.7 | 2.1 |
|  |  |  |  | 4 | 1 |  |  | (±4.5) |
|  |  |  |  | 1 | 4 | 0.5 | 0.6 | 1.9 |
|  |  |  |  | 2 | 4 | 0.5 | 0.6 | 1.7 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 64 |  | 16 | 1 | 1 | 0.6 | 0.7 | 2.1 |
|  |  |  |  | 2 | 1 | 0.5 | 0.6 | 1.9 |
|  |  |  |  | 4 | 1 |  |  | (±4.5) |
|  |  |  |  | 1 | 4 | 0.4 | 0.5 | 1.8 |
|  |  |  |  | 2 | 4 | 0.4 | 0.6 | 1.6 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 128 |  | 8 | 1 | 1 | 0.5 | 0.6 | 1.8 |
|  |  |  |  | 2 | 1 | 0.4 | 0.5 | 1.7 |
|  |  |  |  | 4 | 1 |  |  | (±2.0) |
|  |  |  |  | 1 | 4 | 0.4 | 0.5 | 1.6 |
|  |  |  |  | 2 | 4 | 0.4 | 0.5 | 1.6 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 12: PRS-RSRP simulation results for FR2, SCS = 120 kHz, TDL-C. Values in parentheses are the Rel-16 baseline requirements (without margin).

Observation 7: For FR2, Rel-16 PRS-RSRP absolute accuracy requirements at SINR = -13 dB can be met at SINR = -6 dB with number of samples M1 = 1, 2.

# Simulation results for RSTD

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%,95%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = 0 dB SINR = -3 dB | SINRref = -3 dB SINR = -6 dB | SINRref = -6 dB SINR = -13 dB |
| FR1 | 24 | 15 | 256 | 1 | 1 | ±145 | ±145 | ±159 |
|  |  |  |  | 2 | 1 | ±145 | ±145 | ±153 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | ±146 | ±146 | ±146 |
|  |  |  |  | 2 | 4 | ±146 | ±145 | ±146 |
|  |  |  |  | 4 | 4 |  |  | (±132) |
|  | 52 |  | 128 | 1 | 1 | ±66 | ±66 | ±67 |
|  |  |  |  | 2 | 1 | ±66 | ±66 | ±66 |
|  |  |  |  | 4 | 1 |  |  | (±98) |
|  |  |  |  | 1 | 4 | ±66 | ±66 | ±66 |
|  |  |  |  | 2 | 4 | ±66 | ±66 | ±66 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 104 |  | 64 | 1 | 1 | ±39 | ±39 | ±39 |
|  |  |  |  | 2 | 1 | ±39 | ±39 | ±39 |
|  |  |  |  | 4 | 1 |  |  | (±42) |
|  |  |  |  | 1 | 4 | ±39 | ±39 | ±39 |
|  |  |  |  | 2 | 4 | ±39 | ±39 | ±39 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 13: RSTD simulation results for FR1, SCS = 15 kHz, AWGN. Values in parentheses are the Rel-16 baseline requirements (without margin).

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%,95%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = 0 dB SINR = -3 dB | SINRref = -3 dB SINR = -6 dB | SINRref = -6 dB SINR = -13 dB |
| FR1 | 24 | 30 | 128 | 1 | 1 | ±68 | ±68 | ±69 |
|  |  |  |  | 2 | 1 | ±67 | ±68 | ±68 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | ±67 | ±67 | ±68 |
|  |  |  |  | 2 | 4 | ±67 | ±67 | ±68 |
|  |  |  |  | 4 | 4 |  |  | (±75) |
|  | 48 |  | 64 | 1 | 1 | ±38 | ±38 | ±39 |
|  |  |  |  | 2 | 1 | ±38 | ±38 | ±38 |
|  |  |  |  | 4 | 1 |  |  | (±48) |
|  |  |  |  | 1 | 4 | ±38 | ±38 | ±38 |
|  |  |  |  | 2 | 4 | ±38 | ±38 | ±38 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 132 |  | 32 | 1 | 1 | ±18 | ±18 | ±18 |
|  |  |  |  | 2 | 1 | ±18 | ±18 | ±18 |
|  |  |  |  | 4 | 1 |  |  | (±24) |
|  |  |  |  | 1 | 4 | ±18 | ±18 | ±18 |
|  |  |  |  | 2 | 4 | ±18 | ±18 | ±18 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 14: RSTD simulation results for FR1, SCS = 30 kHz, AWGN. Values in parentheses are the Rel-16 baseline requirements (without margin).

Observation 8: For FR1, SCS = 15 kHz and 30 kHz, and num RB ≥48, Rel-16 RSTD accuracy requirements in AWGN at SINRref/SINR = -6/-13 dB can be met at SINRref/SINR = -3/-6 dB with number of samples M1 = 1, 2.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%,95%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = 0 dB SINR = -3 dB | SINRref = -3 dB SINR = -6 dB | SINRref = -6 dB SINR = -13 dB |
| FR1 | 24 | 15 | 256 | 1 | 1 | ±161 | ±161 | ±220 |
|  |  |  |  | 2 | 1 | ±161 | ±161 | ±184 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | ±155 | ±155 | ±161 |
|  |  |  |  | 2 | 4 | ±155 | ±153 | ±153 |
|  |  |  |  | 4 | 4 |  |  | (±247) |
|  | 52 |  | 128 | 1 | 1 | ±101 | ±102 | ±103 |
|  |  |  |  | 2 | 1 | ±101 | ±101 | ±103 |
|  |  |  |  | 4 | 1 |  |  | (±140) |
|  |  |  |  | 1 | 4 | ±101 | ±101 | ±101 |
|  |  |  |  | 2 | 4 | ±101 | ±101 | ±101 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 104 |  | 64 | 1 | 1 | ±53 | ±47 | ±48 |
|  |  |  |  | 2 | 1 | ±49 | ±47 | ±47 |
|  |  |  |  | 4 | 1 |  |  | (±86) |
|  |  |  |  | 1 | 4 | ±48 | ±47 | ±47 |
|  |  |  |  | 2 | 4 | ±48 | ±47 | ±46 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 15: RSTD simulation results for FR1, SCS = 15 kHz, TDL-A. Values in parentheses are the Rel-16 baseline requirements (without margin).

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%,95%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = 0 dB SINR = -3 dB | SINRref = -3 dB SINR = -6 dB | SINRref = -6 dB SINR = -13 dB |
| FR1 | 24 | 30 | 128 | 1 | 1 | ±100 | ±100 | ±119 |
|  |  |  |  | 2 | 1 | ±100 | ±100 | ±105 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | ±100 | ±100 | ±100 |
|  |  |  |  | 2 | 4 | ±100 | ±100 | ±100 |
|  |  |  |  | 4 | 4 |  |  | (±118) |
|  | 48 |  | 64 | 1 | 1 | ±50 | ±50 | ±56 |
|  |  |  |  | 2 | 1 | ±50 | ±50 | ±52 |
|  |  |  |  | 4 | 1 |  |  | (±109) |
|  |  |  |  | 1 | 4 | ±50 | ±50 | ±51 |
|  |  |  |  | 2 | 4 | ±50 | ±50 | ±51 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 132 |  | 32 | 1 | 1 | ±24 | ±23 | ±24 |
|  |  |  |  | 2 | 1 | ±24 | ±23 | ±24 |
|  |  |  |  | 4 | 1 |  |  | (±28) |
|  |  |  |  | 1 | 4 | ±24 | ±24 | ±24 |
|  |  |  |  | 2 | 4 | ±24 | ±24 | ±23 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 16: RSTD simulation results for FR1, SCS = 30 kHz, TDL-A. Values in parentheses are the Rel-16 baseline requirements (without margin).

Observation 9: For FR1, SCS = 15 kHz and 30 kHz, and num RB ≥ 24, Rel-16 RSTD accuracy requirements in TDL-A at SINRref/SINR = -6/-13 dB can be met at SINRref/SINR = -3/-6 dB with number of samples M1 = 1, 2.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%,95%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = 0 dB SINR = -3 dB | SINRref = -3 dB SINR = -6 dB | SINRref = -6 dB SINR = -13 dB |
| FR2 | 32 | 120 | 32 | 1 | 1 | ±17 | ±17 | ±18 |
|  |  |  |  | 2 | 1 | ±17 | ±17 | ±17 |
|  |  |  |  | 4 | 1 |  |  | (±24) |
|  |  |  |  | 1 | 4 | ±17 | ±17 | ±17 |
|  |  |  |  | 2 | 4 | ±17 | ±17 | ±17 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 64 |  | 16 | 1 | 1 | ±10 | ±10 | ±9 |
|  |  |  |  | 2 | 1 | ±9 | ±9 | ±10 |
|  |  |  |  | 4 | 1 |  |  | (±13) |
|  |  |  |  | 1 | 4 | ±10 | ±10 | ±10 |
|  |  |  |  | 2 | 4 | ±9 | ±9 | ±9 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 128 |  | 8 | 1 | 1 | ±5 | ±5 | ±5 |
|  |  |  |  | 2 | 1 | ±5 | ±5 | ±5 |
|  |  |  |  | 4 | 1 |  |  | (±6) |
|  |  |  |  | 1 | 4 | ±5 | ±5 | ±5 |
|  |  |  |  | 2 | 4 | ±5 | ±5 | ±5 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 17: RSTD simulation results for FR2, SCS = 120 kHz, AWGN. Values in parentheses are the Rel-16 baseline requirements (without margin).

Observation 10: For FR2 and SCS = 120 kHz, Rel-16 RSTD accuracy requirements in AWGN at SINRref/SINR = -6/-13 dB can be met at SINRref/SINR = -3/-6 dB with number of samples M1 = 1, 2.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%,95%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = 0 dB SINR = -3 dB | SINRref = -3 dB SINR = -6 dB | SINRref = -6 dB SINR = -13 dB |
| FR2 | 32 | 120 | 32 | 1 | 1 | ±57 | ±59 | ±62 |
|  |  |  |  | 2 | 1 | ±57 | ±59 | ±57 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | ±57 | ±59 | ±56 |
|  |  |  |  | 2 | 4 | ±57 | ±59 | ±57 |
|  |  |  |  | 4 | 4 |  |  | (±48) |
|  | 64 |  | 16 | 1 | 1 | ±35 | ±52 | ±57 |
|  |  |  |  | 2 | 1 | ±36 | ±54 | ±56 |
|  |  |  |  | 4 | 1 |  |  | (±54) |
|  |  |  |  | 1 | 4 | ±36 | ±53 | ±55 |
|  |  |  |  | 2 | 4 | ±34 | ±52 | ±54 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 128 |  | 8 | 1 | 1 | ±29 | ±31 | ±73 |
|  |  |  |  | 2 | 1 | ±29 | ±31 | ±69 |
|  |  |  |  | 4 | 1 |  |  | (±36) |
|  |  |  |  | 1 | 4 | ±29 | ±32 | ±57 |
|  |  |  |  | 2 | 4 | ±29 | ±30 | ±56 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 18: RSTD simulation results for FR2, SCS = 120 kHz, TDL-C. Values in parentheses are the Rel-16 baseline requirements (without margin).

Observation 11: For FR2, SCS = 120 kHz and num RB ≥ 64, Rel-16 UE RSTD requirements in TDL-C at SINRref/SINR = -6/-13 dB can be met at SINRref/SINR = -3/-6 dB with number of samples M1 = 1, 2.

# Conclusions

Observation 1: There are differences in sampling rate captured in the simulation assumptions Tdoc from RAN4#101 [2] vs. the assumptions in Rel-16.

- for FR1, SCS = 15 kHz, num RB = 24, the sampling period was 128 Tc in Rel-16

- for FR2, SCS = 60 kHz, num RB = 24, the sampling period was 32 Tc in Rel-16

- for FR2, SCS = 120 kHz, num RB = 32, the sampling period was 16 Tc in Rel-16

Observation 2: For FR1, SCS = 15 kHz and 30 kHz, and num RB ≥48, Rel-16 UE Rx-Tx accuracy requirements in AWGN at SINR = -13 dB can be met at SINR ≥ -6 dB with number of samples M1 = 1, 2.

Observation 3: For FR1, SCS = 15 kHz and 30 kHz, and num RB ≥ 104, Rel-16 UE Rx-Tx accuracy requirements in TDL-A at SINR = -13 dB can be met at SINR = -3 dB with number of samples M1 = 1, 2.

Observation 4: For FR2 and SCS = 120 kHz, Rel-16 UE Rx-Tx accuracy requirements in AWGN at SINR = -13 dB can be met at SINR ≥ -6 dB with number of samples M1 = 1, 2.

Observation 5: For FR2, SCS = 120 kHz and num RB ≥ 64, Rel-16 UE Rx-Tx accuracy requirements in TDL-C at SINR = -13 dB can be met at SINR = -3 dB with number of samples M1 = 1, 2.

Observation 6: For FR1, Rel-16 PRS-RSRP absolute accuracy requirements at SINR = -13 dB can be met at SINR = -6 dB with number of samples M1 = 1, 2.

Observation 7: For FR2, Rel-16 PRS-RSRP absolute accuracy requirements at SINR = -13 dB can be met at SINR = -6 dB with number of samples M1 = 1, 2.

Observation 8: For FR1, SCS = 15 kHz and 30 kHz, and num RB ≥48, Rel-16 RSTD accuracy requirements in AWGN at SINRref/SINR = -6/-13 dB can be met at SINRref/SINR = -3/-6 dB with number of samples M1 = 1, 2.

Observation 9: For FR1, SCS = 15 kHz and 30 kHz, and num RB ≥ 24, Rel-16 RSTD accuracy requirements in TDL-A at SINRref/SINR = -6/-13 dB can be met at SINRref/SINR = -3/-6 dB with number of samples M1 = 1, 2.

Observation 10: For FR2 and SCS = 120 kHz, Rel-16 RSTD accuracy requirements in AWGN at SINRref/SINR = -6/-13 dB can be met at SINRref/SINR = -3/-6 dB with number of samples M1 = 1, 2.

Observation 11: For FR2, SCS = 120 kHz and num RB ≥ 64, Rel-16 UE RSTD requirements in TDL-C at SINRref/SINR = -6/-13 dB can be met at SINRref/SINR = -3/-6 dB with number of samples M1 = 1, 2.

# References

[1] R4-2120419, WF on NR Positioning Enhancements (Part 1), RAN4#101-e.

[2] R4-2120330, Link level simulation assumptions for NR PRS measurements with reduced latency, RAN4#101-e.

[3] R4-2202776, WF on NR Positioning Enhancements (Part 1), RAN4#101-bis-e.