3GPP TSG-RAN WG4 Meeting # 103-e	R4-2208059

Electronic Meeting, 9th-20th May, 2022

Agenda Item:	9.18.2.2

Source:	Intel Corporation

Title:	Discussion about remaining issues about inter-cell beam management in FeMIMO

Document for:	Discussion

# Introduction

In this meeting, we will provide our view regarding to inter-cell beam management, some open issues are as follows:

- Applicability of simultaneousRxDataSSB-DiffNumerology - Whether to jointly consider the requirement of IBM and inter cell beam management

# Applicability of simultaneousRxDataSSB-DiffNumerology

For UEs which support [simultaneousRxDataSSB-DiffNumerology] [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement. For UEs which do not support [simultaneousRxDataSSB-DiffNumerology] [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement. - The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the SSB indexes configured for L1-RSRP measurement, where the transmission of PUCCH/PUSCH/SRS and reception of PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI may be on serving cell(s) and cell(s) with PCI different from serving cell(s). Editor’s note: FFS simultaneousRxDataSSB-DiffNumerology can be reused for non-serving cell.

In last meeting, it’s still FFS whether simultaneousRxDataSSB-DiffNumerology can be reused for non-serving cell. The definition of simultaneousRxDataSSB-DiffNumerology is defined as below:

Table 1. simultaneousRxDataSSB-DiffNumerology defined in TS 38.306

| simultaneousRxDataSSB-DiffNumerology Indicates whether the UE supports concurrent intra-frequency measurement on serving cell or neighbouring cell and PDCCH or PDSCH reception from the serving cell with a different numerology as defined in clause 8 and 9 of TS 38.133 [5]. | UE | No | No | Yes |
| --- | --- | --- | --- | --- |

It shows that it can be used for intra-frequency neighbouring cell measurement, and it’s used for legacy scheduling restriction for intra-frequency L3 measurement.

Since current non-serving cell L1-RSRP measurement is also intra-frequency measurement for neighboring cell , it is similar with intra-frequency L3 measurement. Therefore, simultaneousRxDataSSB-DiffNumerology can be reused for non-serving cell.

Proposal 1: SimultaneousRxDataSSB-DiffNumerology can be reused for non-serving cell.

Issue 1-2-5 Whether to jointly consider the requirement of IBM and inter cell beam management - Proposal:RAN4 to discuss whether the scheduling restriction requirement of inter-cell beam management should be extend to FR2 inter-band CA with independent beam management. - Option 1: Support. - Option 2: Do not support.

Currently for inter-cell beam measurement, only intra-frequency measurement will be considered. For intra-band carrier aggregation in FR2, there is no IBM issue. For inter-band CA, there are no scheduling restrictions on FR2 serving cells in the bands due to inter-cell L1-RSRP measurement performed on FR2 serving cell(s) in different band(s), provided that UE is capable of independent beam management on this FR2 band pair. Therefore, the legacy scheduling restriction for serving cell L1-RSRP measurement on FR2 can be re-used.

Proposal 2:  The scheduling restriction requirement of inter-cell beam management can be extended to FR2 inter-band CA with independent beam management.

# Conclusion

In this contribution, we provide our views regarding inter-cell beam management in FeMIMO:

Proposal 1: SimultaneousRxDataSSB-DiffNumerology can be reused for non-serving cell.

Proposal 2:  The scheduling restriction requirement of inter-cell beam management can be extended to FR2 inter-band CA with independent beam management.

# Reference

[1] R4-2207081, WF on FeMIMO RRM requirements for inter-cell beam management