3GPP TSG-RAN WG4 Meeting # 103-e	R4-2208063

Electronic Meeting, 9th-20th May, 2022

Agenda Item:	9.9.1.3

Source:	Intel Corporation

Title:	Discussion about remaining issue about PUCCH SCell activation

Document for:	Discussion

# Introduction

According to WF about PUCCH with SCell activation[1], there are limited open issues:

- Whether the PL-RS will introduce extra delay time when the known condition is met - Relation between the associated RS for TCI state, PL-RS and spatial relation indication - The delay requirements for PUCCH SCell activation with multiple DL Scells

In this contribution, we will provide our views regarding these issues.

# Discussion

- Issue 1-2-4: Whether the PL-RS will introduce extra delay time when the known condition is met? Agreements: - When PL-RS of target PUCCH Scell is known, the [X] sample measurement time is introduced. - FFS under what condition the [X] = 0 or [X] = 5.

In last meeting, there are many discussions regarding whether 5 samples are always needed for pathloss calculation. According the comments of company, if PL-RS is measured for other active SCell, it’s still possible that UE may not maintain the PL-RS for the deactive PUCCH SCell. We are fine that 5 samples are always needed.

Proposal 1: When PL-RS of target PUCCH Scell is known, 5  sample measurement time is always needed.

- Issue 1-2-6: Relation between the associated RS for TCI state, PL-RS and spatial relation indication? Agreements: - FFS: No need to have restrictions on the relation between the associated RS for TCI state, PL-RS and spatial relation indication.

In Rel-16, spatial relation indication and PL-RS can be activated independently and there is no limitation about the relation between them. Therefore, we suggest not to have restrictions about the relation between them.

Proposal 2: No need to have restrictions on the relation between the associated RS for TCI state, PL-RS and spatial relation indication.

- Issue 1-4-2: The delay requirements for PUCCH SCell activation with multiple DL Scells? - Define the delay requirements for PUCCH SCell activation with multiple DL Scells taking normal Scell activation with multiple DL Scell as baseline - FFS on the requirements: - the normal SCell activation delay requirement for deactivated SCell with multiple Downlink SCells defined in clause 8.3.7 of current specification 38.133 apply for other downlink Scells. - PUCCH Scell activation delay requirements can be derived from single PUCCH SCell activation delay by replacing Tactivation_time with Tactivation_time_multiple_scells

For PUCCH Scell activation with multiple DL SCells, from our understanding, PUCCH Scell didn’t need to be activated prior to other SCells.  Therefore, the legacy Scell activation delay requirement with multiple DL SCells in clause 8.3.7 can be re-used  for other SCells. While for PUCCH Scell activation, delay requirements can be derived from single PUCCH SCell activation delay by replacing Tactivation_time with  Tactivation_time_multiple_scells.

Proposal 3: The delay requirements for PUCCH SCell activation with multiple DL Scells are:

- the normal SCell activation delay requirement for deactivated SCell with multiple Downlink SCells defined in clause 8.3.7 of current specification 38.133 apply for other downlink Scells.

- PUCCH Scell activation delay requirements can be derived from single PUCCH SCell activation delay by replacing Tactivation_time with  Tactivation_time_multiple_scells

# Conclusion

In this contribution, we provide our views regarding PUCCH SCell activation:

Proposal 1: When PL-RS of target PUCCH Scell is known, 5  sample measurement time is always needed.

Proposal 2: No need to have restrictions on the relation between the associated RS for TCI state, PL-RS and spatial relation indication.

Proposal 3: The delay requirements for PUCCH SCell activation with multiple DL Scells are:

- the normal SCell activation delay requirement for deactivated SCell with multiple Downlink SCells defined in clause 8.3.7 of current specification 38.133 apply for other downlink Scells.

- PUCCH Scell activation delay requirements can be derived from single PUCCH SCell activation delay by replacing Tactivation_time with  Tactivation_time_multiple_scells

# Reference

[1] R4-2206869, WF on further RRM enhancement for NR and MR-DC – PUCCH SCell activation/deactivation requirements