3GPP TSG-RAN WG4 Meeting # 103-e 	                                            R4-228068

Electronic Meeting, May 09 – May 20

Agenda Item:	9.10.2.2.

Source:	Intel Corporation

Title:	Discussion on NR concurrent MG test cases configuration and list

Document for:		Discussion

# Introduction

In the last RAN4 meeting, the RRM core requirements related to the concurrent MGs were completed [1]. Accordingly, RAN4 is required to define the necessary test cases to verify the RRM requirements in TS38.133 [2].

In this paper we firstly provide the considerations on the basic test configurations of test cases for measurements with concurrent MGs. And the list of the test cases to verify the corresponding core requirements is proposed.

# Discussion

- Issue 1. NR standalone test cases only?

In Rel17, the concurrent MG will be configured in NR SA only. That is currently it is enough to define the test cases for NR standalone scenarios.

Proposal 1: It is necessary to define the test cases for NR standalone scenario only.

- Issue 2: which types of reference signal shall be measured by concurrent MGs?

One of important purpose of concurrent MG is to reduce the measurement delay for the dedicated usage of measurements. Thus, multiple types of measurements which are associated with the independent gap instance can be configured (e.g. one is for SSB the other PRS).

Thus, we proposed that:

Proposal 2: Two types of measurement reference signals shall be associated with the different gap instances within the concurrent MG.

For the testing of concurrent measurement gap, the multiple measurement objects with the different measurement gap instance shall be configured by the network.

Proposal 3: General testing configurations for SSB, CSI-RS and PRS in A.3.10	SSB Configurations, A.3.30	CSI-RS configurations for RRM and A.3.31	PRS Configurations can be reused for these test cases of concurrent gaps.

- Issue 3: Gap combination configuration supported

RAN4 defined multiple gap combination which can be supported by UE in Rel17 [2,TS38.133]. In our views, the configuration #2, 3 is more general to verify the concurrent gaps.

| Gap Combination Configuration Id | The number of simultaneous configured measurement gap patterns |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 measurement gap | Per-FR2 measurement gap | Per-UE measurement gap |
| 0 | 2 | 1 | 0 |
| 1 | 1 | 2 | 0 |
| 2 | 0 | 0 | 2 |
| 3Note 1 | 1 | 0 | 1 |
| 4Note 1 | 0 | 1 | 1 |
| 5Note 1 | 1 | 1 | 1 |
| Note 1: Gap Combination Configuration Id #3, #4, #5 will be only applied when the per-UE measurement gap is associated to measure PRS for any RSTD, PRS-RSRP, and UE Rx-Tx time difference measurement defined in TS 38.215 [4]. |  |  |  |

Proposal 4: Gap combination configuration #2, 3 is sufficient to verify the concurrent gaps requirements.

Regarding to the issue 2 and 3 jointly, we can propose that:

Proposal 5: Several typical testing configuration combinations for concurrent MGs can be also defined in A.3 in TS38.133, e.g.

Table A.3.x.y.z-1: Concurrent MG Pattern 1:  SSB SCS=15 KHz

| Parameters | Value |  |
| --- | --- | --- |
| Reference channel | Concurrent MG#1 |  |
| SCS | 15k |  |
| Gap combination configuration | #2 |  |
| Multiple Measurement objects | SSB.1 FR1 | PRS.1.1 FR1 |

- Issue 4: Collision between concurrent gaps

When UE is configured with concurrent measurement gaps, two measurement gap occasions can be  considered as no colliding and colliding if at least one of the following proximity conditions met:

  - X=4 in FR1

  - X=[1 or 4] in FR2

And when they are collided, one of them shall be prioritized.

Proposal 6: Both the non-collision and collision concurrent gaps test cases shall be defined, in which the proper proximity conditions should be configured.

- Issue 5: DRX

As in WID, the measurement gap enhancement is only considered in RRC_CONNECT, we need not any test cases for DRX.

Proposal 7: Non-DRX cases will be tested only in Rel17.

- Issue 6: Gap based measurement

Since there is more direct impacts on measurement delay requirements for the measurement with gap (intra-frequency with gap and inter-frequency with gap), we need define test cases for these cases. However, in order to reduce the overall testing efforts due to concurrent measurement, we suggest:

Proposal 8: RAN4 needs define the test cases below at least.

- inter-frequency with gap

- RLM

- Issue 7. Testing procedure

The test cases for the concurrent measurement gaps can be based on the test cases in Rel16 in which the legacy gaps are configured.

Figure 1. Testing procedure

That is the testing procedure for measurements with concurrent MGs can consist of two successive time periods, with time durations of T1, T2 respectively.

- During the duration of T1, UE can be configured with single measurement gap only

- At the start of time duration T2, the network can configure the other additional measurement gap to UE which is dedicated used for PRS. UE may not have any timing information of neighbor cell to be measured (e.g. cell 2).

- UE can perform the measurements on the measurement object associated with GAP2 within T2.

In summary, the following test cases list for NR concurrent MGs can be:

Proposal 9: The following test cases for core requirement (e.g. reporting delay tests) are listed in Table  1 below.

Table 1 Test cases for concurrent measurement gaps core requirements

| No | Type of Test | Description | Test purpose | Notes |
| --- | --- | --- | --- | --- |
| 1-1 | Inter-freq measurement with gap reporting | TDD/FDD, FR1/FR2 SSB measured by MG#1 PRS measured by MG#2 Gap combination : #2,#3 Collision/non-collision No DRX cycle AWGN | Core requirements in section 9.9.2.4 which is also rely on UE’s processing capability to be verified. UE reports RSTD within required delay for certain number of cells |  |
| 2-1 | RLM | TDD/FDD, FR1/FR2 SSB1 measured by MG#1 CSI-RS measured by MG#2 Gap combination : #2,#3 Collision No DRX cycle AWGN | Core requirements in section 8.1.2.2 which is also rely on UE’s processing capability to be verified. UE reports RSTD within required delay for certain number of cells | For RLM testing, only collision case will be tested. |

# Summary

This paper proposes the main configurations parameters and test cases list to verify the RRM requirements related to NR concurrent gaps.

Proposal 1: It is necessary to define the test cases for NR standalone scenario only.

Proposal 2: Two types of measurement reference signals shall be associated with the different gap instances within the concurrent MGs.

Proposal 3: General testing configurations for SSB, CSI-RS and PRS in TS38.133[2] (e.g.A.3.10	SSB Configurations, A.3.30	CSI-RS configurations for RRM and A.3.31	PRS Configurations) can be reused for these test cases of concurrent gaps.

Proposal 4: Gap combination configuration #2, 3 is sufficient to verify the concurrent gaps requirements.

Proposal 5: Several typical testing configuration combinations for concurrent MGs can be also defined in A.3 in TS38.133, e.g.

Table A.3.x.y.z-1: Concurrent MG Pattern 1:  SSB SCS=15 KHz

| Parameters | Value |  |
| --- | --- | --- |
| Reference channel | Concurrent MG#1 |  |
| SCS | 15k |  |
| Gap combination configuration | #2 |  |
| Multiple Measurement objects | SSB.1 FR1 | PRS.1.1 FR1 |

Proposal 6: Both the non-collision and collision concurrent gaps test cases shall be defined, in which the proper proximity conditions should be configured.

Proposal 7: Non-DRX cases will be tested only in Rel17.

Proposal 8: RAN4 needs define the test cases below at least.

- inter-frequency with gap

- RLM

Proposal 9: The following test cases for core requirement (e.g. reporting delay tests) are listed in Table  1 below.

Table 1 Test cases for concurrent measurement gaps core requirements

| No | Type of Test | Description | Test purpose | Notes |
| --- | --- | --- | --- | --- |
| 1-1 | Inter-freq measurement with gap reporting | TDD/FDD, FR1/FR2 SSB measured by MG#1 PRS measured by MG#2 Gap combination : #2,#3 Collision/non-collision No DRX cycle AWGN | Core requirements in section 9.9.2.4 which is also rely on UE’s processing capability to be verified. UE reports RSTD within required delay for certain number of cells |  |
| 2-1 | RLM | TDD/FDD, FR1/FR2 SSB1 measured by MG#1 CSI-RS measured by MG#2 Gap combination : #2,#3 Collision No DRX cycle AWGN | Core requirements in section 8.1.2.2 which is also rely on UE’s processing capability to be verified. UE reports RSTD within required delay for certain number of cells | For RLM testing, only collision case will be tested. |

# References

- R4-2206874, WF on R17 NR MG enhancements – multiple concurrent MGs

- 3GPP TS 38.133 v17.5.0

# Appendix: [Draft] CR for the multiple measurement objects configuration within concurrent MGs

Start of Change

### A.3.x.y.	Configurations for FR1

Table A.3.x.y.z-1: Concurrent MG Pattern 1:  SSB SCS=15 KHz

| Parameters | Value |  |
| --- | --- | --- |
| Reference channel | Concurrent MG#1 |  |
| SCS | 15k |  |
| Gap combination configuration | #2 |  |
| Multiple Measurement objects | SSB.1 FR1 | PRS.1.1 FR1 |

End of Change