3GPP TSG-RAN WG4 Meeting #103-e              	R4-2208083

Electrical Meeting, 9th May-20th May, 2022

Agenda item:	9.17.2.2

Source:	Samsung

Title:	View on PUCCH demodulation requirement for Rel-17 coverage enhancement

Document for:	Discussion

# 1	Introduction

In the last RAN4 meeting, the test scope of PUCCH for Rel-17 coverage enhancement was under discussed. The related agreement was captured in the WF [1] as

In this contribution, view on the remaining issue of test setup for PUCCH requirement is provided.

# 2	PUCCH enhancements

In this section, the potential PUCCH performance requirement impacts for sub-objective of coverage enhancement are analyzed.

## 2. 1 PUCCH demodulation with Joint channel estimation

In the last meeting, the following open issues are under discussed

Configured TDW length for JCE in BS PUCCH demodulation requirements

The following is the agreement in the last meeting

- For TDD - Option 1: Use the max number cTDW length to be [16] slots - Option 2: cTDW length is configured same as the aTDW length - For FDD - Option 1: 8 slots - Option 2: cTDW length is configured same as the aTDW length

Regarding the Configured TDW, we prefer to apply the same value agreed for PUSCH requirement with JCE.

Proposal 1: Configure the same value for cTDW and aTDW length, with same value agreed for PUSCH requirement with JCE for PUCCH requirement with JCE.

Number of repetitions for BS PUCCH demodulation requirements with JCE

The following is the agreement in the last meeting

- Option 1: 8 for FDD and TDD - Option 2: 2 - Option 3: Same with aTDW length

Similar as PUSCH with joint channel estimation, we prefer 2 consecutive slots for TDD and FDD for PUCCH requirement, which can be comparable with Rel-15 PUCCH with multi-slot transmission with 2 repetitions.

Proposal 2: RAN4 applies 2 consecutive slots for PUCCH requirement with JCE in TDD and FDD.

Other parameters for JCE

The following is the agreement in the last meeting

- DMRS configuration for PUCCH format3 - Option 1: only with or without additional DMRS Option 3: Cover both with and without additional DMRS

PUCCH format 3 can support both with and without additional DMRS configuration. For Rel-15, PUCCH requirement with two configurations were specified.  In general, in our understanding, PUCCH is not the bottleneck of uplink, considering the payload is small with low modulation order. With additional DMRS configuration, the channel estimation performance is not the limit factor of PUCCH performance.

The purpose of JCE is to improve the channel estimation performance, while considering the multiple repetition gain, the PUCCH performance improvement due to JCE is minor. Therefore, we prefer to not configure additional DMRS for PUCCH with JCE

Proposal 3: RAN4 applies 2 consecutive slots for PUCCH requirement with JCE in TDD and FDD.

Frequency hopping

The following is the agreement in the last meeting

- In case of BS PUCCH demodulation requirements with JCE is introduced, agree to disable PUCCH intra-slot frequency hopping - Inter-slot frequency hopping - Option 1: Disabled - Option 2: Activate inter-slot frequency hopping, with inter-slot hopping length 2 for TDD and 4 for FDD

With inter-slot frequency hopping, the power consistency and phase continuously will be impacted. It is not feasible to apply DMRS bundling for joint channel estimation with small number of slots. Therefore, we prefer to disable inter-slot frequency hopping for PUCCH requirement with JCE.

Proposal 4: Disable intra-slot and inter-slot frequency hopping for PUCCH requirement with JCE.

# 3	Conclusion

In this contribution, the overview impact on PUCCH performance requirement for Rel-17 coverage enhancement WI is provided.

Proposal 1: Configure the same value for cTDW and aTDW length, with same value agreed for PUSCH requirement with JCE for PUCCH requirement with JCE.

Proposal 2: RAN4 applies 2 consecutive slots for PUCCH requirement with JCE in TDD and FDD.

Proposal 3: RAN4 applies 2 consecutive slots for PUCCH requirement with JCE in TDD and FDD.

Proposal 4: Disable intra-slot and inter-slot frequency hopping for PUCCH requirement with JCE.

# References

R4-2207211, WF on PUCCH demodulation requirement, Nokia