3GPP TSG-RAN WG4 Meeting # 103-e	   R4-2208142

Electronic Meeting, May 09 – May 20, 2022

Title: 	Discussion of FR2 repeater declaration

Source: 	CATT

Agenda item:	9.5.5.3.3

Document for:	Approval

- Introduction

When repeater is tested, the declarations should be provided. This contribution provides the analysis for the FR2 repeater declarations.

- Discussion

Similar with our contribution for FR1, taking BS declaration as the starting point, the following are the repeater modifications compared with BS type 2-O declarations. One issue may be that if the declaration identifier number for FR1 and FR2 are numbered as whole. Our understanding is that they can be defined separately as the specifications are separated. The following modifications are based on that understanding.

Table 4.6-1 Manufacturers declarations for repeater type 2-O radiated test requirements

| Declaration identifier | Declaration | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Downlink | Uplink | Common fo DL and UL |
| D.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the repeater enclosure. | n/a | n/a | x |
| D.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the repeaterenclosure. | n/a | n/a | x |
| D.3 | Beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. The vendor may declare any number of beams with unique identifiers. The minimum set to declare for conformance, corresponds to the beams at the reference beam direction with the highest intended EIRP, and covering the properties listed below: 1) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ possible when narrowest intended BeWθ is used. 2) A beam with the narrowest intended BeWϕ and narrowest intended BeWθ possible when narrowest intended BeWϕ is used. 3) A beam with the widest intended BeWθ and widest intended BeWϕ possible when widest intended BeWθ is used. 4) A beam with the widest intended BeWϕ and widest intended BeWθ possible when widest intended BeWϕ is used. 5) A beam which provides the highest intended EIRP of all possible beams. When selecting the above five beam widths for declaration, all beams that the BS is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. (Note 3) | x | x | n/a |
| D.4 | Operating bands and frequency ranges | List of NR operating band(s) supported by the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in. Supported bands declared for every beam (D.3). (Note 4) |  |  | x |
| D.5 | Reapter class | Declared as Wide Area BS, Medium Range BS, or Local Area BS. | x | x | n/a |
| D.6 | OTA peak directions set reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam (D.3). | x | x | n/a |
| D.7 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam (D.3). | x | x | n/a |
| D.8 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). | x | x | n/a |
| D.9 | Beamwidth | The beamwidth for the reference beam direction pair and the four maximum steering directions. Declared for every beam (D.3). | x | x | n/a |
| D.10 | Equivalent beams | List of beams which are declared to be equivalent. Equivalent beams imply that the beams are expected to have identical OTA peak directions sets and intended to have identical spatial properties at all steering directions within the OTA peak directions set when presented with identical signals. All declarations (D.4 – D.12) made for the beams are identical and the transmitter unit, RDN and antenna array responsible for generating the beam are of identical design. | x | x | n/a |
| D.11 | Parallel beams | List of beams which have been declared equivalent (D.13) and can be generated in parallel using independent RF power resources. Independent power resources mean that the beams are transmitted from mutually exclusive transmitter units. | x | x | n/a |
| D.12 | Operating bands with multi-band dependencies | List of operating bands which are generated using transceiver units supporting operation in multiple operating bands through common active RF components. Declared for each operating band for which multi-band transceiver is used. | n/a | n/a | x |
| D.13 | Maximum radiated repeater passband RF Bandwidth | Maximum repeater passband RF Bandwidth in the operating band, declared for each supported operating band (D.4). (Note 15) | n/a | n/a | x |
| D.14 | Maximum passband Radio Bandwidth of the operating band with multi-band dependencies | Largest Radio Bandwidth that can be supported by the operating bands with multi-band dependencies. Declared for each supported operating band which has multi-band dependencies (D.16). | n/a | n/a | x |
| D.15 | Contiguous or non-contiguous spectrum operation support | Ability of repeater to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi passband in an operating band. | x | x | n/a |
| D.16 | OTA coverage range | Declared as a single range of directions within which selected output OTA requirements are intended to be met. (Note 10) | x | x | n/a |
| D.17 | OTA coverage range reference direction | The direction describing the reference direction of the OTA converge range (D.34). (Note 11) | x | x | n/a |
| D.18 | OTA coverage range maximum directions | The directions corresponding to the following points: 1) The direction determined by the maximum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. | x | x | n/a |
| D.19 | Rated transmitter TRP, Prated,t,TRP | Rated total radiated output power. Declared per supported operating band. (Note 12,14, 18) | x | x | n/a |
| D.20 | Spurious emission category | Declare the repeater spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [5]. | n/a | n/a | x |
| D.21 | Additional operating band unwanted emissions | The manufacturer shall declare whether the repeater under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.7.4 apply. (Note 16, Note 19) | n/a | n/a | x |
| D.22 | Co-existence with other systems | The manufacturer shall declare whether the repeater under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA, PHS and/or NR operating in another operating band are deployed. | n/a | n/a | x |
| D.23 | Co-location with other base stations/IABs | The manufacturer shall declare whether the BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or NR operating in another operating band. | n/a | n/a | x |
| D.24 | Single-band RIB or multi-band RIB | List of single-band RIB and/or multi-band RIB for the supported operating bands (D.4). | n/a | n/a | x |
| D.25 | OTA input RoAoA | Range of angles of arrival associated with the OTA output power. | x | x | n/a |
| D.26 | OTA input receiver target reference direction | Reference direction inside the OTA input RoAoA (D.53). | x | x | n/a |
| D.27 | OTA input conformance test directions | The following four OTA input conformance test directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the OTA input RoAoA, while θ value being the closest possible to the OTA input receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA input RoAoA, while θ value being the closest possible to the OTA input receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA input RoAoA, while φ value being the closest possible to the OTA input receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA input RoAoA, while φ value being the closest possible to the OTA input receiver target reference direction. | x | x | n/a |
| D.28 | Supported frequency range of the NR operating band | List of supported frequency ranges representing fractional bandwidths (FBW) of operating bands with FBW larger than 6%. | n/a | n/a | x |
| D.29 | Rated beam EIRP at lower end of the fractional bandwidth (Prated,FBWlow) | The rated EIRP level per passband at lower frequency range of the fractional bandwidth (Prated,FBWlow), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.8). Declared per beam for all supported frequency ranges (D.56). (Note 12, 13, 14, 15, 18) | x | x | n/a |
| D.30 | Rated beam EIRP at higher frequency range of the fractional bandwidth (Prated,c,FBWhigh) | The rated EIRP level per carrier at higher frequency range of the fractional bandwidth (Prated,c,FBWhigh), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.8). Declared per beam for all supported frequency ranges in (D.56). (Note 12, 13, 14 ,15, 18) | x | x | n/a |
| D.31 | Relation between supported maximum RF bandwidth, number of carriers and Rated maximum TRP | If the rated transmitter TRP and total number of supported carriers are not simultaneously supported, the manufacturer shall declare the following additional parameters: - The reduced number of supported carriers at the rated transmitter TRP; - The reduced total output power at the maximum number of supported carriers. | x | x | n/a |
|  |  | - |  |  |  |
| D.32 | Intra-band contiguous CA | Declaration of operating band(s) supporting intra-band contiguous CA. Declared per operating band with CA support. | x | x | n/a |
| D.33 | Intra-band non-contiguous CA | Declaration of operating band(s) supporting intra-band non-contiguous CA. Declared per operating band with CA support. | x | x | n/a |
| D.34 | Total maximum number of supported carriers in multi-band operation | Maximum number of supported carriers for all supported operating bands declared to have multi-band dependencies (D.16). | x | x |  |

- Conclusion

This contribution provides our consideration of the repeater declarations. The modification compared with BS is provided in the contribution.

Reference

[1] RP-212129, “Revised WID on NR Repeaters”, Qualcomm, RAN#93e

[2] TS 38.106 v1.0.0

[3] TS 38.141-2

[4] TS 38.143