3GPP TSG-RAN WG4 Meeting # 103-e		R4-2208153

# Electronic Meeting, May 09 – May 20, 2022

Agenda item:	9.8.2.1

Source: 	CATT

Title: 	Discussion on remaining issues for FR2 HST

Document for:	Discussion

Introduction

In RAN4#102-e meeting, the core requirements of Rel-17 NR FR2 HST are completed. There is only one open issue of capability for one shot large UL timing adjustment listed in WF [1]. In this contribution, we discussed the remaining issues for FR2 HST and provide our proposals.

Discussion

In RAN4#102-e meeting, there is only one open issue of capability for one shot large UL timing adjustment listed in [1] as below:

Issue 1-2-2: Capability for one shot large UL timing adjustment

Agreement: - Introduce feature group x-2 “Support of one shot large UL timing adjustment” with prerequisite feature group (x-1, “Support of FR2 HST operation”) Way forward: Companies are encouraged to discuss further the following two options: - Option 1: Define feature as mandatory with capability signaling. - Option 2: Define feature as optional with capability signaling.

In our view, firstly, it is up to network to enable one shot large uplink timing adjustment. If one shot large uplink timing adjustment is disabled, existing uplink timing adjustment, RA based mechanism can also be used. Therefore, optional capability signal can be defined.

Proposal 1: For capability for one shot large UL timing adjustment, define feature as optional with capability signaling.

For the applicability of all requirements for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17 should be applied only for PCell such as RLM/BFD (not for PScell for RLM/BFD, and Scell for BFD), and cell identification requirements (not applied for SCell) and so on.

Proposal 2: Add clear clarification for the applicability of all requirements for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17 should be applied only for PCell such as RLM/BFD (not for PScell for RLM/BFD, and Scell for BFD), and cell identification requirements (not applied for SCell) and so on.

Conclusion

In this contribution, we provide our consideration of remaining issues of FR2 HST core requirements:

Proposal 1: For capability for one shot large UL timing adjustment, define feature as optional with capability signaling.

Proposal 2: Add clear clarification for the applicability of all requirements for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17 should be applied only for PCell such as RLM/BFD (not for PScell for RLM/BFD, and Scell for BFD), and cell identification requirements (not applied for SCell) and so on.

Reference

[1] R4-2206848, WF on FR2 HST RRM (part 1), Nokia, Nokia Shanghai Bell