3GPP TSG-RAN WG4 Meeting # 103-e			                                   R4-2208359

Electronic Meeting, May 09 – May 20, 2022

Agenda item:	9.12.6.3

Source: 	OPPO

Title: 	Discussion on mobility requirements for NTN

Document for:	Discussion

Introduction

This paper will give our views on mobility requirements for NTN, including:

- Measurement with paging reception if enhanced RRM requirements like HST are used

- GNSS margin for location-assisted cell (re)selection

Discussion

2.1 Measurement with paging reception

#### Agreement: - Scaling factor M1 and M2 on measurement relaxation with paging shall be updated in NTN. - M1=[2.5] if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second, for serving cell measurement, upon more than one SMTC. - M2= [2] if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ [0.64] second, for intra-frequency [and inter-frequency cell measurement], upon more than one SMTC. - The enhanced cell reselection delay requirements (Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra) defined for FR1 HST can be applied to NTN scenario. - The above is subject to NW indication and UE capability.

To protect PO when multiple SMTCs are configured, scaling factor M1 and M2 are further increased. On the other hand, it was also agreed that the enhanced cell reselection delay requirements defined for FR1 HST can apply to NTN subjecting to NW signalling and UE capability. As shown by the tables below, scaling factors M2/M3/M4 with different SMTC periodicity conditions are used when enhanced RRM flag (e.g. highSpeedMeasFlag-r16) is configured. Similar extension for scaling factors M2/M3/M4 should be considered for NTN. For example, when SMTC periodicity >20ms and DRX cycle < 0.64s, M2=2, M3=M4=2.5, upon more than one SMTC.

Proposal 1: For UE configured with enhanced RRM flag (similar as highSpeedMeasFlag-r16), M2=2, M3=M4=2.5 when SMTC periodicity > 40ms and DRX cycle < 0.64s.

Table 4.2.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
| 0.64 |  | 5 | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1 or 5, N1 = 8 for all DRX cycle length. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. |  |  |  |  |  |

Table 4.2.2.3-2: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE configured with highSpeedMeasFlag-r16 (Frequency range FR1)

| DRX cycle length [s] | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.56 x M2 (8 x M2) | 0.32 x M3 (1 x M3) | 0.96 x M4 (3 x M4) |
| 0.64 | 5.12 (8) | 0.64 (1) | 1.92 (3) |
| 1.28 | 8.96 (7) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. |  |  |  |

2.2 Location-assisted cell (re)selection

Agreement: - UE initiates the measurement for cell-reselection in IDLE/Inactive mode if the distance between UE and serving cell reference location is longer than a ‘network-configured threshold + Dmargin’, where Dmargin is [X] meters. - (Note) A value of X will be determined in performance requirement development phase.

For location-assisted cell (re)selection, the margin for UE location via GNSS measurement needs to be determined. Generally, 50ms GNSS positioning margin is assumed for UL timing requirements and can also be used as Dmargin for cell (re)selection.

Proposal 2: Dmargin = 50m is assumed for location-assisted cell (re)selection.

Conclusion

In this contribution, we provide our considerations on the mobility requirements for NTN and the following proposals.

Proposal 1: For UE configured with enhanced RRM flag (similar as highSpeedMeasFlag-r16), M2=2, M3=M4=2.5 when SMTC periodicity > 40ms and DRX cycle < 0.64s.

Proposal 2: Dmargin = 50m is assumed for location-assisted cell (re)selection.

- References

- R4-2206899, WF on NR NTN RRM requirements, Qualcomm Incorporated