3GPP TSG-RAN WG4 Meeting # 103-e											R4-2208366

Electronic Meeting, May 09 – May 20, 2022

Source: 	OPPO

Title: 	Discussion on mobility requirements on RedCap UE

Agenda Item:	9.19.3.1.2

Document for:	Discussion

# 1 Introduction

In last RAN4 meeting, a way forward on RRM requirements for RedCap UE was agreed in [1]. In this contribution, we provide our further consideration on the remaining open issues of mobility requirements for RedCap UE.

- Discussion

Impact on Tsearch (in HO) for 1Rx for FR1 - Option 2 (HW, QC): - For HO to FR1, Tsearch = 3*Trs for intra-frequency HO and Tsearch = 5* Trs for inter-frequency HO - Compromise option from moderator (E///, Apple, MTK, Nokia, vivo): - For HO to FR1, - Tsearch = 2*Trs for intra-frequency HO - Tsearch = 5* Trs for inter-frequency HO HO to target cell when SSB used for measurement and SSB associated with RACH BWP is different - Option 1 (E///): The UE is not expected to perform HO to the target cell when: - It has done measurement in target cell on SSB which is different than the SSB associated with BWP where RACH is configured and if the separation between initial BWP and Redcap BWPs is larger than 20 MHz for FR1 and 100 MHz for FR2 HO to a RedCap specific BWP with NCD-SSB (no CD-SSB) - Option 1 (E///): Handover directly to a RedCap specific BWP where only NCD-SSB is transmitted is supported when: - the difference of center frequency between NCD-SSB of target BWP and CD-SSB of initial BWP of target cell is not larger than 20MHz in FR1 and 100MHz in FR2, - the difference of reception power between NCD-SSB and CD-SSB is less than 3dB, - the periodicity of NCD-SSB and CD-SSB is the same Requirements for handover to RedCap specific BWP with NCD-SSB (no CD-SSB) - Option 1 (E///): Legacy NR handover requirements can be reused for handover to a target RedCap specific BWP where NCD-SSB is transmitted when the periodicity of NCD-SSB and CD-SSB is same. Handover to RedCap specific BWP with no SSB - Option 1 (E///): When there is no SSB transmission in a target RedCap specific BWP, handover to that RedCap specific BWP is supported when: - target RedCap specific BWP and initial BWP of target cell are not separated by larger than 20MHz in FR1 and 100MHz in FR2. - Option 2 (Apple): When there is no SSB transmission in a target RedCap specific BWP, handover to that RedCap specific BWP is supported but no requirement applies. - Option 3 (MTK): When there is no SSB transmission in a target RedCap specific BWP, handover to that RedCap then the UE shall switch back to CD-SSB. - New Option 4 (Qualcomm): When the Redcap specific initial DL BWP is configured for RA only, and there is no SSB available in the redcap specific initial BWP, define the interruption time as - Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin + TBWP-switching-delay ms - Where, TBWP-switching-delay is the BWP switching delay

As shown in the legacy requirements, if the target cell is known, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = Trs ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 3* Trs ms.

For Tsearch (in HO) for 1Rx for FR1, most companies can agree Tsearch = 5* Trs for inter-frequency HO when the target cell is unknown and Es/Iot≥-2 dB. For intra-frequency HO, we can compromise to 2 Trs when the target cell is unknown and Es/Iot≥-2 dB.

Proposal 1: For Tsearch (in HO) for 1Rx for FR1, we propose when the target cell is unknown and Es/Iot≥-2 dB, Tsearch = 2*Trs for intra-frequency HO and Tsearch = 5* Trs for inter-frequency HO.

Whether the handover to RedCap specific BWP with and without NCD-SSB is supported in release 17 RedCap has been discussed. As agreed in RAN2 that “NCD-SSB should not be indicated in the handover command, i.e., network sets ServingCellConfigCommon => downlinkConfigCommon => frequencyInfoDL => absoluteFrequencySSB to the frequency of the CD-SSB (not the NCD-SSB)”, only CD-SSB shall be indicated in the handover command. Hence no need to consider the case of using NCD-SSB.

For the issue when UE handover to RedCap specific BWP with no SSB, UE needs to switch to CD-SSB and measure CD-SSB in the non-Redcap initial BWP. UE handover to that RedCap specific BWP is supported but no requirement applies.

Proposal 2: Only CD-SSB shall be indicated in the handover command and no need to consider the case of using NCD-SSB

Proposal 3: UE handover to RedCap specific BWP is supported but no requirement applies.

- Conclusion

Proposal 1: For Tsearch (in HO) for 1Rx for FR1, we propose when the target cell is unknown and Es/Iot≥-2 dB, Tsearch = 2*Trs for intra-frequency HO and Tsearch = 5* Trs for inter-frequency HO.

Proposal 2: Only CD-SSB shall be indicated in the handover command and no need to consider the case of using NCD-SSB

Proposal 3: UE handover to RedCap specific BWP is supported but no requirement applies.

- Reference

[1] R4-2206950, WF on RedCap RRM requirements, Ericsson