3GPP TSG-RAN WG4 Meeting #103-e													R4-2208479

Electronic meeting, May 9 – 20, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	System parameters for a NR band in the range 52.6GHz – 71GHz

Agenda item:	9.15.2

Document for:	Approval

# Introduction

Building on top of the earlier agreements, this contribution discusses open issues related to systems parameters for a NR band in the range 52.6GHz – 71GHz.

# Discussion

#### Synchronization raster and channel raster

The current WID for extending current NR operation to 71 GHz in [1] states the following on synchronization raster:

  - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints:

    - Limited sync raster entry numbers

      - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n257 is 599). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz.

Agreement: Use the fixed RF channel raster with the step size of 1680 (100.8 MHz) as baseline to define the channel raster for the unlicensed band, and accordingly provide the channel raster numbers

- To Check if the above solution can support CA with different bandwidth combinations

  - If there is issue identified, then the above agreement can be revisited

Agreement: Specify SSB SCS 960KHz in the GSCN and add a note to clarify that 960KHz SCS cannot be used for initial access.

Agreement: Use the fixed RF channel raster with the step size of 1680 (100.8 MHz) as baseline to define the channel raster for the unlicensed band, and accordingly provide the channel raster numbers

- To Check if the above solution can support CA with different bandwidth combinations

  - If there is issue identified, then the above agreement can be revisited

Agreement: Specify SSB SCS 960KHz in the GSCN and add a note to clarify that 960KHz SCS cannot be used for initial access.

In RAN4#101bis-e RAN4 agreed to define fixed synchronization raster for unlicensed operating band n263 and floating synchronization raster for licensed operation. Furthermore in RAN4#102-e the following agreements were reached for unlicensed operation in [2]

In principle no limitations for RF channel raster are required as UE will be told where to find the channel. However, there was a strong desire to limit the number of possible RF raster points. Therefore, we have analyzed how channel placements would work when using the 100.8 MHz step size in the raster and allowing to use only the raster points based on 100 MHz channel bandwidth. This is illustrated in Figure 1.

Figure 1: Channel placement according to 100.8 MHz raster step size for 100 MHz ChBWs

It can be seen in Figure 1 that if channel placement is limited to raster points defined for 100 MHz channel bandwidth, carrier aggregation between 100 MHz carriers and other channel bandwidths will not work. In addition, roughly 50 MHz spectrum will be wasted at the band edge for channel bandwidths wider than 100 MHz.

It should be also considered if the wider than 100 MHz ChBWs are allowed to use all raster points, or if the raster points are limited to being 4, 8, 16 or 20 times 100.8 MHz apart from each other. If such limitation is applied, almost 2 GHz of spectrum will be wasted for 2000 MHz ChBW, as only 6 channels will fit into 14 GHz of available spectrum.

Observation 1: Raster based purely on 100.8 MHz step size based on 100 MHz ChBW does not enable carrier aggregation between 100 MHz and other channel bandwidths and wastes spectrum.

An alternative approach would be to define additional raster points which enable placing all channel bandwidths near band edge. This is illustrated in Figure 2:

Figure 2: Raster points added to enable placing channels near band edge.

With the approach in Figure 2, additional raster points would need to be defined for 400, 800 and 1600 MHz ChBWs, while 2000 MHz channel center points end up always overlapping with 400 MHz channel center points. This approach also enables CA between 100 MHz and other channel bandwidths for some 100 MHz channel positions, while in general case wider channel bandwidth cannot always be placed next to 100 MHz carrier. Furthermore, if the placement of the wider carriers is restricted, the issue of wasting spectrum remains.

Observation 2: Adding additional raster points for wider channels to allow placing channel next to band edge does not resolve the issues for CA and spectrum waste.

One solution to enable flexible CA operation as well as keep the number of RF raster points limited would be to keep the 100.8 MHz raster step size for 100 MHz ChBW and to specify 50.4 MHz raster step size for wider channels without restricting the channel placement. This would enable that for any 100 MHz channel placement it is also possible to place wider channel next to allowing flexible usage of contiguous carrier aggregation. This is shown in Figure 3.

Figure 3: Free channel placement in 50.4 MHz raster for wider than 100 MHz channels while 100 MHz channels kept at 100.8 MHz raster.

Furthermore, the free placement of channels means that also wider channels could be placed close to both band edges, and as such the spectrum usage would in general be much better. The total number of unique raster points with this design would be 276, while the number of sync raster points could still be based on 100 MHz channel bandwidth and be limited to 138 for 120 kHz SCS. Table 1 shows the synchronization raster points for the fixed channelization.

It has been confirmed that the coreset#0 offsets required for RAN1 specification are feasible with these synchronization raster points. The details can be found in [3].

Proposal 1: Adopt synchronization raster points for n263 as shown in Table 1 of the Annex

Proposal 2: RF channel raster is fixed based on 100.8 MHz step size for 100 MHz ChBW and 50.4 MHz step size for wider channel bandwidths, network implementation will take care that SSB and coreset#0 fit within channel bandwidth.

# 3 Conclusion

In this contribution channel arrangement and spectrum utilization for NR operation above 52.6 GHz was discussed. The following proposals were made:

Proposal 1: Adopt synchronization raster points shown in Table 2 for n263

Proposal 2: RF channel raster is fixed based on 100.8 MHz step size for 100 MHz ChBW and 50.4 MHz step size for wider channel bandwidths, network implementation will take care that SSB and coreset#0 fit within channel bandwidth.

# References

[1] RP-213540, “Revised WID: Extending current NR operation to 71 GHz”, Qualcomm, Intel

[2] R4-2206535, “WF on system parameters of FR2-2”, Intel Corporation

[3] R1-2204599, “Initial access aspects”, Nokia, Nokia Shanghai Bell

# Annex

Table 1 GSCNs in fixed channelization

| SCS | 120 kHz |  |  | 480 kHz |  |  |  | 960 kHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth | 100 MHz | 400 MHz |  | 400 MHz | 800 MHz | 1600 MHz |  | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
|  | 24153 | 24153 |  | 24157 | 24157 | 24157 |  | 24160 | 24160 | 24166 | 24166 |
|  | 24159 |  |  | 24181 |  |  |  | 24184 |  |  |  |
|  | 24165 |  |  | 24205 | 24205 |  |  | 24208 | 24208 |  |  |
|  | 24171 |  |  | 24229 |  |  |  | 24232 |  |  |  |
|  | 24177 | 24177 |  | 24253 | 24253 | 24253 |  | 24256 | 24256 | 24256 |  |
|  | 24183 |  |  | 24277 |  |  |  | 24280 |  |  | 24280 |
|  | 24189 |  |  | 24301 | 24301 |  |  | 24304 | 24304 |  |  |
|  | 24195 |  |  | 24325 |  |  |  | 24322 |  |  |  |
|  | 24201 | 24201 |  | 24349 | 24349 | 24349 |  | 24346 | 24346 | 24352 |  |
|  | 24207 |  |  | 24373 |  |  |  | 24370 |  |  |  |
|  | 24213 |  |  | 24397 | 24397 |  |  | 24394 | 24394 |  | 24400 |
|  | 24219 |  |  | 24421 |  |  |  | 24418 |  |  |  |
|  | 24225 | 24225 |  | 24445 | 24445 | 24445 |  | 24442 | 24442 | 24442 |  |
|  | 24231 |  |  | 24469 |  |  |  | 24466 |  |  |  |
|  | 24234 |  |  | 24493 | 24493 |  |  | 24490 | 24490 |  |  |
|  | 24240 |  |  | 24517 |  |  |  | 24514 |  |  | 24514 |
|  | 24246 | 24246 |  | 24529 | 24541 | 24541 |  | 24532 | 24538 | 24538 |  |
|  | 24252 |  |  | 24553 |  |  |  | 24556 |  |  |  |
|  | 24258 |  |  | 24577 | 24577 |  |  | 24580 | 24580 |  |  |
|  | 24264 |  |  | 24601 |  |  |  | 24604 |  |  |  |
|  | 24270 | 24270 |  | 24625 | 24625 | 24625 |  | 24628 | 24628 | 24628 | 24628 |
|  | 24276 |  |  | 24649 |  |  |  | 24652 |  |  |  |
|  | 24282 |  |  | 24673 | 24673 |  |  | 24676 | 24676 |  |  |
|  | 24288 |  |  | 24697 |  |  |  | 24700 |  |  |  |
|  | 24294 | 24294 |  | 24721 | 24721 | 24721 |  | 24724 | 24724 | 24724 |  |
|  | 24300 |  |  | 24745 |  |  |  | 24742 |  |  | 24748 |
|  | 24306 |  |  | 24769 | 24769 |  |  | 24766 | 24766 |  |  |
|  | 24312 |  |  | 24793 |  |  |  | 24790 |  |  |  |
|  | 24318 | 24318 |  | 24817 | 24817 | 24817 |  | 24814 | 24814 | 24814 |  |
|  | 24324 |  |  | 24841 |  |  |  | 24838 |  |  |  |
|  | 24330 |  |  | 24865 | 24865 |  |  | 24862 | 24862 |  |  |
|  | 24336 |  |  | 24889 |  |  |  | 24886 |  |  |  |
|  | 24339 | 24342 |  | 24913 | 24913 |  |  | 24910 | 24910 |  |  |
|  | 24345 |  |  | 24937 |  |  |  | 24934 |  |  |  |
|  | 24351 |  |  |  |  |  |  |  |  |  |  |
|  | 24357 |  |  |  |  |  |  |  |  |  |  |
|  | 24363 | 24363 |  |  |  |  |  |  |  |  |  |
|  | 24369 |  |  |  |  |  |  |  |  |  |  |
|  | 24375 |  |  |  |  |  |  |  |  |  |  |
|  | 24381 |  |  |  |  |  |  |  |  |  |  |
|  | 24387 | 24387 |  |  |  |  |  |  |  |  |  |
|  | 24393 |  |  |  |  |  |  |  |  |  |  |
|  | 24399 |  |  |  |  |  |  |  |  |  |  |
|  | 24405 |  |  |  |  |  |  |  |  |  |  |
|  | 24411 | 24411 |  |  |  |  |  |  |  |  |  |
|  | 24417 |  |  |  |  |  |  |  |  |  |  |
|  | 24423 |  |  |  |  |  |  |  |  |  |  |
|  | 24429 |  |  |  |  |  |  |  |  |  |  |
|  | 24435 | 24435 |  |  |  |  |  |  |  |  |  |
|  | 24441 |  |  |  |  |  |  |  |  |  |  |
|  | 24444 |  |  |  |  |  |  |  |  |  |  |
|  | 24450 |  |  |  |  |  |  |  |  |  |  |
|  | 24456 | 24456 |  |  |  |  |  |  |  |  |  |
|  | 24462 |  |  |  |  |  |  |  |  |  |  |
|  | 24468 |  |  |  |  |  |  |  |  |  |  |
|  | 24474 |  |  |  |  |  |  |  |  |  |  |
|  | 24480 | 24480 |  |  |  |  |  |  |  |  |  |
|  | 24486 |  |  |  |  |  |  |  |  |  |  |
|  | 24492 |  |  |  |  |  |  |  |  |  |  |
|  | 24498 |  |  |  |  |  |  |  |  |  |  |
|  | 24504 | 24504 |  |  |  |  |  |  |  |  |  |
|  | 24510 |  |  |  |  |  |  |  |  |  |  |
|  | 24516 |  |  |  |  |  |  |  |  |  |  |
|  | 24522 |  |  |  |  |  |  |  |  |  |  |
|  | 24528 | 24528 |  |  |  |  |  |  |  |  |  |
|  | 24534 |  |  |  |  |  |  |  |  |  |  |
|  | 24540 |  |  |  |  |  |  |  |  |  |  |
|  | 24546 |  |  |  |  |  |  |  |  |  |  |
|  | 24549 | 24552 |  |  |  |  |  |  |  |  |  |
|  | 24555 |  |  |  |  |  |  |  |  |  |  |
|  | 24561 |  |  |  |  |  |  |  |  |  |  |
|  | 24567 |  |  |  |  |  |  |  |  |  |  |
|  | 24573 | 24573 |  |  |  |  |  |  |  |  |  |
|  | 24579 |  |  |  |  |  |  |  |  |  |  |
|  | 24585 |  |  |  |  |  |  |  |  |  |  |
|  | 24591 |  |  |  |  |  |  |  |  |  |  |
|  | 24597 | 24597 |  |  |  |  |  |  |  |  |  |
|  | 24603 |  |  |  |  |  |  |  |  |  |  |
|  | 24609 |  |  |  |  |  |  |  |  |  |  |
|  | 24615 |  |  |  |  |  |  |  |  |  |  |
|  | 24621 | 24621 |  |  |  |  |  |  |  |  |  |
|  | 24627 |  |  |  |  |  |  |  |  |  |  |
|  | 24633 |  |  |  |  |  |  |  |  |  |  |
|  | 24639 |  |  |  |  |  |  |  |  |  |  |
|  | 24645 | 24645 |  |  |  |  |  |  |  |  |  |
|  | 24651 |  |  |  |  |  |  |  |  |  |  |
|  | 24654 |  |  |  |  |  |  |  |  |  |  |
|  | 24660 |  |  |  |  |  |  |  |  |  |  |
|  | 24666 | 24666 |  |  |  |  |  |  |  |  |  |
|  | 24672 |  |  |  |  |  |  |  |  |  |  |
|  | 24678 |  |  |  |  |  |  |  |  |  |  |
|  | 24684 |  |  |  |  |  |  |  |  |  |  |
|  | 24690 | 24690 |  |  |  |  |  |  |  |  |  |
|  | 24696 |  |  |  |  |  |  |  |  |  |  |
|  | 24702 |  |  |  |  |  |  |  |  |  |  |
|  | 24708 |  |  |  |  |  |  |  |  |  |  |
|  | 24714 | 24714 |  |  |  |  |  |  |  |  |  |
|  | 24720 |  |  |  |  |  |  |  |  |  |  |
|  | 24726 |  |  |  |  |  |  |  |  |  |  |
|  | 24732 |  |  |  |  |  |  |  |  |  |  |
|  | 24738 | 24738 |  |  |  |  |  |  |  |  |  |
|  | 24744 |  |  |  |  |  |  |  |  |  |  |
|  | 24750 |  |  |  |  |  |  |  |  |  |  |
|  | 24756 |  |  |  |  |  |  |  |  |  |  |
|  | 24759 | 24762 |  |  |  |  |  |  |  |  |  |
|  | 24765 |  |  |  |  |  |  |  |  |  |  |
|  | 24771 |  |  |  |  |  |  |  |  |  |  |
|  | 24777 |  |  |  |  |  |  |  |  |  |  |
|  | 24783 | 24783 |  |  |  |  |  |  |  |  |  |
|  | 24789 |  |  |  |  |  |  |  |  |  |  |
|  | 24795 |  |  |  |  |  |  |  |  |  |  |
|  | 24801 |  |  |  |  |  |  |  |  |  |  |
|  | 24807 | 24807 |  |  |  |  |  |  |  |  |  |
|  | 24813 |  |  |  |  |  |  |  |  |  |  |
|  | 24819 |  |  |  |  |  |  |  |  |  |  |
|  | 24825 |  |  |  |  |  |  |  |  |  |  |
|  | 24831 | 24831 |  |  |  |  |  |  |  |  |  |
|  | 24837 |  |  |  |  |  |  |  |  |  |  |
|  | 24843 |  |  |  |  |  |  |  |  |  |  |
|  | 24849 |  |  |  |  |  |  |  |  |  |  |
|  | 24855 | 24855 |  |  |  |  |  |  |  |  |  |
|  | 24861 |  |  |  |  |  |  |  |  |  |  |
|  | 24864 |  |  |  |  |  |  |  |  |  |  |
|  | 24870 |  |  |  |  |  |  |  |  |  |  |
|  | 24876 | 24876 |  |  |  |  |  |  |  |  |  |
|  | 24882 |  |  |  |  |  |  |  |  |  |  |
|  | 24888 |  |  |  |  |  |  |  |  |  |  |
|  | 24894 |  |  |  |  |  |  |  |  |  |  |
|  | 24900 | 24900 |  |  |  |  |  |  |  |  |  |
|  | 24906 |  |  |  |  |  |  |  |  |  |  |
|  | 24912 |  |  |  |  |  |  |  |  |  |  |
|  | 24918 |  |  |  |  |  |  |  |  |  |  |
|  | 24924 | 24924 |  |  |  |  |  |  |  |  |  |
|  | 24930 |  |  |  |  |  |  |  |  |  |  |
|  | 24936 |  |  |  |  |  |  |  |  |  |  |
|  | 24942 |  |  |  |  |  |  |  |  |  |  |
|  | 24948 |  |  |  |  |  |  |  |  |  |  |
|  | 24954 |  |  |  |  |  |  |  |  |  |  |