3GPP TSG-RAN4 WG4 Meeting #103-e	R4-2208808

# Electronic meeting, May 9 – May 20, 2022

Source: 	vivo

Title: 	Further simulation results for PRS measurement with reduced number of samples

Agenda Item:	9.20.2.2

Document for:	Information

Introduction

In order to specify the positioning accuracy requirement for latency reduction, in this paper, we further provide simulation results for PRS measurement with reduced sample based on the simulation assumptions in the Appendix which made some following corrections based on [1].

- Remove the TDL-A channel model for FR1 and TDL-C channel model for FR2

- Remove the Es/Iot of (-6, -13, -13) for three cells

- Remove the combination of 15kHz SCS and 24 PRS RBs

- Remove the sample number of 2 and 4

Simulation results

UE Rx-Tx time difference

Table 1 and Table 2 are the measurement accuracy of UE Rx-Tx time difference.

Table 1. Simulation results for AWGN channel

| FR | BW (RB) | SCS (kHz) | PRS_Lenth PerSlot | Accuracy [90%] (Tc), SINR = -3dB | Accuracy [90%] (Tc), SINR = -6dB |
| --- | --- | --- | --- | --- | --- |
| FR1 | 52 | 15 | 1 | 92.0 | 90.0 |
|  | 104 | 15 | 1 | 45.5 | 45.5 |
|  | 24 | 30 | 4 | 98.7 | 97.4 |
|  | 48 | 30 | 1 | 49.1 | 47.7 |
|  | 132 | 30 | 1 | 19.3 | 19.3 |
|  | 24 | 60 | 4 | 49.0 | 49.1 |
|  | 64 | 60 | 1 | 19.9 | 19.7 |
|  | 132 | 60 | 1 | 9.8 | 9.7 |
| FR2 | 24 | 60 | 4 | 48.8 | 48.5 |
|  | 64 | 60 | 1 | 19.6 | 19.9 |
|  | 132 | 60 | 1 | 9.5 | 9.7 |
|  | 32 | 120 | 1 | 19.9 | 19.1 |
|  | 64 | 120 | 1 | 9.8 | 10.0 |
|  | 128 | 120 | 1 | 4.9 | 4.8 |
| Note1: PRS_LenthPerSlot = (DL_PRS_NumSymbols x DL_PRS_ResourceRepetitionFactor) /DL_PRS_CombSizeN |  |  |  |  |  |

Table 2. Simulation results for TDL-D channel

| FR | BW (RB) | SCS (kHz) | PRS_Lenth PerSlot | Accuracy [90%] (Tc), SINR = -3dB | Accuracy [90%] (Tc), SINR = -6dB |
| --- | --- | --- | --- | --- | --- |
| FR1 | 52 | 15 | 1 | 100.9 | 107.3 |
|  | 104 | 15 | 1 | 47.8 | 48.7 |
|  | 24 | 30 | 4 | 109.8 | 109.2 |
|  | 48 | 30 | 1 | 54.7 | 56.4 |
|  | 132 | 30 | 1 | 19.5 | 20.3 |
|  | 24 | 60 | 4 | 50.6 | 55.1 |
|  | 64 | 60 | 1 | 20.2 | 21.8 |
|  | 132 | 60 | 1 | 9.7 | 10.2 |
| FR2 | 24 | 60 | 4 | 50.5 | 53.0 |
|  | 64 | 60 | 1 | 20.2 | 21.2 |
|  | 132 | 60 | 1 | 9.7 | 10.0 |
|  | 32 | 120 | 1 | 21.3 | 25.7 |
|  | 64 | 120 | 1 | 10.2 | 10.6 |
|  | 128 | 120 | 1 | 4.9 | 5.1 |
| Note1: PRS_LenthPerSlot = (DL_PRS_NumSymbols x DL_PRS_ResourceRepetitionFactor) /DL_PRS_CombSizeN |  |  |  |  |  |

PRS RSTD

Simulation results for PRS RSTD measurement accuracy are shown in Table 3 and 4 for AGWN and TDL-D channels.

Table 3. Simulation results for AWGN channel

| FR | BW (RB) | SCS (kHz) | PRS_Lenth PerSlot | Accuracy [90%] (Tc), SINR = [-3, -6] dB |
| --- | --- | --- | --- | --- |
| FR1 | 52 | 15 | 1 | 90.2 |
|  | 104 | 15 | 1 | 44.4 |
|  | 24 | 30 | 4 | 92.8 |
|  | 48 | 30 | 1 | 47.0 |
|  | 132 | 30 | 1 | 22.3 |
|  | 24 | 60 | 4 | 43.1 |
|  | 64 | 60 | 1 | 21.8 |
|  | 132 | 60 | 1 | 10.9 |
| FR2 | 24 | 60 | 4 | 51.2 |
|  | 64 | 60 | 1 | 26.8 |
|  | 132 | 60 | 1 | 12.6 |
|  | 32 | 120 | 1 | 28.3 |
|  | 64 | 120 | 1 | 13.6 |
|  | 128 | 120 | 1 | 7.0 |
| Note1: PRS_LenthPerSlot = (DL_PRS_NumSymbols x DL_PRS_ResourceRepetitionFactor) /DL_PRS_CombSizeN |  |  |  |  |

Table 4. Simulation results for TDL-D channel

| FR | BW (RB) | SCS (kHz) | PRS_Lenth PerSlot | Accuracy [90%] (Tc), SINR = [-3, -6] dB |
| --- | --- | --- | --- | --- |
| FR1 | 52 | 15 | 1 | 131.1 |
|  | 104 | 15 | 1 | 56.3 |
|  | 24 | 30 | 4 | 113.8 |
|  | 48 | 30 | 1 | 69.0 |
|  | 132 | 30 | 1 | 25.1 |
|  | 24 | 60 | 4 | 59.0 |
|  | 64 | 60 | 1 | 30.2 |
|  | 132 | 60 | 1 | 12.9 |
| FR2 | 24 | 60 | 4 | 56.5 |
|  | 64 | 60 | 1 | 24.3 |
|  | 132 | 60 | 1 | 12.5 |
|  | 32 | 120 | 1 | 33.1 |
|  | 64 | 120 | 1 | 13.4 |
|  | 128 | 120 | 1 | 6.4 |
| Note1: PRS_LenthPerSlot = (DL_PRS_NumSymbols x DL_PRS_ResourceRepetitionFactor) /DL_PRS_CombSizeN |  |  |  |  |

PRS RSRP

Simulation results for PRS RSRP measurement accuracy are shown in Table 5 and 6 for AGWN and TDL-D channels.

Table 5. Simulation results for AWGN channel

| FR | BW (RB) | PRS_Lenth PerSlot | Accuracy [5%, 95%] (dB), SINR = -3dB |  | Accuracy [5%, 95%] (dB), SINR = -6dB |  |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | 24 | All | -0.76 | 1.06 | -0.80 | 1.30 |
|  | 52 | All | -0.50 | 0.82 | -0.55 | 0.94 |
|  | 104 | All | -0.36 | 0.57 | -0.41 | 0.64 |
| FR2 | 24 | All | -0.76 | 1.04 | -0.88 | 1.33 |
|  | 52 | All | -0.41 | 0.68 | -0.58 | 0.76 |
|  | 104 | All | -0.30 | 0.48 | -0.24 | 0.69 |
| Note1: PRS_LenthPerSlot = (DL_PRS_NumSymbols x DL_PRS_ResourceRepetitionFactor) /DL_PRS_CombSizeN |  |  |  |  |  |  |

Table 6. Simulation results for TDL-D channel

| FR | BW (RB) | PRS_Lenth PerSlot | Accuracy [5%, 95%] (dB), SINR = -3dB |  | Accuracy [5%, 95%] (dB), SINR = -6dB |  |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | 24 | All | -1.41 | 1.18 | -1.94 | 1.43 |
|  | 52 | All | -0.90 | 0.80 | -1.15 | 1.03 |
|  | 104 | All | -0.60 | 0.60 | -0.76 | 0.65 |
| FR2 | 24 | All | -1.38 | 1.03 | -1.94 | 1.50 |
|  | 52 | All | -0.73 | 0.69 | -0.94 | 0.92 |
|  | 104 | All | -0.51 | 0.44 | -0.70 | 0.62 |
| Note1: PRS_LenthPerSlot = (DL_PRS_NumSymbols x DL_PRS_ResourceRepetitionFactor) /DL_PRS_CombSizeN |  |  |  |  |  |  |

- References

- R4-2120330 Link level simulation assumptions for NR PRS measurements with reduced latency, Huawei, HiSilicon

- Appendix

Table 1: General parameters

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Cell layout | 3 cells at distinct locations: <cell 1, cell 2, cell 3>, where cell 1 is the serving cell/reference cellNOTE 1 |  |
| Network synchronization | • Synchronous with time shifts <0, 0, 3 us> • Asynchronous with time shifts: <0, 7 symbols, -7 symbols> |  |
| Duplex modes | FDD and TDD |  |
| TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) | - TDDConf.3.1 (120 kHz) |
| Data and CCH load in PRS symbols | no other cell transmissions in its positioning symbols, except PRS |  |
| Data and CCH load in non-PRS symbols | - 50% utilization in time - 100% RE utilization |  |
| Cyclic prefix | Normal |  |
| DRX | OFF |  |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 15 kHz - FDD, TDD - 4 GHz - 30 kHz - FDD, TDD | - 40 GHz - 120 kHz - TDD |
| Propagation conditions [TS 38.101-4] | AWGN, TDL-D (30 ns delay spread, 5Hz) | AWGN, TDL-D (30 ns delay spread, 5Hz) |
| Es/Iot for tw cells (cell 1, cell 2, cell 3), [dB] | (-3, -6, -6) |  |
| Number of UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming) |  |
| UE measurement bandwidth | Full carrier bandwidth |  |
| Number of transmit PRS antennas | 1 |  |
| Cell ID, TRP ID, PRS Resource Set ID | Selected to ensure non-overlapping PRS REs in frequency |  |
| Number of DL PRS Resource sets for a positioning fix | 1 (including all PRS resource repetitions) |  |
| PRS muting | No muting, muting (comb-2) |  |
| Power boosting | No power boosting |  |
| ExpectedRSTD-UncertaintyNOTE 2 | 5 us (15 kHz), 2.5 us (30 kHz), 0.625 us (120 kHz) |  |
| NOTE 1: nr-DL-PRS-ReferenceInfo is a member of NR-DL-PRS-AssistanceData which is a common NR positioning IE applicable to all positioning methods (see clause 6.4.3 of TS 37.355). NOTE 2: nr-DL-PRS-expectedRSTD-uncerainty is a member of NR-DL-PRS-AssistanceData which is a common NR positioning IE applicable to all positioning methods (see clause 6.4.3 of TS 37.355). |  |  |

Table 2: PRS transmission configuration parameters

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 52 | 1 | 128 |
|  |  | 104 | 1 | 64 |
|  | 30 | 24 | 4 | 128 |
|  |  | 48 | 1 | 64 |
|  |  | 132 | 1 | 32 |
|  | 60, FR1 | 24 | 4 | 64 |
|  |  | 64 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 60, FR2 | 24 | 4 | 64 |
|  |  | 64 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 120 | 32 | 4 | 32 |
|  |  | 64 | 1 | 16 |
|  |  | 128 | 1 | 8 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| PRS periodicity | 40ms |  |  |  |
| Sample number | 1 |  |  |  |
| Note: Repetition 4 is achieved by PRS in 4 consecutive slots with PRS-ResourceTimeGap=1. |  |  |  |  |

At least the following performance characteristics are to be provided for PRS RSTD:

- RSTD error CDFs for the 2 neighbour cells

- 5%-ile and 95%-tile of the RSTD errors for each neighbour cell

At least the following performance characteristics are to be provided for PRS RSRP:

- RSRP error CDFs for 3 cells

- 90%-ile of the RSRP errors for each cell

At least the following performance characteristics are to be provided for TUE-RX:

- TUE-RX error CDFs for the 3 cells

- 90%-ile of the TUE-RX errors for each cell

In the above,

- RSTD error = estimated RSTD – ideal RSTD (based on perfect channel knowledge).

- PRS-RSRP error = estimated PRS-RSRP – ideal PRS-RSRP (based on perfect channel knowledge).

- TUE-RX error = abs (estimated TUE-RX – ideal TUE-RX) (based on perfect channel and UE location knowledge).