3GPP TSG-RAN WG4 Meeting #103-e	R4-2208998

# Online, May 09 – 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2352 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on maintaining RLM/BFD relaxation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In R17, the RLM/BFD relaxation requirements have been introduced into TS38.133. However, the signalling names and references are still TBD. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on RAN2’s spec 38.331, RLM/BFD relaxation requirements have been further updated. - To update the signalling and references used for RLM relaxation requirements in section 8.1.1.2. - To update the signalling and references used for BFD relaxation requirements in section 8.5.1.2. - To correct the referred section number. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RLM/BFD relaxation requirements are not completed in R17. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2, 8.5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of change 1>

#### 8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria

For the UE supports [connected mode power saving] and configured with goodServingCellEvaluationRLM:

- when the UE is not performing intra-band carrier aggregation configured with RLM and BFD on SCell, and

  - UE has fulfilled good serving cell quality criterion defined in clause 5.7.x.2 in TS 38.331 [2] if lowMobilityEvaluationConnected [2] is not configured, or

  - UE is also configured with lowMobilityEvaluationConnected [2] and UE has fulfilled both good serving cell quality criterion defined in clause 5.7.x.2 in TS 38.331 [2] and low mobility criterion defined in clause 5.7.x.1 in TS 38.331 [2].

- when the UE is performing intra-band carrier aggregation configured with RLM and BFD on SCell, and

  - UE has fulfilled good serving cell quality criterion defined in clause 5.7.x.2 in TS 38.331 [2] for RLM on SpCell and for BFD on SCell if lowMobilityEvaluationConnected [2] is not configured, or

  - the UE is configured with lowMobilityEvaluationConnected [2], and UE has fulfilled both good serving cell quality criterion defined in clause 5.7.x.2 in TS 38.331 [2] and low mobility criterion defined in clause 5.7.x.1 in TS 38.331 [2] for RLM on SpCell and for BFD on SCell.

The UE is allowed to apply the relaxed requirements defined in clause 8.1.2.4 for SSB based radio link monitoring and the relaxed requirements defined in clause 8.1.3.4 for CSI-RS based radio link monitoring; otherwise, UE shall apply the requirements defined in clause 8.1.2.2 for SSB based radio link monitoring and the requirements defined in clause 8.1.3.2 for CSI-RS based radio link monitoring.

The UE is not allowed to relax RLM measurements and apply the relaxed radio link monitoring provided that at least one of the following conditions is met:

- The UE sends out-of sync indications to the higher layers,

- The UE has triggered T310 timer

### <End of change 1>

### <Start of change 2>

#### 8.5.1.2	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria

For the UE supports [connected mode power saving] and configured with goodServingCellEvaluationBFD:

- when the UE is not performing intra-band carrier aggregation configured with RLM on SpCell and BFD on SCell provided that:

- UE has fulfilled good serving cell quality criterion defined in clause 5.7.x.2 in TS 38.331 [2] if lowMobilityEvaluationConnected [2] is not configured, or

- UE is also configured with lowMobilityEvaluationConnected [2] and UE has fulfilled both good serving cell quality criterion defined in clause 5.7.x.2 in TS 38.331 [2] and low mobility criterion defined in clause 5.7.x.1 in TS 38.331 [2].

- when the UE is performing intra-band carrier aggregation configured with RLM on SpCell and BFD on SCell provided that:

- UE has fulfilled good serving cell quality criterion defined in clause 5.7.x.2 in TS 38.331 [TBD] for RLM on SpCell and for BFD on SCell if lowMobilityEvaluationConnected [2] is not configured, or

- the UE is also configured with lowMobilityEvaluationConnected [2], and UE has fulfilled both good serving cell quality criterion defined in clause 5.7.x.2 in TS 38.331 [2] and low mobility criterion defined in clause 5.7.x.1 in TS 38.331 [2] for RLM on SpCell and for BFD on SCell.

The scenario and RS resource configurations in the set  defined in section 8.5.1 apply for this section.

The UE is allowed to apply the relaxed requirements defined in clause 8.5.2.4 for SSB based beam failure detection and the relaxed requirements defined in clause 8.5.3.4 for CSI-RS based beam failure detection; otherwise, UE shall apply the requirements defined in clause 8.5.2.2 for SSB based beam failure detection and the requirements defined in clause 8.5.3.2 for CSI-RS based beam failure detection.

The UE is not allowed to apply the relaxed link recovery procedures provided that at least one of the following conditions is met:

- The UE sends a beam failure instance indication to the higher layers,

- The UE has triggered T310 timer.

### <End of change 2>