3GPP TSG-RAN WG4 Meeting # 103-e                     R4-2210030

Electronic Meeting, 9st of May – 20th of May, 2022

Agenda item:	     9.12.3.4

Source:	THALES

Title:	pCR for Clause 7.4 In-band selectivity and blocking - TS 38.108

Document for:	Approval

# Introduction

Draft TS skeleton for Non-terrestrial networks (NTN) related to NR; Satellite Access Node radio transmission and reception (TS 38.108) has been approved in R4-2203087 [1] and the work split has been summarized in R4-2203080 Way Forward on NTN_solutions_Part1 [2] approved in RAN4 101-bis-e meeting.

For further information, please also follow the discussions in R4-2203111 [3] and the updated TS 38.108 at RAN4 102-e meeting, and which can be found in R4-2207513 [4].

In this document, following changes have been proposed as a pCR to update TS 38.108:

- Section 7.4: In-band selectivity and blocking

# Reference

[1] R4-2203087 Skeleton for TS 38.108 NR Satellite Access Node radio transmission and reception v0.0.1, THALES.

[2] R4-2203080 Way Forward on NTN_solutions_Part1, THALES.

[3] R4-2203111 Email discussion summary for [101-bis-e][306] NTN_Solutions_Part1, THALES.

[4] R4-2207513 TS 38.108 v0.1.0, THALES.

# Text proposals for TS 38.108

------------------------------------------------<Start of TP>----------------------------------------------

## 7.4	In-band selectivity and blocking

### 7.4.1	Adjacent Channel Selectivity (ACS)

#### 7.4.1.1	General

Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency at TAB connector for SAN type 1-H in the presence of an adjacent channel signal with a specified center frequency offset of the interfering signal to the band edge of a victim system.

#### 7.4.1.2	Minimum requirements for Satellite Access Node

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For SAN, the wanted and the interfering signal coupled to the SAN type 1-H TAB connector are specified in table 7.4.1.2-1 and the frequency offset between the wanted and interfering signal in table 7.4.1.2-2 for ACS. The reference measurement channel for the wanted signal is identified in table 7.2.2-1 and 7.2.2-2 for each SAN channel bandwidth in any operating band and further specified in annex A.1. The characteristics of the interfering signal is further specified in [annex D].

The ACS requirement is applicable outside the SAN RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the SAN RF Bandwidth edges or Radio Bandwidth edges.

Minimum conducted requirement is defined at the TAB connector for SAN type 1-H.

Table 7.4.1.2-1: Satellite Access Node ACS requirement

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20 (Note 1) | PREFSENS + 6 dB | GEO SAN class: -57 LEO SAN class: -60 |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the SAN for that bandwidth. NOTE 2: PREFSENS depends on the SAN channel bandwidth as specified in table 7.2.2-1. |  |  |

Table 7.4.1.2-2: Satellite Access Node ACS interferer frequency offset values

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal center frequency offset from the lower/upper SAN RF Bandwidth edge (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±2.5025 |  |
| 10 | ±2.5075 | 5 MHz OFDM NR signal |
| 15 | ±2.5125 | 15 kHz SCS, 25 RBs |
| 20 | ±2.5025 |  |

### 7.4.2 	In-band blocking

#### 7.4.2.1	General

The in-band blocking characteristics is a measure of the receiver's ability to receive a wanted signal at its assigned channel at the TAB connector for SAN type 1-H in the presence of an unwanted interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

#### 7.4.2.2	Minimum requirements for Satellite Access Node

In-band blocking requirement is not applicable for SAN.

------------------------------------------------<End of TP>-----------------------------------------------