3GPP TSG-RAN4 Meeting #103-e	R4-2210404

# Electronic Meeting, 9th - 20th May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-1 to introduce new configurations for NR inter-band CA 3 bands DL with 2 bands UL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R17_3BDL_2BUL-Core |  |  |  |  |  | Date: |  |  | 2022-04-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Specify new configurations to inter-band CA with 3BDL_2BUL within FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding following new configurations because no further technical study needed. CA_n3A-n28A-n41B CA_n3A-n41B-n77A CA_n28A-n41B-77A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Corresponding band combiniations will not exist in current specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

5.5A.3.2	Configurations for inter-band CA (three bands)

Table 5.5A.3.2-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n5A | CA_n1A-n3A CA_n1A-n5A CA_n3A-n5A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n7A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A-n3A-n7B | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n1A-n3A-n8A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n18A | CA_n1A-n3A CA_n1A-n18A CA_n3A-n18A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n18 | 5, 10, 15 |  |
| CA_n1A-n3A-n20A | CA_n1A-n3A CA_n1A-n20A CA_n3A-n20A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n20 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n28A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  | CA_n1A-n3A CA_n1A-n28A CA_n3A-n28A | n1 | 5, 10, 15, 20 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 201, 301 |  |
| CA_n1A-n3A-n41A | CA_n1A-n3A CA_n1A-n41A CA_n3A-n41A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n3A-n67A | CA_n1A-n3A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n77A | CA_n1A-n3A CA_n1A-n77A CA_n3A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n3A-n77(2A) | CA_n1A-n3A CA_n1A-n77A CA_n3A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n3A-n78A | CA_n1A-n3A CA_n1A-n78A7 CA_n3A-n78A7A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n78(2A) | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3A-n79A | CA_n1A-n3A CA_n1A-n79A CA_n3A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n5A-n7A | CA_n1A-n5A CA_n1A-n7A CA_n5A-n7A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A-n5A-n7B | CA_n1A-n5A CA_n1A-n7A CA_n5A-n7A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n1A-n5A-n28A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | 28 | 5, 10, 15, 20, 30 |  |
| CA_n1A-n5A-n78A | CA_n1A-n5A CA_n1A-n78A CA_n5A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n1A-n7A-n8A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20, 35 |  |
| CA_n1A-n8A-n28A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20, 35 |  |
|  |  | n28 | 10, 15, 20 |  |
| CA_n1A-n8A-n77A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n8A-n77(2A) | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n7A-n28A | CA_n1A-n7A CA_n1A-n28A CA_n7A-n28A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n7B-n28A | CA_n1A-n28A CA_n1A-n7A CA_n7A-n28A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n7A-n78A | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 901, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 901, 100 |  |
| CA_n1A-n7B-n78A | CA_n1A-n78A CA_n1A-n7A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n1A-n7A-n78(2A) | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n8A-n78A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n8A-n78(2A) | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
| CA_n1A-n8A-n79A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n18A-n28A | CA_n1A-n18A CA_n1A-n28A CA_n18A-n28A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
| CA_n1A-n18A-n41A | CA_n1A-n18A CA_n1A-n41A CA_n18A-n41A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n18A-n77A | CA_n1A-n18A CA_n1A-n77A CA_n18A-n77A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n20A-n67A | CA_n1A-n20A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
| CA_n1A-n20A-n78A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n40A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
| CA_n1A-n28A-n40B | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n40 | CA_n40B_BCS0 |  |
| CA_n1A-n28A-n41A | CA_n1A-n28A CA_n1A-n41A CA_n28A-n41A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n28A-n77A | CA_n1A-n28A CA_n1A-n77A CA_n28A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n28A-n77(2A) | CA_n1A-n28A CA_n1A-n77A CA_n28A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n1A-n28A-n78A | CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n78(2A) | CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n28A-n79A | CA_n1A-n28A CA_n1A-n79A CA_n28A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n40A-n78A | CA_n1A-n40A CA_n1A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n1A-n40A CA_n1A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20 | 1 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n1A-n40A CA_n1A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20 | 2 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n40B-n78A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n40 | CA_n40B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n41A-n77A | CA_n1A-n41A CA_n1A-n77A CA_n41A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n77A-n79A4 | CA_n1A-n77A CA_n1A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n77(2A)-n79A4 | CA_n1A-n77A CA_n1A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n78A-n79A5 | CA_n1A-n78A CA_n1A-n79A CA_n78A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n78(2A)-n79A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n2A-n5A-n30A | CA_n2A-n5A CA_n2A-n30A CA_n5A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n5A-n48A | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n48B | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n2A-n5A-n48(2A) | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
| CA_n2A-n5A-n48(A-B) | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
| CA_n2(2A)-n5A-n30A | CA_n2A-n5A CA_n2A-n30A CA_n5A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n5A-n66A | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n5A-n66A | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n5A-n66(2A) | CA_n2A-n5A CA_n2A-n66A CA_n5A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n2A-n5A-n77A | n777 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n77C | CA_n2A-n5A CA_n2A-n77A CA_n5A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n5A-n77(2A) | n777 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n5A-n77A | n777 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n30A | CA_n2A-n12A CA_n2A-n30A CA_n12A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2(2A)-n12A-n30A | CA_n2A-n12A CA_n2A-n30A CA_n12A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n12A-n66A | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n12A-n66A | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n12A-n66(2A) | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2(2A)-n12A-n66(2A) | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n12A-n66(3A) | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n2A-n12A-n77A | n777 CA_n2A-n12A CA_n2A-n77A7 CA_n12A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n12A-n77A | n777 CA_n2A-n12A CA_n2A-n77A7 CA_n12A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n77(2A) | n777 CA_n2A-n12A CA_n2A-n77A7 CA_n12A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n14A-n30A | CA_n2A-n14A CA_n2A-n30A CA_n14A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2(2A)-n14A-n30A | CA_n2A-n14A CA_n2A-n30A CA_n14A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n14A-n66A | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n14A-n66A | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n14A-n66(2A) | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n14A-n77A | n777 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n14A-n77(2A) | n777 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n14A-n77A | n777 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n29A-n30A | CA_n2A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2(2A)-n29A-n30A | CA_n2A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n29A-n66A | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n29A-n66A | CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n29A-n66(2A) | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2(2A)-n29A-n66(2A) | CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n29A-n77A | n777 CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n29A-n77A | n777 CA_n2A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n29A-n77(2A) | n777 CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n30A-n66A | CA_n2A-n30A CA_n30A-n66A CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n2(2A)-n30A-n66A | CA_n2A-n30A CA_n30A-n66A CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n2A-n30A-n66(2A) | CA_n2A-n30A CA_n30A-n66A CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n2A-n30A-n77A | n777 CA_n2A-n30A CA_n2A-n77A7 CA_n30A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n30A-n77(2A) | n777 CA_n2A-n30A CA_n2A-n77A7 CA_n30A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n30A-n77A | n777 CA_n2A-n30A CA_n2A-n77A7 CA_n30A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48(A-B)-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48B-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48(2A)-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48A-n77A | CA_n2A-n48A CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A-n77C | CA_n2A-n48A CA_n2A-n77A CA_n77C | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n48B-n77A | CA_n2A-n48A CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48(2A)-n77A | CA_n2A-n48A CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n77A | n777 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n66A-n77A | n777 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66(2A)-n77A | n777 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n77C | CA_n2A-n66A CA_n66A-n77A CA_n2A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n66A-n77(2A) | n777 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n66A-n78A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n78(2A) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n2A-n71A-n78A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n71A-n78(2A) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n5A-n7A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n3A-n5A-n7B | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  | CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n3A-n5A-n28A | - | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n3A-n5A-n78A | CA_n3A-n5A CA_n3A-n78A CA_n5A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n8A | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20, 35 |  |
| CA_n3A-n7A-n28A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  | CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3A-n7B-n28A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  | CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3A-n7A-n78A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n3A-n7B-n78A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n3A-n7A-n78(2A) | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n8A-n28A | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20, 35 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n3A-n8A-n77A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n8A-n77(2A) | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n3A-n8A-n78A | CA_n3A-n8A CA_3A-n78A CA_n8A-n78A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n18A-n28A | CA_n3A-n18A CA_n3A-n28A CA_n18A-n28A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
| CA_n3A-n18A-n41A | CA_n3A-n41A CA_n3A-n18A CA_n18A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n18A-n77A | CA_n3A-n18A CA_n3A-n77A CA_n18A-n77A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n20A-n67A | CA_n3A-n20A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
| CA_n3A-n20A-n78A | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n41A | CA_n3A-n28A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n28A-n41B | CA_n3An28A | n3 | 5, 10, 15, 20 | 0 |
|  | CA_n3A-n41A | n28 | 5, 10 |  |
|  | CA_n28A-n41A | n41 | CA_n41B_BCS0 |  |
| CA_n3A-n28A-n77A | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n28A-n77(2A) | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  | CA_n77(2A) | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n3A-n28A-n77(3A) | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| CA_n3A-n28A-n78A | CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n28 | 5, 10 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n28A-n78(2A) | CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n28A-n79A | CA_n3A-n28A CA_n3A-n79A CA_n28A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
| CA_n3A-n77A-n79A4 | CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n77(2A)-n79A4 | CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n40A-n41A | CA_n3A-n40A CA_n3A-n41A CA_n40A-n41A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n41A-n77A | CA_n3A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | CA_n3A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n77A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n41B-n77A | CA_n3A-n41A | n3 | 5, 10, 15, 20 | 0 |
|  | CA_n3A-n77A | n41 | CA_41B_BCS0 |  |
|  | CA_n41A-n77A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n41A-n77(2A) | CA_n3A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | CA_n3A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n77A | n77 | CA_n77(2A)_BCS0 |  |
| CA_n3A-n41A-n78A | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  | CA_n3A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n78A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n41A-n78(2A) | CA_n3A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | CA_n3A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n78A | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n41A-n79A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n5A-n7A-n28A | - | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 25, 30, 35, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n5A-n7A-n78A | CA_n5A-n78A7 CA_n7A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A | n5 | 5, 10, 15, 20 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n5A-n7B-n78A | CA_n5A-n78A7 CA_n7A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A CA_n7B | n5 | 5, 10, 15, 20 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n5A-n12A-n77A | n777 CA_n5A-n12A CA_n5A-n77A7 CA_n12A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n12A-n77(2A) | n777 CA_n5A-n12A CA_n5A-n77A7 CA_n12A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n14A-n77A | n777 CA_n5A-n14A CA_n5A-n77A7 CA_n14A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n14A-n77(2A) | n777 CA_n5A-n14A CA_n5A-n77A7 CA_n14A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25A-n66A | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n25(2A)-n66A | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n25A-n66(2A) | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n5A-n25(2A)-n66(2A) | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n5A-n25A-n77A | CA_n5A-n25A | n5 | 5, 10, 15, 20 | 0 |
|  | CA_n5A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | CA_n25A-n77A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25(2A)-n77A | CA_n5A-n25A CA_n5A-n77A CA_n25A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25A-n77(2A) | CA_n5A-n25A CA_n5A-n77A CA_n25A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25(2A)-n77(2A) | CA_n5A-n25A CA_n5A-n77A CA_n25A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25A-n78A | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25(2A)-n78A | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25A-n78(2A) | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n25(2A)-n78(2A) | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n29A-n77A | n777 CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n29A-n77(2A) | n777 CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n30A-n66A | CA_n5A-n30A CA_n30A-n66A CA_n5A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n5A-n30A-n66(2A) | CA_n5A-n30A CA_n30A-n66A CA_n5A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n5A-n30A-n77A | n777 CA_n5A-n30A CA_n5A-n77A7 CA_n30A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n30A-n77(2A) | n777 CA_n5A-n30A CA_n5A-n77A7 CA_n30A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n48A-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48(A-B)-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48B-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48(2A)-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48A-n77A | CA_n5A-n48A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48A-n77C | CA_n5A-n48A CA_n5A-n77A CA_n77C | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n48B-n77A | CA_n5A-n48A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48B-n77C | CA_n5A-n48A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | CA_n77C BCS1 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | CA_n77C BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 3 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | CA_n77C BCS1 |  |
| CA_n5A-n48(2A)-n77A | CA_n5A-n48A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48(2A)-n77C | CA_n5A-n48A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 3 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n66A-n77A | n777 CA_n5A-n66A CA_n66A-n77A7 CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66(2A)-n77A | n777 CA_n5A-n66A CA_n66A-n77A7 CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66(2A)-n77(2A) | CA_n5A-n66A CA_n66A-n77A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n66A-n77C | CA_n5A-n66A CA_n66A-n77A CA_n5A-n77A | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n66A-n77(2A) | n777 CA_n5A-n66A CA_n66A-n77A7 CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n66A-n78A | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66(2A)-n78A | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66A-n78(2A) | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n66(2A)-n78(2A) | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n8A-n28A | - | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20, 35 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n7A-n8A-n78A | - | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20, 35 |  |
|  |  | n78 | 10, 15, 20, 30 |  |
| CA_n7A-n25A-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7A-n25(2A)-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7A-n25(2A)-n66(2A) | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n7A-n25A-n66(2A) | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n7(2A)-n25A-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7(2A)-n25(2A)-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7(2A)-n25A-n66(2A) | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n7(2A)-n25(2A)-n66(2A) | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n7A-n25A-n77A | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25(2A)-n77A | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25A-n77(2A) | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n25(2A)-n77(2A) | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n25A-n77A | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25(2A)-n77A | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25A-n77(2A) | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n25(2A)-n77(2A) | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n25A-n78A | CA_n7A-n25A CA_n7A-n78A CA_n25A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |
| CA_n7A-n25A-n78(2A) | CA_n7A-n25A CA_n7A-n78A CA_n25A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n28A-n78(2A) | CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n28A-n78A | CA_n7A-n78A7 CA_n28A-n78A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n7B-n28A-n78A | CA_n7A-n78A7 CA_n28A-n78A7 | n7 | CA_n7B_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A CA_n7B | n7 | CA_n7B_BCS0 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n7A-n46A-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n46C-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n46D-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66A-n77A | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66(2A)-n77A | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66A-n77(2A) | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n66(2A)-n77(2A) | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n66A-n77A | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66(2A)-n77A | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66A-n77(2A) | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n66(2A)-n77(2A) | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n66A-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66A-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7(2A)-n66A-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66(2A)-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66(2A)-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66A-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7(2A)-n66(2A)-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n8A-n28A-n78A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n8A-n39A-n41A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  |  | n8 | 5, 10, 15, 20 | 1 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
| CA_n8A-n40A-n41A | CA_n8A-n40A CA_n8A-n41A CA_n40A-n41A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n8A-n41A-n79A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n8 | 5, 10, 15, 20 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n8A-n78A-n79A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n8A-n78(2A)-n79A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n12A-n30A-n66A | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n12 | 5, 10, 15 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n12A-n30A-n66(2A) | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n12 | 5, 10, 15 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n12A-n30A-n66(3A) | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n12 | 5, 10, 15 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n12A-n30A-n77A | n777 CA_n12A-n30A, CA_n12A-n77A7 CA_n30A-n77A7 | n12 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n30A-n77(2A) | n777 CA_n12A-n30A CA_n12A-n77A7 CA_n30A-n77A7 | n12 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n12A-n66A-n77A | n777 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 | n12 | 5, 10, 15 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n66(2A)-n77A | n777 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 | n12 | 5, 10, 15 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n66A-n77(2A) | n777 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 | n12 | 5, 10, 15 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n13A-n25A-n66A | CA_n13A-n25A CA_n13A-n66A CA_n25A-n66A | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n13A-n25A-n77A | - | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n13A-n66A-n77A | - | n13 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n30A-n66A | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n14A-n30A-n66(2A) | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n14A-n30A-n77A | n777 CA_n14A-n30A CA_n14A-n77A7 CA_n30A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n30A-n77(2A) | n777 CA_n14A-n30A CA_n14A-n77A7 CA_n30A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n14A-n66A-n77A | n777 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n66(2A)-n77A | n777 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n66A-n77(2A) | n777 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n18A-n28A-n41A | CA_n18A-n28A CA_n18A-n41A CA_n28A-n41A | n18 | 5, 10, 15 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n18A-n28A-n77A | CA_n18A-n28A CA_n18A-n41A CA_n28A-n41A | n18 | 5, 10, 15 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n18A-n41A-n77A | CA_n18A-n28A CA_n18A-n41A CA_n28A-n41A | n18 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n20A-n28A-n78A | - | n20 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n24A-n41A-n48A | CA_n24A-n41A CA_n24A_n48A CA_n41_n48A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n41(2A)-n48A | CA_n24A-n41A CA_n24A_n48A CA_n41_n48A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A) BCS1 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n41A-n48(2A) | CA_n24A-n41A CA_n24A_n48A CA_n41_n48A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n48 | CA_n48(2A) BCS0 |  |
| CA_n24A-n41(2A)-n48(2A) | CA_n24A-n41A CA_n24A_n48A CA_n41_n48A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A) BCS1 |  |
|  |  | n48 | CA_n48(2A) BCS0 |  |
| CA_n24A-n41A-n77A | CA_n24A-n41A CA_n24A_n77A CA_n41_n77A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n41(2A)-n77A | CA_n24A-n41A CA_n24A_n77A CA_n41_n77A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n24 | 5, 10 | 1 |
|  |  | n41 | CA_n41(2A) BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n41A-n77(2A) | CA_n24A-n41A CA_n24A_n77A CA_n41_n77A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n24 | 5, 10 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A) BCS0 |  |
| CA_n24A-n41(2A)-n77(2A) | CA_n24A-n41A CA_n24A_n77A CA_n41_n77A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n24 | 5, 10 | 1 |
|  |  | n41 | CA_n41(2A) BCS1 |  |
|  |  | n77 | CA_n77(2A) BCS0 |  |
| CA_n24A-n48A-n77A |  | n24 | 5, 10 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n48(2A)-n77A |  | n24 | 5, 10 | 0 |
|  |  | n48 | CA_n48(2A) BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n48A-n77(2A) |  | n24 | 5, 10 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A) BCS0 |  |
| CA_n24A-n48(2A)-n77(2A) |  | n24 | 5, 10 | 0 |
|  |  | n48 | CA_n48(2A) BCS0 |  |
|  |  | n77 | CA_n77(2A) BCS0 |  |
| CA_n25A-n29A-n66A | CA_n25A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n38A-n66A | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25(2A)-n38A-n66A | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25(2A)-n38A-n66(2A) | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n25A-n38A-n66(2A) | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n25A-n38A-n78A | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n38A-n78(2A) | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25(2A)-n38A-n78A | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n38A-n78(2A) | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n41A-n66A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n41A-n66(2A) | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n25A-n41C-n66A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n41(2A)-n66A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25(2A)-n41A-n66A | - | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | CA_n25(2A)_BCS1 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 30, 40 |  |
| CA_n25A-n41A-n71A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n25A-n41A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  | CA_n41A-n71A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n25A-n71A | n71 | 5, 10, 15, 20 |  |
| CA_n25A-n41A-n71B | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | CA_n71B_BCS2 |  |
|  | CA_n25A-n41A CA_n41A-n71A CA_n25A-n71A | n25 | 5, 10, 15, 20, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | CA_n71B_BCS2 |  |
| CA_n25A-n41A-n71(2A) | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  | CA_n25A-n41A CA_n41A-n71A CA_n25A-n71A | n25 | 5, 10, 15, 20, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | CA_n71B_BCS0 |  |
| CA_n25A-n41(2A)-n71A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n25A-n41A CA_n41A-n71A CA_n25A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n25A-n41C-n71A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n25A-n41A CA_n41A-n71A CA_n25A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n25(2A)-n41A-n71A | - | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n25A-n41A CA_n41A-n71A CA_n25A-n71A | n25 | CA_n25(2A)_BCS1 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n25A-n41A-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n41(2A)-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n25A-n41A-n77(2A) | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n25(2A)-n41A-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n41C-n77A | CA_n41C CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41C_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n41A-n78A | CA_n25A-n41A CA_n25A-n78A CA_n41A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n41A-n78(2A) | CA_n25A-n41A CA_n25A-n78A CA_n41A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n48A-n66A | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 50 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n48(2A)-n66A | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n48C-n66A | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n66A-n71A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n25A-n66A-n71B | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71B_BCS2 |  |
| CA_n25A-n66A-n71(2A) | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n25A-n66(2A)-n71A | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n25(2A)-n66A-n71A | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n25A-n66A-n77A | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66(2A)-n77A | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66A-n77(2A) | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n25A-n66(2A)-n77(2A) | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n25(2A)-n66A-n77A | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n66(2A)-n77A | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n66A-n77(2A) | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n25(2A)-n66(2A)-n77(2A) | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n25A-n66A-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n66A-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66(2A)-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66A-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25(2A)-n66(2A)-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n66A-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n66(2A)-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25(2A)-n66(2A)-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n71A-n77A | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n71B-n77A | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n71(2A)-n77A | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n71A-n77A | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n71A-n78A | CA_n25A-n71A CA_n25A-n78A CA_n71A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n71A-n78(2A) | CA_n25A-n71A CA_n25A-n78A CA_n71A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n26A-n66A-n70A | CA_n26A-n66A CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n26A-n66(2A)-n70A | CA_n26A-n66A CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n28A-n40A-n41A | CA_n28A-n40A CA_n28A-n41A CA_n40A-n41A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n40A-n78A | CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A | n28 | 5, 10, 15, 20 | 1 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n28A-n40A-n79A | CA_n28A-n40A CA_n28A-n79A CA_n40A-n79A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n41A-n77A | CA_n28A-n41A | n28 | 5, 10, 15, 20, 30 | 0 |
|  | CA_n28A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n77A | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n41B-n77A | CA_n28A-n41A | n28 | 5, 10 | 0 |
|  | CA_n28A-n77A | n41 | CA_n41B_BCS0 |  |
|  | CA_n41A-n77A | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n41A-n77(2A) | CA_n28A-n41A | n28 | 5, 10, 15, 20, 30 | 0 |
|  | CA_n28A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n77A | n77 | CA_n77(2A)_BCS0 |  |
| CA_n28A-n41A-n78A | CA_n28A-n41A CA_n41A-n78A CA_n28A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n28A-n41A-n78(2A) | CA_n78(2A) | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n28A-n41A-n79A | CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n46A-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n46C-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n46D-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n77A-n79A4 | CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n77(2A)-n79A4 | CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n78A-n79A | CA_n28A-n78A CA_n28A-n79A CA_n78A-n79A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n29A-n30A-n66A | CA_n30A-n66A | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n29A-n30A-n66(2A) | CA_n30A-n66A | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n29A-n30A-n77A | n777 CA_n30A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n30A-n77(2A) | n777 CA_n30A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n29A-n66A-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66B-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66B_BCS0. |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66(2A)-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66A-n77A | n777 CA_n66A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n66(2A)-n77A | n777 CA_n66A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n66A-n77(2A) | n777 CA_n66A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n30A-n66A-n77A | n777 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 | n30 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n30A-n66(2A)-n77A | n777 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 | n30 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n30A-n66A-n77(2A) | n777 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 | n30 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n38A-n66A-n78A | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n38A-n66A-n78(2A) | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n38A-n66(2A)-n78A | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n38A-n66(2A)-n78(2A) | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n39A-n40A-n41A | CA_n39A-n40A CA_n39A-n41A CA_n40A-n41A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n39A-n40A-n79A | CA_n39A-n40A CA_n40A-n79A CA_n39A-n79A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n39A-n41A-n79A | - | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n40A-n41A-n79A | CA_n40A-n41A CA_n40A-n79A CA_n41A-n79A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  |  | n79 | , 40, 50, 60, 80, 100 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n79 | , 40, 50, 60, 80, 100 |  |
| CA_n41A-n66A-n70A | CA_n41A-n66A CA_n41A-n70A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n41A-n66A-n71A | - | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n41A-n66A-n71B | CA_n41A-n66A CA_n41A-n71A CA_n66A-n71A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71B_BCS2 |  |
| CA_n41A-n66A-n71(2A) | CA_n41A-n66A CA_n41A-n71A CA_n66A-n71A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n71(2A)_BCS0 |  |
| CA_n41A-n66(2A)-n71A | CA_n41A-n66A CA_n66A-n71A CA_n41A-n71A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n41(2A)-n66A-n71A | - | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A | n41 | CA_n41(2A)_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n41C-n66A-n71A | - | n41 | CA_n41C_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A | n41 | CA_n41C_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n41A-n66A-n77A | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66A-n77(2A) | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n41A-n66(2A)-n77A | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66(2A)-n77(2A) | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n41(2A)-n66A-n77A | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41C-n66A-n77A | CA_41C CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | CA_n41C_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66A-n78A | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66A-n78(2A) | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n41A-n66(2A)-n78A | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66(2A)-n78(2A) | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n41A-n70A-n78A | CA_n41A-n70A CA_n41A-n78A CA_n70A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n71A-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n71B-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n71(2A)-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n71A-n77(2A) | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n41(2A)-n71A-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41C-n71A-n77A | CA_41C CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | CA_n41C_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n71A-n78A | CA_n41A-n71A CA_n41A-n78A CA_n71A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n71A-n78(2A) | CA_n41A-n71A CA_n41A-n78A CA_n71A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n46A-n48A-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46B-n48A-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46C-n48A-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46D-n48A-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46N-n48A-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46A-n48B-n96A | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46B-n48B-n96A | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46C-n48B-n96A | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46D-n48B-n96A | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46N-n48B-n96A | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46A-n48C-n96A | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46B-n48C-n96A | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46C-n48C-n96A | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46D-n48C-n96A | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46N-n48C-n96A | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46A-n48A-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46B-n48A-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46C-n48A-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46D-n48A-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46N-n48A-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46A-n48B-n96C | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46B-n48B-n96C | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46C-n48B-n96C | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46D-n48B-n96C | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46N-n48B-n96C | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46A-n48C-n96D | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46B-n48C-n96D | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46C-n48C-n96D | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46D-n48C-n96D | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46N-n48C-n96D | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46A-n48C-n96E | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46B-n48C-n96E | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46C-n48C-n96E | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46D-n48C-n96E | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46N-n48C-n96E | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46A-n48(2A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46B-n48(2A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46C-n48(2A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46D-n48(2A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46N-n48(2A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46A-n48(2A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46B-n48(2A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46C-n48(2A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46D-n48(2A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46N-n48(2A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46A-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46B-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46C-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46D-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46N-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46A-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46B-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46C-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46D-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46N-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46A-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46B-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46C-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46D-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46N-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46A-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46B-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46C-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46D-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46N-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46A-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46B-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46C-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46D-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46N-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46A-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46B-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46C-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46D-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46N-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46A-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46B-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46C-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46D-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46N-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46A-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46B-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46C-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46D-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46N-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46A-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46B-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46C-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46D-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46N-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46A-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46B-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46C-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46D-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46N-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46A-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46B-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46C-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46D-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46N-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46A-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46B-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46C-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46D-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46N-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46A-n48(4A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46B-n48(4A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46C-n48(4A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46D-n48(4A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46N-n48(4A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n48A-n66A-n70A | CA_n48A-n66A CA_n48-n70A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48A-n66(2A)-n70A | CA_n48A-n66A CA_n48-n70A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48(2A)-n66A-n70A | CA_n48A-n66A CA_n48-n70A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48B-n66A-n70A | CA_n48A-n66A CA_n48-n70A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48A-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n66(2A)-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(2A)-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48B-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n66A-n71(2A) | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n48A-n66A-n77A | CA_n48A-n66A CA_n66A-n77A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n66A-n77C | CA_n48A-n66A CA_n66A-n77A CA_n77C | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48B-n66A-n77A | CA_n48A-n66A CA_n66A-n77A | n48 | CA_n48B_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48(2A)-n66A-n77A | CA_n48A-n66A CA_n66A-n77A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48(2A)-n66A-n77C | CA_n48A-n66A CA_n66A-n77A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48(2A)_BCS0 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 3 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48A-n70A-n71A | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(2A)-n70A-n71A | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48B-n70A-n71A | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n70A-n71(2A) | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n66A-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66A-n70A-n78A | CA_n66A-n78A CA_n70A-n78A | n66 | 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n70A-n71(2A) | CA_n66A-n71A CA_n70A-n71A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n66B-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | CA_n66B_BCS0. | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66(2A)-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66A-n71A-n77A | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n71B-n77A | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n71(2A)-n77A | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66(2A)-n71A-n77A | CA_n66A-n71A, CA_n66A-n77A, CA_n71A-n77A | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n71A-n77(2A) | CA_n66A-n71A, CA_n66A-n77A, CA_n71A-n77A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n66(2A)-n71A-n77(2A) | CA_n66A-n71A, CA_n66A-n77A, CA_n71A-n77A | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n66A-n71A-n78A | CA_n66A-n78A CA_n66A-n71A CA_n71A-n78A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n71A-n78(2A) | CA_n66A-n78A CA_n66A-n71A CA_n71A-n78A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n66(2A)-n71A-n78A | CA_n66A-n78A CA_n66A-n71A CA_n71A-n78A | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66(2A)-n71A-n78(2A) | CA_n66A-n78A CA_n66A-n71A CA_n71A-n78A | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink NOTE 2: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. NOTE 3: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 5: Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n78 with an n77 implementation. NOTE 6: Only single uplink carriers with power class other than PC3 are listed. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

<End of changes>