3GPP TSG-RAN WG4 Meeting #103-e	R4-2211148

Electronic Meeting, May 09 – 20, 2022

Title: 	WF on FeMIMO RRM requirements for inter-cell beam management and TRP-specific link recovery

Source: 	Huawei, HiSilicon

Agenda item:	9.18.2

Document for:	Approval

Introduction

This contribution is to capture the agreements for the email discussion for Rel-17 FeMIMO RRM in RAN4 #103-e meeting.

Way-forward

Inter-cell beam measurement

Issue 2-1-1: Whether to consider additional known cell condition

- Agreements:

  - Don’t need additional known cell condition, if benefit is not identified.

- FFS:

  - Add an additional known cell condition with L1 measurement only. It is network scheduling choice whether to schedule both or either L1 measurement and/or L3 measurements in inter-cell.

Issue 2-1-2 Whether Inter-cell L1-RSRP requirements are applicable for inter cell mTRP

- Option 1:

  - Inter-cell L1-RSRP requirements are not applicable for inter cell mTRP case, which UE is required to be able simultaneously receive from both serving cell and non-serving cell at a time, i.e. inter cell joint transmission (JT)

- Option 2:

  - Inter-cell L1-RSRP measurements – measurements on cell with different PCI are applicable for both inter-cell BM and inter-cell mTRP

- Option 3:

  - No clarification is needed.

Issue 2-2-1: UE reporting behaviour

- Option 1:

  - add actual UE reporting behaviors complying to known or unknown cell conditions as

    - The UE shall send L1-RSRP reports only for report configurations configured for the active BWP for cell with different PCI from serving cell under the known cell condition. Otherwise, the UE shall not send L1-RSRP reports.

- Option 2:

  - Don’t need to add such condition.

Issue 2-3-1 Sharing factors

- Option 1:

  - Update the sharing factors PSC and PCDP for scenarios 3 and 4 to also consider SMTC periodicity.

- Option 2:

  - RAN4 to further study R17 sharing factor for L1-RSRP measurement, considering the remaining occasions are fully overlapped between serving cell and non-serving cell.

- Option 3:

  - Value of PSC and PCDP to be confirmed as 2.

Issue 2-3-1a:  Applicability of Sharing factors

- Option 1: Sharing factors are applicable when SSB from serving cell and cell with different PCI are overlapping with same SSB index

  - Example1: TSSB,SC = 20, with offset 1 and TSSB,NSC = 20, with offset 1; SSB index from SC is 4 and SSB index of NSC is 4

  - Example2: TSSB,SC = 40, with offset 1 and TSSB,NSC = 20, with offset 1;  SSB index from SC is 4 and SSB index of NSC is 4

- Option 2: Sharing factors are applicable when SSB from serving cell and cell with different PCI are overlapping with same SSB index, or are adjacent SSB index with no symbol gap

  - Example1: TSSB,SC = 20, with offset 1 and TSSB,NSC = 20, with offset 1; SSB index from SC is 0 and SSB index of NSC is 0 OR SSB index from SC is 0 and SSB index of NSC is 1

  - Example2: TSSB,SC = 40, with offset 1 and TSSB,NSC = 20, with offset 1;  SSB index from SC is 2 and SSB index of NSC is 2 OR SSB index from SC is 2 and SSB index of NSC is 3

- Option 3: Sharing factors are applicable when SSB periodicity and offset are the same or overlapping between serving and cell with different PCI.

  - Example1: TSSB,SC = 20, with offset 1 and TSSB,NSC = 20, with offset 1; SSB index could be different

  - Example2: TSSB,SC = 40, with offset 1 and TSSB,NSC = 20, with offset 1; SSB index could be different

- Other options are not precluded.

Issue 2-3-2 Number of non-serving TRPs to be measured

- Option 1:

  - Number of other PCI UE can measure for L1-RSRP on FR1 is same as RAN1 capability and i.e., it can be more than 1.

- Option 2:

  - The number of cell with different PCI is 1.

Issue 2-3-3 Number of SSB resources UE need to measure

- Agreements:

  - The total number of SSB (including both with serving PCI and with different PCI) and CSI-RS configured for L1-RSRP measurements is no more than beamManagementSSB-CSI-RS.

Issue 2-4-1 Scheduling restriction for serving cell

- FFS:

  - Whether to introduce scheduling restriction for dynamic TDD when L1-RSRP measurement on cell with different PCI overlaps with serving cell UL slots.

Issue 2-4-2 Applicability of simultaneousRxDataSSB-DiffNumerology for serving cell

- Agreements

  - The IE simultaneousRxDataSSB-DiffNumerology can be reused for L1-RSRP measurements based on SSB with different PCI.

Issue 2-4-3 Whether to define scheduling restriction for non-serving cell

- Agreements:

  - The current scheduling restriction requirements are also applied to the data from a TRP with different PCI.

  - Further discuss whether any needs on the update of the existing capability signalling.

Issue 2-4-4: The joint requirement of IBM and inter-cell BM

- Agreements:

  - To consider the inter-band CA case and UE should be able to perform the SSB based measurements on non-serving cell in one band without any measurement and scheduling restrictions in the other band.

Issue 2-4-5: Measurement restriction

- Agreements

  - In FR2, there is no need to define measurement restrictions between serving cell SSB configured for L1-RSRP measurements and SSB with different PCI configured for L1-RSRP measurements since sharing factor PCDP is applied for L1-RSRP measurements.

Issue 2-5-1: MRTD requirement

- Agreements:

  - There is no impacts on existing MRTD requirements due to inter-cell BM.

Issue 2-6-1: Applicability of ICBM

- Agreements:

  - The ICBM feature shall be applicable to PCell and PSCell

- FFS:

  - The ICBM feature can be applicable to SCell

  - For intra-band ICBM using common TCI configurations, different reference CCs in the same CC list between the serving cell and a cell with different PCI is not supported in R17.

  - For intra-band ICBM using common TCI configurations, requirements are defined for the case when SSB measurements for a cell with different PCI are only performed in the cell that has the same SSB frequency as the reference CC.

  - Further discuss the UE capability and corresponding FR2 UE behaviour for simultaneous detection of time and frequency full-overlapped SSBs in R18 FR2 multi-Rx chain WI.

  - R17 ICBM feature is applicable to FR1 HST and FR2 HST. If RAN4 identifies any issue in applying HST related enhancements to ICBM related RRM requirements, RAN4 solve them in the R17 maintenance phase.

  - R17 ICBM feature is applicable to the scenarios when UE is configured with R17 enhanced gaps. If RAN4 identifies any issue in applying R17 enhanced gaps to ICBM related RRM requirements, RAN4 solve them in the R17 maintenance phase.

TRP specific link recovery and QCL definition

Issue 3-1-1 The conditions on applying PTRP = 2 in FR2

- Agreements:

  - When BFD-RS (or CBD-RS) resource in one BFD (or CBD) set are overlapped with the RS resource from the other BFD (or CBD) set.

    - FFS: whether to consider additonal condition “BFD-RS (or CBD-RS) resources in two BFD (or CBD) sets are received using different QCL type-D at the same time”

Issue 3-1-2 The conditions on applying PTRP = 1 in FR2

- Agreements:

  - Delete the sentences in square brackets for exceptions to PTRP=2 in TRP specific BFD requirements.

- FFS:

  - Whether a UE can evaluate BFD/CBD with evaluation priority over a BFD-RS set (corresponding to serving cell or primary TRP)

  - Whether a UE can stay only with non-serving-cell connection in serving-cell beam failure status.

Issue 3-1-3 other TRP-specific link recovery requirements

- Agreements:

  - The following requirements need to be introduced for completing TRP-specific link recovery requirements in R17.

    - TRP specific CBD measurements

    - L1 indication for TRP specific BFR

    - Scheduling restrictions due to TRP specific BFD/CBD

  - If a FR2 UE does not support simultaneous reception, the following scheduling restriction applies due to beam detection on mTRP.

    - The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH, CSI-RS for tracking or CSI-RS for CQI on reference symbols to be measured for candidate beam detection.

  - RAN4 to agree table 1 and table 2 as the evaluation period for SSB based CBD for each TRP in m-TRP operation.

    - Table 1: Evaluation period of one SSB based CBD-RS set in m-TRP operation of FR1

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(3 * P *PCBD) *TSSB) |
| DRX cycle > 320ms | Ceil(3 *P *PCBD) *TDRX |
| Note: TSSB is the SSB periodicity of the SSB in the set . TDRX is the DRX cycle length. |  |

    - Table 2: Evaluation period of one SSB based  CBD-RS set in m-TRP operation of FR2

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(3 *P *N *PCBD*TTRP) *TSSB) |
| DRX cycle > 320ms | Ceil(3 *P *N *PCBD*TTRP) *TDRX |
| Note: TSSB is the SSB periodicity of the SSB in the set . TDRX is the DRX cycle length. |  |

  - RAN4 to agree table 3 and table 4 as the evaluation period for CSI-RS based CBD for each TRP in m-TRP operation.

    - Table 3: Evaluation period for one CSI-RS based CBD-RS set in m-TRP operation of FR1

| Configuration | TEvaluateC_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(MCBD P PCBD) TCSI-RS) |
| DRX cycle > 320ms | Ceil(MCBD P PCBD) TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

    - Table 4: Evaluation period for one CSI-RS based CBD-RS set in m-TRP operation of FR2

| Configuration | TEvaluate_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(MCBD P N PCBD*TTRP) TCSI-RS) |
| DRX cycle > 320ms | Ceil(MCBD P N PCBD*TTRP) TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

Issue 3-1-4 TRP-specific link recovery requirements structure in spec

- Agreements:

  - keep the current structure for TRP-specific BFR requirements