3GPP TSG-RAN WG4 Meeting #103-e	R4-2207622

Online Meeting, 09 – 20 May 2022

3GPP TSG-RAN3 Meeting # 115-e	R3-222721

# 21st  February – 3rd March 2022

Title:	Questions concerning the implementation of RAN1 agreements in NRPPa

Response to:	-

Work Item:	NR_pos_enh-Core

Source:	RAN3

To:	RAN1, RAN2

Cc:	RAN4

Contact Person:

#### Name:	Yazid Lyazidi

E-mail Address:	yazid dot lyazidi at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN3 discussed the open issues for Rel-17 NR positioning enhancements WI, the following issues are identified as issues requiring RAN1 input:

Table 1: Issues requiring RAN1 inputs:

| Topic | Issue | Required RAN1 work |
| --- | --- | --- |
| UL-SRS-RSRPP | RAN1 has defined a new measurement for UL SRS-RSRPP: Agreement#6: Definition UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. For frequency range 1, the reference point for the UL SRS-RSRPP shall be the antenna connector of the gNB. For frequency range 2, UL SRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. Note: The following two options are supported by gNB to LMF: Option 1 (RX diversity for the first path UL SRS-RSRPP) - The same RX branch(es) as applied for the first path UL SRS-RSRPP measurements are used for the additional paths UL SRS-RSRPP measurements if those are provided together - For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements, then reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches Option 2 (RX diversity for UL SRS-RSRP) The same RX branch(es) as applied for UL SRS-RSRP measurements are used for UL SRS-RSRPP measurements (i.e., the first and additional paths UL SRS-RSRPP if those are provided). RAN3 will define the value range of the UL-SRS-RSRPP measurement for the i-th path as an INTEGER (0..126). Some companies question whether the information on the RX Diversity options, captured as a note in the RAN1 agreement, need to be signalled to LMF along with the UL-SRS-RSRPP measurement. | RAN1 to feedback if information on the Rx Diversity options needs to be signalled to LMF and to comment on the UL-SRS-RSRPP value range, if needed? |
| TEG | Some companies in RAN3 believe it is beneficial to signal the SRS port index to the LMF, so that LMF can group measurements based on Port index. RAN3 would like to know if SRS Port Index needs to be signalled to the LMF when SRS resource for MIMO is used? | RAN1 to feedback if information on the SRS port index needs to be signalled to LMF when SRS resource for MIMO is used. |
| On demand PRS | RAN3 has observed the following RAN1 agreement on On-demand PRS ON/OFF indicator (for LMF initiated request only): “In "On-demand PRS information for LMF-initiated on-demand DL PRS requests"; either per resource, or per resource set, or per UE”. RAN3 would like to inform RAN1 that the procedures defined by RAN3 for on-demand PRS transmission are non-UE-associated (i.e., cell specific) and thus an ON/OFF indication per UE does not make sense. Instead, it is considered to have an OFF indication per TRP to minimize the transmission power. | RAN1 to take into account and update their agreed parameter lists for On-demand PRS. |

Based on the assistance information for DL-AoD specified by RAN1/RAN2, the corresponding NRPPa signaling could require, in the case where maximum granularity is used uniformly in azimuth and elevation, an excess of 6 million relative powers per TRP / resource to be signaled over NRPPa and, as consequence, via F1-AP and per NG-RAN design, over NG transport. RAN3 assumes (and would like to confirm) that realistic implementations would not require this high level of data volume traffic and would also use this function sparingly.

RAN3 has agreed to include some mitigations over NRPPa/F1AP, e.g. allowing for the indication of “no change” if a previous TRP beam antenna configuration is still valid.

Note that, as with other TRP configuration items, RAN3 has agreed that OAM is also a possible option for providing such information to LMF.

2. Actions:

To RAN WG1, WG2

ACTION:

- RAN3 respectfully asks RAN1 to provide feedback on RAN3’s questions.

- RAN3 respectfully asks RAN1/RAN2 to provide feedback, if any, on the issue of signalling load for DL-AoD information.

3. Date of Next RAN WG3 Meetings:

RAN3#116-e	16 – 27 May 2022	Electronic meeting

RAN3#117	TBD