3GPP TSG-RAN WG4 Meeting #103e	R4-2211122

Electronic Meeting, 9th – 20th May, 2022

Title:	 reply LS on TA validation for CG-SDT

Response to:	R2-2204269/R4-2207621

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	RAN4

To:	RAN2

Cc:	-

Contact Person:

#### Name:	Aijun Cao

E-mail:	cao.aijun@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN4 thanks RAN2 for the LS sharing RAN2 agreements on the topic of CG-SDT resource validation based on RSRP change.

RAN4 discussed the RAN2 LS and identified that there is an inconsistency on T1 definition corresponding to the reference RSRP, where in the RAN2 agreement it refers to the time when RRCRelease message is received, however, this time instance is specified in RAN4 specs as the moment of when the latest NTA was obtained by the UE via Timing Advance Command MAC control element – see TS 38.133.  RAN4 is aware of that both cases are valid, and agreed to update T1 definition as below:

- When changing from RRC_CONNECTED to RRC_INACTIVE state, T1 is the time when RRCRelease with CG-SDT configuration is received

- If TA command is received while in RRC_INACTIVE state, T1 is the time when the latest MAC CE TA command is received

In addition, during the discussion on SDT prioritization over inter-frequency and inter-RAT neighbor cell measurement requirements including EMR and positioning measurements, there are different readings on the RAN2 agreements “The UE is not required to perform Idle/inactive measurements in section 5.7.8 of RRC during SDT.  Check the details of this requirements”, thus RAN4 would like to know RAN2’s view on whether or not Section 5.7.8 covers EMR requirements.

2. Actions:

To RAN2 group:

ACTION: 	RAN4 respectfully asks RAN2 to take the above information into account for the specification of CG-SDT.

3. Dates of next TSG-RAN WG4 meetings:

RAN4#104-e	15 – 26 Aug 2022			e-Meeting