3GPP TSG RAN WG4 Meeting #103-e	R4-2211167

Electronic Meeting, 09-20 May 2022

Title:	LS on timing advance (TADV) report mapping for NR UL E-CID positioning

Response to:	-

Release:	Rel-17

Work Item:	TEI17

Source:	TSG RAN4

To:	TSG RAN WG1, TSG RAN3

Cc:	TSG RAN WG2

Contact Person:

#### Name:	          Muhammad Kazmi

E-mail Address:	muhammad.kazmi@ericsson.com

1. Overall Description:

RAN4 would like to inform that RAN4 has introduced the following Timing advance (TADV) measurement report mapping in TS 38.133, Rel-17, for the timing advance (TADV) measurement specified in clause 5.2.7 of TS 38.215 v17.1.0:

The reporting range of Timing advance (TADV) is defined from 0 to 3150848 Tc with:

- 128 Tc resolution for timing advance less or equal to 262144 Tc and

- 512 Tc for timing advance greater than 262144 Tc.

The mapping of measured quantity is defined in Table 13.X.1-1:

Table 13.X.1-1: Timing advance (TADV) measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| TIME_ADVANCE_00 | TADV 128 | Tc |
| TIME_ADVANCE_01 | 128 TADV < 256 | Tc |
| TIME_ADVANCE_02 | 256 TADV < 384 | Tc |
| … | … | … |
| TIME_ADVANCE_2046 | 261888 TADV < 262016 | Tc |
| TIME_ADVANCE_2047 | 262016 TADV < 262144 | Tc |
| TIME_ADVANCE_2048 | 262144 TADV < 262656 | Tc |
| TIME_ADVANCE_2049 | 262656 TADV < 263168 | Tc |
| … | … | … |
| TIME_ADVANCE_7688 | 3149824 TADV < 3150336 | Tc |
| TIME_ADVANCE_7689 | 3150336 TADV < 3150848 | Tc |
| TIME_ADVANCE_7690 | 3150848 TADV | Tc |

NOTE:	For report mapping, the TADV equal is to (gNB Rx – Tx time difference) + NTA_offset, where NTA_offset is based on the information n-TimingAdvanceOffset as specified in TS 38.331.

2. Actions:

To RAN1

ACTION: 	The above timing advance (TADV) report mapping consists of only positive values. It is up to RAN1 to judge if this has any impact on RAN1 specifications.

To RAN3

ACTION: 	RAN4 respectfully asks RAN3 to consider the above Timing advance (TADV) report mapping for introducing the necessary signaling in their specifications.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #104-e	    August 15-26, 2022, Electronic Meeting

TSG-RAN WG4 Meeting #104-bis-e 	    October 10-19, 2022, Electronic Meeting