# 3GPP TSG-RAN WG4 Meeting #103-e                                                        R4-2211204

# Electronic Meeting, May 09 – May 20, 2022

Title:	LS on Incorrect PMI Reporting

Release:	Rel-17

Work Item:	TEI17

Source:	RAN WG4

To:	RAN

Cc:	RAN1, RAN2

Contact Person:

#### Name:	Manasa Raghavan

E-mail Address:	manasa.raghavan@apple.com

Attachments:	None

1. Overall Description:

In RAN4#103-e meeting, the issue of incorrect PMI reporting was discussed. Following agreements are reached by RAN4:

- On the existence and impact of the incorrect PMI reporting:

- RAN4 observed that the issue only exists when all the conditions are met:

  - Cell Edge condition

  - Colliding NZP-CSI-RS configured with interference cells

  - Simplistic UE implementation for CSI-RS channel estimation without FD (frequency domain) filtering

- Companies also observed less prevalent incorrect PMI reporting in newer UE implementation, likely because such simplistic UE implementation does not exist anymore.

- It is common understanding that incorrect PMI reporting will impact performance and throughput.

- On usefulness of introducing requirements for incorrect PMI reporting in ICI, we have 2 sets of views in RAN4:

- Proponents of PMI requirements in ICI are claiming usefulness as -

- Without requirements, future issues in deployment due to UEs with CSI-RS channel estimation without FD (frequency domain) filtering cannot be avoided.

- It can ensure the proper implementation for newly released UEs.

- There are some benefits to use the configuration of overlapping CSI-RS, e.g., ease for cell planning, higher peak PDSCH throughput. Also, RAN4 already define CSI requirements with the configuration of overlapping CSI-RS.

- Non-overlapping NZP CSI-RS is not a solution that will be implemented by the BS frequently.

- Opponents of PMI requirements in ICI are refuting usefulness as -

- Issues only exists under certain rare conditions with overlapping CSI-RS resource configuration and very early too simplistic UE implementation.

- Issues can be handled by NW configuration. One solution could be NW configuring non-overlapping NZP CSI-RS. Even partially overlapping NZP CSI-RS is expected to alleviate the issue of pilot contamination.

- New UEs will not have the issue, hence no need to create requirements to ensure this.

- New performance requirements will not be applicable to older UEs and introducing new requirements will not solve the issue in legacy UEs where the issue is observed.

- On estimated workload for RAN4 to define requirements:

- RAN4 estimates at least 3 meetings to introduce requirements, including discussion on whether a new UE capability (e.g., in relation to a UE reference receiver) is needed.

2. To 3GPP RAN

ACTION: RAN4 respectfully requests RAN to take the above information into account.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #104-e 	15 Aug - 26 Aug 2022		        		Online

TSG-RAN WG4 Meeting #104-bis-e 	10 Oct - 19 Oct 2022	                                       Online