3GPP TSG-RAN4 Meeting #103-e	R4-2207631

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1048 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | V2X intra-band con-current operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2022-04-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clarifications are made so as to have different specifications for intra-band non-simultaneous Uu and SL operation compared to intra-band simultaneous Uu and SL operation |  |  |  |  |  |  |  |  |
| Summary of change: |  | Several sections of the transmitter characteristics and receiver characteristics where V2X intra-band con-current operation is specified was modfied by adding scenarios for intra-band con-current non-simultaneous Uu and SL operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current specification would be unclear as to whether it appies to either or both intra-band simultaneous and non-simultaneous Uu and SL operation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5E.2.2.2, 6.5E.2.4.2, 6.5E.3.3, 6.5E.4.2, 7.4E.2, 7.5E.2, 7.6E.2.2, 7.6E.3.2, 7.7E.2, 7.8E.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change >

##### 6.5E.2.2.2	Spectrum emission mask for V2X con-current operation

For the inter-band con-current NR V2X operation, the general/additional SEM requirements specified in clause 6.5.2 shall apply for the uplink in licensed band and the general/additional SEM requirements specified in clause 6.5E.2.2.1 shall apply for the sidelink in licensed band or Band n47.

For intra-band  con-current NR V2X operation for simultaneous transmission of Uu and SL with bandwidth class B the specifications in section 6.5A.2.2.1 and 6.5A.2.2.2 apply.

For the intra-band con-current NR V2X operation for non-simultaneous transmission of Uu and SL the requirements specified in clause 6.5.2 shall apply for the uplink in licensed band and the requirements specified in clause 6.5E.2.2.1 shall apply for the sidelink in licensed band.

## <End of Change >

## <Start of Change >

##### 6.5E.2.4.2	ACLR for V2X con-current operation

For the inter-band con-current NR V2X operation, the ACLR requirement specified in clause 6.5.2.4 shall apply for the uplink in licensed band and the ACLR requirement specified in clause 6.5E.2.4.1 shall apply for the sidelink in licensed band or Band n47.

For the intra-band con-current NR V2X operation for simultaneous transmission of Uu and SL with bandwidth classes B, the ACLR requirement specified in clause 6.5A.2.4.1 shall apply for the both uplink and sidelink transmission in licensed band.

For the intra-band con-current NR V2X operation for non-simultaneous transmission of Uu and SL the ACLR requirements specified in clause 6.5.2.4 shall apply for the uplink in licensed band and the ACLR requirement specified in clause 6.5E.2.4.1 shall apply for the sidelink in licensed band.

## <End of Change >

## <Start of Change >

#### 6.5E.3.3	Spurious emissions for UE co-existence for V2X con-current operation

For the inter-band con-current NR V2X operation, the UE-coexistence requirements in Table 6.5E.3.3.1-1 apply for the corresponding inter-band con-current operation with transmission assigned to both uplink in licensed band and sidelink in Band n47.

Table 6.5E.3.3.1-1: Requirements for inter-band con-current V2X operation

| V2X | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| con-current operating band configuration | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| V2X_n1A-n47A | E-UTRA Band 1, 3, 5, 7, 8, 22 26, 28, 34, 40, 41, 42, 44, 45, 65, 68, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| V2X_n8A-n47A | E-UTRA Band 1, 3, 7, 8, 22, 28, 34, 39, 40, 41, 42, 45, 65, 68, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| V2X_n39A-n47A | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| V2X_n40A-n47A | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 42, 44, 45, 68, 72 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| V2X_n41A-n47A | E-UTRA Band 1, 3, 5, 8, 26, 28, 34, 39, 42, 44, 45, 65, 73 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| V2X_n71A-n47A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 2 |
|  | NR Band n71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| V2X_n78A-n47A | E-UTRA Band 1, 3, 5, 7, 8, 26 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| V2X_n79A-n47A | E-UTRA Band 1, 3, 5, 8, 28, 34, 39, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 2: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 3: Applicable when NS_33 is configured by the pre-configured radio parameters for power class 3 V2X UE. NOTE 4: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. |  |  |  |  |  |  |  |

For the intra-band con-current simultaneous Uu and SL NR V2X operation, the UE-coexistence requirements in Table 6.5A.3.2.1-1 apply for the corresponding intra-band con-current operation for the both uplink and sidelink transmission in licensed band.

For intra-band con-current non-simultaneous Uu and SL NR V2X operation the UE requirements in Table 6.5.3.2-1 apply to both Uu and SL.

## <End of Change >

## <Start of Change >

#### 6.5E.4.2	Transmit intermodulation for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 6.5.4 shall apply for the uplink in licensed band and the requirements specified in clause 6.5E.4.1 shall apply for the sidelink in licensed band or Band n47.

For the intra-band con-current simultaneous Uu and SL NR V2X operation, the requirements specified in clause 6.5A.4 shall apply for both uplink and sidelink in licensed band

For intra-band con-current non-simultaneous Uu and SL NR V2X operation the requirements specified in clause 6.5.4 shall apply for the uplink in licensed band and the requirements specified in clause 6.5E.4.1 shall apply for the sidelink.

## <End of Change >

## <Start of Change >

### 7.4E.2	Maximum input level for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 7.4E.1 shall apply for the NR sidelink reception in the operating Bands in Table 5.2E.1-1 and the requirements specified in clause 7.4 shall apply for the NR downlink reception in licensed band while all downlink carriers are active.

For the intra-band con-current simultaneous Uu and SL NR V2X operation, the requirements specified in clause 7.4A shall be applied which is defined as the maximum mean power received at the UE antenna port, over the transmission bandwidth configuration of each CC.

For intra-band con-current non-simultaneous Uu and SL NR V2X operation the requirements specified in clause 7.4E.1 shall apply for the NR sidelink reception, and the requirements specified in clause 7.4 shall apply for the NR downlink reception.

## <End of Change >

## <Start of Change >

### 7.5E.2	Adjacent channel selectivity for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 7.5E.1 shall apply for the NR sidelink reception in the operating Bands in Table 5.2E.1-1 and the requirements specified in clause 7.5 shall apply for the NR downlink reception in licensed band while all downlink carriers are active.

For the intra-band con-current simultaneous Uu and SL NR V2X operation, the SL carrier is configured with nominal channel spacing to the NR downlink carrier. The minimum requirement specified in clause 7.5A.1 shall be applied to the corresponding intra-band con-current V2X UE.

For intra-band con-current non-simultaneous Uu and SL NR V2X operation the requirements specified in clause 7.5E.1 shall apply for the NR sidelink reception and the requirements specified in clause 7.5 shall apply for the NR downlink reception.

## <End of Change >

## <Start of Change >

#### 7.6E.2.2	In-band blocking for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 7.6E.2.1 shall apply for the NR sidelink reception in the operating Bands in Table 5.2E.1-1 and the requirements specified in clause 7.6.2 shall apply for the NR downlink reception in licensed band while all downlink carriers are active.

For the intra-band con-current simultaneous Uu and SL NR V2X operation, the SL carrier is configured with nominal channel spacing to the NR downlink carrier. The minimum requirement specified in clause 7.6A.2.1 shall be applied to the corresponding intra-band con-current V2X UE.

For intra-band con-current non-simultaneous Uu and SL NR V2X operation the requirements specified in clause 7.6E.2.1 shall apply for the NR sidelink reception and the requirements specified in clause 7.6.2 shall apply for the NR downlink reception.

## <End of Change >

## <Start of Change >

#### 7.6E.3.2	Out-of-band blocking for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 7.6E.3.1 shall apply for the NR sidelink reception in Band n47 and the requirements specified in clause 7.6.3 shall apply for the NR downlink reception in licensed band while all downlink carriers are active.

For the intra-band con-current simultaneous Uu and SL NR V2X operation, the SL carrier is configured with nominal channel spacing to the NR downlink carrier. The minimum requirement specified in clause 7.6A.3.1 shall be applied to the corresponding intra-band con-current V2X UE.

For intra-band con-current non-simultaneous Uu and SL NR V2X operation the requirements specified in clause 7.6E.3.1 shall apply for the NR sidelink reception and the requirements specified in clause 7.6.3 shall apply for the NR downlink reception.

## <End of Change >

## <Start of Change >

### 7.7E.2	Spurious response for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 7.7E.1 shall apply for the NR sidelink reception in the operating Bands in Table 5.2E.1-1 and the requirements specified in clause 7.7 shall apply for the NR downlink reception in licensed band while all downlink carriers are active.

For the intra-band con-current simultaneous Uu and SL NR V2X operation, the SL carrier is configured with nominal channel spacing to the NR downlink carrier. The minimum requirement specified in clause 7.7A.1 shall be applied to the corresponding intra-band con-current V2X UE.

For intra-band con-current non-simultaneous Uu and SL NR V2X operation the requirements specified in clause 7.7E.1 shall apply for the NR sidelink reception and the requirements specified in clause 7.7 shall apply for the NR downlink reception.

## <End of Change >

## <Start of Change >

#### 7.8E.2.2	Wide band Intermodulation for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 7.8E.2.1 shall apply for the NR sidelink reception in the operating Bands in in Table 5.2E.1-1 and the requirements specified in clause 7.8 shall apply for the NR downlink reception in licensed band while all downlink carriers are active.

For the intra-band con-current simultaneous Uu and SL NR V2X operation, the SL carrier is configured with nominal channel spacing to the NR downlink carrier. The minimum requirement specified in clause 7.8A.2.1 shall be applied to the corresponding intra-band con-current V2X UE

For intra-band con-current non-simultaneous Uu and SL NR V2X operation the requirements specified in clause 7.8E.2.1 shall apply for the NR sidelink reception and the requirements specified in clause 7.8 shall apply for the NR downlink reception.

## <End of Change >